# 3GPP TSG-RAN1 #83	R1-157397

# 15 - 22 Nov 2015

# Anaheim, US

Source:                    	Ericsson

Title:  	NB-IoT – SC-FDMA UL spectrum characteristics, PAPR, and relative cubic metric

Document for:        	Discussion and decision

Agenda Item:         	6.2.6.1.1

# Introduction

At RAN#69, a new work item named NarrowBand IOT (NB-IOT) was approved, see [1]. The objective is to specify a radio access for cellular internet of things that addresses improved indoor coverage, support for massive number of low throughput devices, low delay sensitivity, ultra-low device cost, low device power consumption and (optimized) network architecture.

NB-IOT should support 3 different modes of operation:

1.	“Stand-alone operation” utilizing for example the spectrum currently being used by GERAN systems as a replacement of one or more GSM carriers

2.	“Guard band operation” utilizing the unused resource blocks within a LTE carrier’s guard-band

3.	“In-band operation” utilizing resource blocks within a normal LTE carrier

NB-IOT currently includes two uplink options under consideration, SC-FDMA, and FDMA with GMSK modulation. In this contribution, we present the spectral characteristics, the peak-to-average power ratio (PAPR), and relative cubic metric, of the SC-FDMA option.

# SC-FDMA uplink option for NB-IOT

The SC-FDMA option in NB-IOT is based on single-carrier frequency-division multiple-access (SC-FDMA). This allows flexible UE bandwidth allocation including single tone transmission as a special case of SC-FDMA.

# Uplink Modulation

The two modulation options considered for the SC-FDMA uplink option are pi/2 BPSK and pi/4 QPSK. In these modulations, the constellation is rotated pi/2 or pi/4 radians every symbol. This enables smoother transitions between constellation points, reducing the peak to average power ratio (PAPR). These modulation options are considered for uplink data and control channel transmissions, except for M-PRACH preambles. This contribution will give the spectral characteristics, the PAPR, and the relative cubic metric compared to GMSK, of SC-FDMA with a subcarrier spacing of 2.5 kHz. The PAPR of M-PRACH is addressed in [2].

## Transmit Filters

The amplitude variations of a transmission depend on the transmit filter, and in this contribution two transmit filters will be studied. The first one, filter G, is a 9-tap Gaussian filter, and the second one, filter F, is a 12-tap filter with rather flat frequency response. The two filters are presented in Figure 1 and Figure 2, respectively. In Table 1 the coefficients for these filters are given.

One realization of the transmitter may be to generate symbols of length 128 samples, before concatenating them with a cyclic prefix of 9 samples (10 samples for every seventh symbol). A subcarrier spacing of 2.5 kHz corresponds to transmitting these samples at 320 ksps (kilo-samples per second). For transmit filtering, this signal can be up-sampled a number of times and fed through a transmit filter. Other implementation alternatives are also possible.

In this contribution, for simplicity, the transmitter is operating directly at 1.28 Msps, the sampling rate at which the transmit filters are applied in the evaluations. With such a setup each OFDM symbol is 512 samples long, excluding the cyclic prefix. At this sampling rate, the cyclic prefix is 36 or 40 samples, which can be compared to the filter length of 9 or 12 samples.

Figure 1 Filter G, a 9-tap Gaussian filter

Figure 2 Filter F, a 12-tap filter with rather flat frequency response

Table 1 Filter coefficients for the two filters

| Filter | Filter coefficients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Filter G, Gaussian | 1 | 2 | 3 | 4 | 5 | 6 |
|  | 0.050876 | 0.28006 | 0.94703 | 1.9671 | 2.5099 | 1.9671 |
|  | 7 | 8 | 9 | 10 | 11 | 12 |
|  | 0.94703 | 0.28006 | 0.050876 |  |  |  |
| Filter F, Flat | 1 | 2 | 3 | 4 | 5 | 6 |
|  | -0.090387 | -0.1485 | 0.081264 | 0.83855 | 1.9791 | 2.8621 |
|  | 7 | 8 | 9 | 10 | 11 | 12 |
|  | 2.8621 | 1.9791 | 0.83855 | 0.081264 | -0.1485 | -0.090387 |

## Spectral characteristics

The SC-FDMA signal can be composed of 1-72 subcarriers if the bandwidth is 180 kHz and the subcarrier spacing is 2.5 kHz. The power spectral density (PSD) of a 72-subcarrier transmission is shown for each filter in Figure 3 and Figure 4. The transmitted SC-FDMA signals have 23 dBm power in all figures, and the GSM spectrum mask shown in the figures is positioned so that its maximum PSD is aligned with what would be the maximum PSD a GSM signal with 23 dBm power.

Figure 3 Spectral characteristics of filter G

Figure 4 Spectral characteristics of filter F

The variations in the pass band of filter F are within 1 dB. As shown, SC-FDMA filtered with these filters fulfill the GSM spectrum mask with a 72 subcarrier transmission.

Figure 5 and Figure 6 illustrate the power spectral density of a single-subcarrier transmission of SC-FDMA with 2.5 kHz subcarrier spacing, positioned in the first, one of the middle, and the last, subcarrier. The GSM spectrum mask is exceeded within the 180 kHz carrier bandwidth. This is expected since the NB-IOT signal is so narrow compared to the wide GSM signal, relative to which the mask is positioned. The more important question is whether the mask is exceeded outside of the 180 kHz carrier bandwidth, and the figures show that SC-FDMA fulfills the mask also with single-subcarrier transmission.

Figure 5 Power spectral density of single 2.5 kHz subcarrier SC-FDMA transmission with filter G.

The power spectral density in Figure 5 and Figure 6 represents both pi/2 BPSK and pi/4 QPSK modulation.

Figure 6 Power spectral density of single 2.5 kHz subcarrier SC-FDMA transmission with filter F.

In Figure 7 and Figure 8 multi-subcarrier transmissions are shown. The curves represent both pi/2 BPSK and pi/4 QPSK modulation. In all cases the transmissions fulfil the GSM spectrum mask outside of the 180 kHz carrier bandwidth.

Figure 7 Power spectral density of multi-subcarrier SC-FDMA transmissions with filter G

Figure 8 Power spectral density of multi-subcarrier SC-FDMA transmissions with filter F

## Peak to average power ratio

Random symbols in the two constellation alphabets pi/2 BPSK and pi/4 QPSK have been generated and mapped to one or more subcarriers, and the instantaneous peak power to average power ratio (PAPR) of the transmitted samples has been measured. The 99.99% percentile of this PAPR ratio is measured, meaning that only 0.01% of the samples would have a higher PAPR. This has been performed for all subcarrier positions, assuming a contiguous subcarrier allocation for multi-subcarrier transmissions, and the maximum PAPR measure out of all positions is presented.

Figure 9 shows the PAPR for pi/2 BPSK for the two transmit filters, with and without the 2 sample windowing and overlap method described in [3] and [4]. We see that with the windowing and overlap method, PAPR is reduced, down to <0.1 dB when the Gaussian filter G is used as transmit filter.

Figure 9 99.99% PAPR of pi/2 BPSK, with the two filters, with and without 2 sample windowing and overlap

Figure 10 shows the PAPR for pi/4 QPSK for single- and multi-subcarrier transmission. For single-subcarrier transmission the windowing and overlap method reduces PAPR down to 0.1 dB, while the PAPR for multi-subcarrier transmission is higher. However, in the following section on cubic metric we will see that although the PAPR is high, the back-off required for multi-subcarrier transmission is limited.

Figure 10 99.99% PAPR of pi/4 QPSK with the two filters, with and without 2 sample windowing and overlap

The results for single-subcarrier transmission are presented in Table 2, and for multi-subcarrier transmission with pi/4 QPSK in Table 3.

Table 2 Summary of PAPR characteristics, with and without 2 sample windowing and overlap

| PAPR [dB] | Filter |  |
| --- | --- | --- |
| Modulation | Gaussian | Flat |
| pi/2 BPSK with windowing | 0.09 | 0.25 |
| pi/2 BPSK w/o windowing | 1.0 | 0.9 |
| pi/4 QPSK with windowing | 0.1 | 0.25 |
| pi/4 QPSK w/o windowing | 1.1 | 1.0 |

Table 3 Summary of PAPR characteristics

| PAPR [dB] | Number of subcarriers |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | 1 | 2 | 4 | 8-72 |
| pi/2 BPSK | 0.09 - 1.0 |  |  |  |
| pi/4 QPSK | 0.1 - 1.1 | 2.4 | 5.4 | 6.0-6.6 |

## Relative Cubic Metric

It is known that PAPR is not a good indicator of how much a PA would need to back off to handle the amplitude variations of a signal. The cubic metric is a much more suitable measure [5]. The cubic metric CM is defined as

where

is the raw cubic metric of a signal, and where

is the raw cubic metric of the WCDMA voice reference signal.

Also  , where N is the number of samples in the signal x, and .

The denominator K is 1.56 for LTE signals [5]. The term X is related to the bandwidth of the signal. To avoid the dependence on the WCDMA voice reference signal and the bandwidth related parameter X, we will study the relative difference between the two NB-IOT uplink options, the cubic metric increase of SC-FDMA compared to GMSK:

where v1 is the SC-FDMA signal and v2 is the GMSK signal.

Figure 11 shows the relative cubic metric of a single-subcarrier SC-FDMA signal. We see that the increase compared to a GMSK signal is negligible, close to 0.01 dB. We also see that the windowing and overlap method, that decreased the PAPR, has no positive effect on cubic metric.

Figure 11 Cubic metric relative to GMSK of pi/2 BPSK with the two filters, with and without 2 sample windowing and overlap

Figure 12 shows the cubic metric increase compared to GMSK for SC-FDMA single- multi-subcarrier transmissions with pi/4 QPSK. Also with pi/4 QPSK the single-subcarrier transmission has a cubic metric increase of only 0.01 dB compared to GMSK.

We also see that the cubic metric increase, which indicates the required PA back-off, for transmissions with as many as 32 or 72 subcarriers is limited to 2.2 dB or less. Thus, even though the PAPR for these multi-subcarrier cases is rather high, the required PA back-off to support them is very limited.

Again it is observed that the windowing and overlap method has no positive effect on the cubic metric.

Figure 12 Cubic metric relative to GMSK of pi/4 QPSK with the two filters, with and without 2 sample windowing and overlap

Table 4 summarizes the cubic metric increase of SC-FDMA relative to GMSK.

Table 4 Summary of SC-FDMA cubic metric increase, compared to GMSK

| Relative Cubic Metric [dB] | Number of subcarriers |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | 1 | 2 | 4 | 8 | 16 | 32 | 72 |
| pi/2 BPSK | 0.01 |  |  |  |  |  |  |
| pi/4 QPSK | 0.01 | 1.5 | 2.0 | 2.2 | 2.2 | 2.2 | 2.1 |

# Conclusions

Observation 1: SC-FDMA allows power efficient PA operation and integration with single subcarrier transmission since the relative cubic metric increase compared to GMSK is negligible (0.01 dB), and PAPR is less than 0.1 dB.

Observation 2: The back-off required for multi-subcarrier transmission with SC-FDMA and pi/4 QPSK, even with as many subcarriers as 72, is limited to 2.2 dB, which allows multi-subcarrier transmission for most UE locations, reducing the transmission time and thus the power consumption.

Observation 3: Although the windowing and overlap method studied reduces PAPR, it has no significant benefit in terms of cubic metric, and may not be needed.

# References

RP-151621, “New Work Item: NarrowBand IOT (NB-IOT)”, source Qualcomm Incorporated, RAN#69, Sep 14th to 16th, 2015, Phoenix, USA.

R1-157424, NB-IoT - Random Access Design, source Ericsson, 3GPP TSG-RAN1 #83, 15-22 Nov 2015, Anaheim, US

R1-156011, “Narrowband LTE – Random Access Design”, source Ericsson, 3GPP TSG-RAN1 #82bis, 5th – 9th October 2015, Malmo, Sweden

R1-156012, “NB-LTE – UL Modulation”, source Ericsson, 3GPP TSG-RAN1 #82bits, 5th – 9th October 2015, Malmo, Sweden

GP-150048, “Peak-to-Average Power Ratio and Power Spectral Density of Tone-Phase-Shift Keying”, source Qualcomm Incorporated, 3GPP TSG GERAN #65, 9 – 13 March 2015, Shanghai, China

R1-060385, “Cubic Metric in 3GPP-LTE”, source Motorola, 3GPP TSG RAN WG1 #44, February 13-17, 2006, Denver, USA