3GPP TSG RAN1 Meeting #83								R1-157402

Anaheim, CA, USA, 16th– 20th November, 2015

Source: 		Ericsson

Title:	NB-IoT - Battery lifetime evaluation for guardband operation

Agenda Item:	6.2.6.1.2

Document for:	Discussion and decision

# Introduction

At RAN#69 a new work item on narrowband IoT and tasked RAN1 to evaluate two numerology options for both UL and DL:

- DL: 15 kHz sub-carrier spacing (with normal or extended CP) and 3.75 kHz sub-carrier spacing.

- UL: FDMA with GMSK modulation and SC-FDMA (including single-tone transmission as a special case of SC-FDMA)

One of the performance objectives is for the IoT device to meet the battery lifetime requirement of 10 years. Even if a possible down selection of DL and UL numerology could be seen as independent evaluation, in order to estimate the battery lifetime, a system with both DL and UL must be assumed for the evaluation. For this reason, in this contribution we have evaluated a NB-IoT system as described in [3] in operating in the guard band using the methodology given in TR45.820 [1]. The evaluation follows the same steps as the evaluation for inband operation in [2].

# NB-LTE energy consumption evaluation

## Assumptions for the guard band evaluation

The difference to the analysis for the inband case in [2] is that due to the lower DL overhead for guard band operation, the transmission times in the DL have been reduced according to the reduction in overhead. The same power boosting of 6 dB is assumed also for the guard band case..

## Results

The overall reception, transmission and idle times per report at different coverage based on the new assumptions are shown in Table 1.

Table 1 Transmission, Reception and Idle time per report (ms)

|  | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| Tx (50 bytes) | 48 | 334 | 2110 |
| Tx (200 bytes) | 80 | 1005 | 3720 |
| Rx | 156 | 430 | 1929 |
| Idle | 1271 | 1505 | 3123 |

The estimated lifetime in years are presented in Table 2for two different packet sizes, two reporting intervals and at different coverage.

Table 2 estimated battery lifetime in years

| Packet size, reporting interval | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| 50 bytes, 2 hours | 27.0 | 12.3 | 2.9 |
| 200 bytes, 2 hours | 24.3 | 5.9 | 1.8 |
| 50 bytes, 1 day | 36.8 | 32.4 | 18.8 |
| 200 bytes, 1 day | 36.3 | 26.1 | 14.1 |

We see that with the updated assumptions in section 2.1, the estimated battery life for NB-LTE is further improved fulfilling the 10 year target for battery lifetime and meets the 10 year requirement for the same scenarios as when operating standalone[4].

Observation 1:

- NB-LTE can meet the battery target of 10 years for the same scenarios when operating in the guard band as for standalone operation.

# Conclusions

In this contribution we evaluated battery lifetime of the NB-LTE when operating in the guard band and made the following observation.

Observation:

- NB-LTE can meet the battery target of 10 years for the same scenarios when operating in the guard band as for standalone operation.

# References

TR 45.820 v13.0.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”

R1-157411, “NB-IoT - Battery lifetime evaluation for inband operation”, Ericsson

R1-156013, “NB-LTE – General L1 concept description”, Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, ZTE, Samsung

R1-157390, “NB-IoT - Battery lifetime comparison for standalone operation”, Ericsson