3GPP TSG RAN WG1 Meeting #83				  	R1-157603

Anaheim, USA, 15th – 22nd November 2015

___________________________________________________________________Agenda item: 6.2.8.1.1

Source: LG Electronics, Huawei, HiSilicon, CATT, Nokia Networks

Title: WF on UE autonomous resource selection principles

Document for: Discussion and Decision

- Proposal

The following is captured in TR 36.843:

- Enhancement to UE autonomous resource selection

  - Collision avoidance based on sensing (P1), enhanced random resource selection (P2), and location-based resource selection (P3) are shown to provide gain when each of them is evaluated individually.

    - Further discussion is needed to identify whether operating a combination of the principles provides more gain than operating an individual principle.

    - It is understood that a combination of P1 and P2 is possible at least in the following example:

      - In resource selection, a UE by sensing excludes the resources that will be occupied by other UEs, and the enhanced random selection applies to the remaining resources.

    - It is understood that a combination of P1 and P3 is possible at least in the following example:

      - Subsets of resources are associated with sets of UE location, and a UE performs P1 in the subset which is associated with its current location.

    - It is understood that a combination of P2 and P3 is possible at least in the following example:

      - Subsets of resources are associated with sets of UE location, and a UE performs P2 in the subset which is associated with its current location.

    - Details FFS