3GPP TSG RAN WG1 Meeting #84	R1-160315

St Julian’s, Malta, 15th-19th February 2016

Agenda Item:	7.2.1.1.2

Source:	Huawei, HiSilicon

Title:	NB-PDCCH resource mapping

Document for:	Discussion and decision

# Introduction

In previous NB-IoT Ad-Hoc meeting, significant progresses were made on NB-PDCCH [1].

Agreements:

- A CCE for NB-PDCCH is composed by resources within a subframe

  - within a PRB pair, 2 CCEs are defined

  - Single antenna port transmission is also supported based on the above definition

- Same transmission scheme is applied to NB-PDCCH, NB-PBCH, and NB-PDSCH

- In in-band, first few OFDM symbols are not used for NB-PDCCH

  - FFS CFI is signaled by SIB or MIB or fixed to 3

- In stand-alone and guard-band, all OFDM symbols are assumed to be available for NB-PDCCH

- REG is not defined for NB-PDCCH

Agreements:

- Within CCE resource(s) for NB-PDCCH transmission, a SFBC pair is transmitted consecutively available two REs which is within a OFDM symbol

  - REs within SFBC pair can be separated at most one tone

  - LTE CRS (if present) is rate matched

  - NB-RS  is rate matched

    - This has no implication of NB-RS patterns

  - FFS: handling of CSI-RS REs

Agreements:

- For NB-IoT,

  - One CCE consists of 6 subcarriers per OFDM symbol in a subframe

  - NB-PDCCH and NB-PDSCH are multiplexed only based on TDM at subframe level at least per UE

    - It means that only cross subframe scheduling is supported

  - Different NB-PDCCHs:

    - TDM at subframe level for extended and extreme coverage.

    - Can be multiplexed in one subframe for normal coverage

    - FFS: FDM+TDM

# NB-CCE mapping

As agreed in previous meeting, 2 CCEs are defined within one PRB pair. Moreover, one NB-CCE consists of 6 subcarriers per OFDM symbol in a subframe. If interleaved mapping between two NB-CCEs is introduced, marginal frequency or time diversity gain would be expected due to one NB-CCE is only mapped within one RB pair. Moreover, the specification needs significant work to determine the exact interleaved pattern for two NB-CCEs within one PRB pair. Considering limited time for NB-IoT WI and trivial gain for interleaved NB-CCE mapping, it is desirable that one NB-CCE consists of 6 fixed consecutive subcarriers in a subframe. For example, as shown in the Figure below, NB CCE #0 consists of subcarriers #0~#5, and NB CCE #1 consists of subcarriers #6~#11.

Figure 1: NB-CCE mapping

The eNB can inform CSI-RS configuration to NB-IoT UEs. For the REs occupied by CSI-RS, the eNB can rate-match control information transmission around these occupied REs.

Proposal 1: One NB-CCE consists of REs corresponding to 6 fixed consecutive subcarriers in a subframe.

# NB-PDCCH mapping

Although one NB-PDCCH could be mapped on multiple subframes to support higher aggregation level, the time diversity gain may be limited considering low mobility of NB-IoT UEs. Moreover, the low code rate of control information transmitted within one subframe can be kept due to compact DCI used for NB-IoT. As a result, NB-PDCCH mapped on multiple subframes to support higher aggregation level is similar to NB-PDCCH repetition.

If one NB-PDCCH is mapped to multiple subframes, a NB-IoT UE has to wait until the final subframe to decode the NB-PDCCH, while a single subframe mapping allows in-place accumulation and thus faster decoding . It should also have some similar performance characteristics to legacy PDCCH.Thus it is preferred that one NB-PDCCH can only be mapped within one subframe, and NB-PDCCH can be repeated within multiple subframes to further improve performance.

Proposal 2: Each NB-PDCCH repetition is mapped into CCEs within one subframe.

Proposal 3: One NB-PDCCH can be repeated within multiple subframes.

The following figure gives an example to show how one NB-PDCCH is mapped (i.e., rate matching) within one subframe, and the NB-PDCCH can repeat within four subframes. The repeated NB-PDCCH within four subframes constructs one NB-PDCCH candidate.

Figure 2: Illustration of NB-PDCCH mapping and repetition

Further, when one NB-PDCCH is only mapped within one subframe, one or two NB-CCEs can be used for NB-PDCCH mapping. For the case of no NB-PDCCH repetition, the aggregation level can be one or two for NB-PDCCH mapping. For NB-PDCCH repetition, the aggregation level could be always 2 to simplify the standardization work.

Proposal 4: For NB-PDCCH repetition, aggregation level 2 is always assumed.

For normal coverage or extended coverage (144 dB or 154 dB MCL respectively), in view of the repetition number would not be very large, the NB-PDCCH can be transmitted within consecutive (valid) subframes. Moreover, the blocking from NB-PDCCH transmission to other DL channels could be avoidable via eNB’s management.

For extreme coverage, in view of the repetition number would be very large, one NB-PDCCH transmission would block other DL transmission in a longer time. In this case, the NB-PDCCH can be discontinuous transmission to reduce the blocking to other DL transmission.

As shown in the Figure below, NB-PDCCHs 1~4 are used for normal coverage or extended coverage, which are respectively transmitted within N consecutive subframes. NB-PDCCH 5 is used for extreme coverage, which is repeated in 4 discontinuous blocks (each block consists of N consecutive subframes). N can be the same for all coverage levels to make multiplexing among different coverage class more straightforward.

Figure 2: NB-PDCCH repetition

Proposal 5: For large NB-PDCCH repetition, NB-PDCCH can be discontinuously transmitted.

# NB-PDCCHs multiplexing

In last meeting, it was agreed:

- Different NB-PDCCHs:

  - TDM at subframe level for extended and extreme coverage.

  - Can be multiplexed in one subframe for normal coverage

  - FFS: FDM+TDM

That is, TDM at subframe level is supported for different NB-PDCCHs for extended and extreme coverage. FDM (i.e., NB-CCE level multiplexing) can be supported for normal coverage. As analyzed in section 3, when NB-PDCCH is repeated, two NB-CCEs can be always used, which means TDM is always used for different NB-PDCCHs in repetition. For two NB-PDCCHs each transmitted with one NB-CCE aggregation level, these two NB-PDCCHs can be multiplexed within one subframe.

Proposal 6: Only TDM is supported for different NB-PDCCHs in repetition. FDM is only supported for a NB-PDCCH without repetition.

# Conclusions

In this contribution, further considerations on NB-PDCCH mapping are provided, and the following proposals are given:

Proposal 1: One NB-CCE consists of REs corresponding to 6 fixed consecutive subcarriers in a subframe.

Proposal 2: Each NB-PDCCH repetition is mapped into CCEs within one subframe.

Proposal 3: One NB-PDCCH can be repeated within multiple subframes.

Proposal 4: For NB-PDCCH repetition, aggregation level 2 is always assumed.

Proposal 5: For large NB-PDCCH repetition, NB-PDCCH can be discontinuously transmitted.

Proposal 6: Only TDM is supported for different NB-PDCCHs in repetition. FDM is only supported for a NB-PDCCH without repetition.

# References

3GPP Chairman notes, RAN1 NB-IoT ad hoc, Budapest, Hungary, January 2016.