3GPP TSG RAN WG1 #84	                      	R1-160822

Malta, 15th - 19th February 2016

Agenda Item:  7.2.1.1.5

Source:	Spreadtrum Communications

Title:	Performance evaluation of NB-PDSCH

Document for:	Discussion and decision

# Introduction

In our companion contribution [1] we propose that CRS can be utilized in conjunction with NB-RS to improve channel estimation for NB-PDCCH/PDSCH reception. In this contribution, performance of PDSCH utilizing CRS in addition to NB-RS is provided.

# Discussion

Link level simulations has been performed for the following two cases:

Case 1: baseline case, channel estimation utilizing NB-RS only,

Case 2: channel estimation utilizing NB-RS in conjunction with CRS

The NB-RS pattern used in the simulation is defined in [2] and is shown in figure 1 for convenience. NB-PDSCH transport block of 96 bits is transmitted during 6 subframes and repeated 40 times and the packet error rate is measured. The detailed simulation assumptions are listed in Table 1.

Figure 1 NB-RS pattern utilized in the simulation

The mean square error of channel estimates is shown in Figure 2 and packet error ratio of NB-PDSCH in Figure 3. Results are shown for both case 1 and case 2. It can be seen from Figure 2 that about 3dB gain in channel estimate mean square error can be provided by combining NB-RS and CRS than NB-RS only. In terms of packet error rate, about 1.5 - 2dB gain is observed at 10 % BLER as shown in Figure 3. Therefore, from the simulation results it can be observed that utilizing CRS in conjunction with NB-RS can provide significant gain for in-band operation which is beneficial to save the UE’s power and improve the system efficiency.

Observation 1: Utilizing CRS in conjunction with NB-RS can provide significant gain for PDSCH decoding.

Figure 2 channel estimate performance for case 1 and case 2

Figure 3 Performance of PDSCH for case 1 and case 2

# Conclusions

In this contribution we present the performance of PDSCH utilizing CRS in addition to NB-RS. Based on the above discussion, we have the following observation:

Observation 1: Utilizing CRS in conjunction with NB-RS can provide significant gain for PDSCH decoding.

# Reference

- R1-160820, “Further discussion on the reference signal design for NB-IoT”, Spreadtrum Communications, 3GPP TSG RAN WG1 #84, Malta, Feb. 15-19, 2016.

- R1-160001, “Downlink reference signal design for NB-IoT”, Nokia Networks, TSG RAN WG1 NB-IoT Adhoc, Budapest, Hungary, Jan. 18-20, 2016

# Annex A

Table 1 Simulation assumptions

| Parameter | Value |
| --- | --- |
| Frequency band | 900MHz |
| Channel model | Typical Urban |
| Doppler spread | 1 Hz |
| Frequency error | Uniform distribution [-50,50] |
| Antenna configuration | 1 Tx, 1 Rx |
| Power offset between NB-RS and CRS | 0 dB |
| Cyclic prefix | Normal |
| Channel estimation | 2D MMSE |
| TB size | 96 bits (transmitted over 6 subframes) |
| Number of repetitions | 40 |