3GPP TSG-RAN WG1 Meeting #84 	R1-160952

# St Julian’s, Malta, 15th – 19th February 2016

Agenda item:		6.2.1

Source:	Huawei, HiSilicon

Title:	Overview of Multi-Carrier enhancements

Document for:		Discussion/Decision

# Introduction

In RAN#70 meeting, a new study item "Study on Multi-Carrier enhancements for UMTS " was approved [1]. The objective of this study is the following:

- Identify the scenarios for DB-DC-HSUPA/DC-HSUPA features and investigate mechanisms to improve coverage/capacity.

  - Study the potential deployment scenarios, e.g. scheduling 2ms+10ms TTI or 10ms+10ms TTI for uplink transmission with dual carrier.(RAN1, RAN2)

  - Investigate the performance for the enhanced scenarios and the impact on current mechanism, e.g. E-TFC selection, 2ms and 10ms switch, RRC measurement and handover.(RAN1, RAN2)

The study shall include considerations to minimize the impact on legacy terminals.

In this contribution we provide an overview of this study and discuss the deployment scenarios.

# Discussion

In Rel-13, Dual-Band UL carrier aggregation (DB DC-HSUPA) was standardized, extending the use of UL carrier aggregation across bands [1]. DB-DC-HSUPA is expected to enable better system/capacity performance and also spectrum deployment flexibility, e.g.,

- When using DB DC-HSDPA, UL carriers on the other band may be efficiently exploited, thus achieving a better UL load balancing.

- DC-HSUPA may not always be suitable for UL data aggregation, i.e. in case 10MHz of contiguous spectrum is not available in a single band (e.g. for some 900MHz networks/markets).

In Rel-13 specifications, for dual band dual carrier scenario only 2ms TTI can be configured simultaneously on both carriers (2ms + 2ms configuration). Considering the coverage difference caused by the path loss characteristics in different bands, the use of DB-DC-HSUPA with 2ms + 2ms configuration will be limited by the band whose coverage is smaller. In order to extend the use of DB-DC-HSUPA, 10ms TTI can be considered on the band with smaller coverage. Whether it is beneficial introducing 10ms TTI for DC-HSUPA (in the same band) is another potential scenario to be further studied.

## Target scenarios

## Dual band dual carrier scenario

In this scenario, the path loss in the two bands can be very different. Assume one band is on 900MHz and the other is on 2GHz. According to Section 5.1.4.2 in [2], the path loss for these two bands can be calculated as follows:

900MHz:		PL(R) = 120.9 + 37.6log10(R)

2GHz:			PL(R) = 128.1 + 37.6log10(R)

where R is the distance between the UE and the NodeB in km. It can be seen that at the same distance, the path loss difference for 900MHz and 2GHz is 7.2dB. This means that in order for the NodeB to receive the signal at the same power level on 900MHz and 2GHz bands, the UE needs to transmit with 7.2dB more power on 2GHz than on 900MHz.

Although more power is needed for transmission on the 2GHz band respect to 900 MHz band to compensate for the path loss, it may still be beneficial for the UE to be scheduled on the two carriers rather than on 900MHz alone for better UL load balancing. Rel-13 DB-DC-HSUPA feature enables the UE to be scheduled on both carriers and allows the UE to achieve a higher throughput respect to single carrier transmission.

In the current specifications, only 2ms TTI is allowed to be scheduled in the DB-DC scenario. As illustrated in Figure 1, the region for DB-DC-HSUPA is determined by the region for 2ms TTI on the 2GHz band. At point A, which is the limit for the 2ms TTI on 2GHz, the UE can obtain some throughputs improvements by being scheduled at 2GHz with DB-DC-HSUPA. Between A and B, however, using 2ms TTI at 2GHz may not be a good option because this region may be better suitable for 10ms TTI deployment. One of the scenarios of interests is the region AB, where 2ms (at 900MHz) + 10ms (at 2GHz) configuration can be used. The gains from using the 2ms + 10ms configuration instead of the 2ms + 2ms configuration in this region need to be studied.

Figure 1 Scenarios for 2ms + 10ms and 10ms + 10ms

It is also shown in Figure 1 that there may be a region BC, where 10ms (at 900MHz) + 10ms (at 2GHz) is beneficial. How much benefit can be obtained by allowing the 10ms + 10ms configuration needs to be studied.

## Dual carrier scenario

If the two carriers are deployed within the same band, the path loss on each carrier is almost the same and obviously there is no coverage difference for 2ms TTI between the two carriers. As a result, the use case for 2ms + 10ms configuration does not exist in this scenario.

Since the 10ms TTI coverage is better than 2ms TTI, it is possible to extend the coverage of a certain throughput on both carriers with 10ms TTI. The scenario of 10ms + 10ms is illustrated in Figure 2. The benefits of using 10ms + 10ms in region AB in Figure 2 need to be studied.

Figure 2 Scenario for 10ms + 10ms

## Summary of target scenarios

In summary, there are three scenarios that are of interest for this study on multi-carrier enhancements:

Scenario 1: 2ms + 10ms configuration for DB-DC scenario

Scenario 2: 10ms + 10ms configuration for DB-DC scenario

Scenario 3: 10ms + 10ms configuration for DC scenario

## Performance evaluations

The uplink performance in the new scenarios is to be evaluated during this study item. In [3], evaluation methodologies are proposed, considering the path loss difference between different bands. In addition, initial evaluation results are provided in [3], showing the potential gains in the uplink performance.

## Impacts on current mechanism

If additional 10ms TTI configurations are introduced in DB-DC-HSUPA/DC-HSUPA, RAN1, RAN2 and RAN3 specifications need to be modified to allow the new configurations. This would impact some existing mechanisms as well, including E-TFC selection, 2ms TTI and 10ms TTI switch, RRC measurements and handover, etc. Other impacts to the specifications may depend on the specific solutions that will be introduced for DB-DC-HSUPA/DC-HSUPA operation.

# Conclusion

This contribution presents an overview of the study in multi-carrier enhancements for UMTS. Target scenarios in DB-DC scenario and DC scenario have been discussed and three scenarios have been identified:

Scenario 1: 2ms + 10ms configuration for DB-DC scenario

Scenario 2: 10ms + 10ms configuration for DB-DC scenario

Scenario 3: 10ms + 10ms configuration for DC scenario

The benefits achievable in the identified scenarios are to be further studied. It is proposed:

Proposal 1: Agree on the target scenarios for multi-carrier enhancements for UMTS.

Proposal 2: Capture the discussion of target scenarios in the technical report.

# References

- RP-152290, New SID: Study on Multi-Carrier Enhancements for UMTS

- 3GPP TR 25.942, v10.0.0, Radio Frequency (RF) system scenarios

- R1-160953, Evaluation methodology and results for Multi-Carrier enhancements, Huawei, HiSilicon