3GPP TSG RAN WG1 #85	R1-164007

# Nanjing, China, 23th – 27th May 2016

Agenda item:	7.1.5.1

Source: 	Samsung

Title: 	Flexibility of LDPC - Length, Rate and IR-HARQ

Document for:	Discussion and Decision

Introduction

In the RAN1 #84bis meeting, it was agreed that the selection of NR channel coding scheme(s) will consider performance, implementation complexity, latency and flexibility. With regards to flexibility, efficient support of variable code length, code rate and HARQ is to be taken into account. During the discussion on NR channel coding scheme(s) in RAN1#84bis, there was some debate on whether or not LDPC codes and relevant specification details can be designed to support variable code block sizes, code rates, and efficient incremental redundancy (IR) based HARQ operation. The purpose of this contribution is to show that these concerns do not take into account the latest developments in LDPC codes.

Although 3GPP has not yet adopted LDPC codes as part of its standard, LDPC codes have been extensively evaluated, enhanced, and deployed in many of the wireless systems ranging from WiFi to digital broadcasting. As part of the research on LDPC codes, there were extensive investigations on the design aspects of LDPC codes with various code length compatibility and code rate compatibility. As a result, the state of the art design on LDPC codes can fully support IR based HARQ while maintaining comparable performance as turbo codes.

This contribution discusses the design aspects of LDPC codes so as to demonstrate that with proper design, LDPC codes can meet all the operational requirements needed in cellular communications. In particular, we will show the flexibility aspects of LDPC as well as the support of IR based HARQ.

Quasi Cyclic LDPC code

General QC LDPC Code description

A QC LDPC code is characterized by the parity-check matrix which consists of small square blocks which are the zero matrix or circulant permutation (right-shifted identity) matrices. Let P=pij (0≤i,j<Z) be the Z×Z permutation matrix defined by

pij=1if i+1≡j mod Z0otherwise      or    P=001001⋯00⋮0⋮0⋮0⋯⋮11000.

Note that Pi is just the circulant permutation matrix which shifts the identity matrix I(=P0) to the right by i times for any integer i, 0≤i<Z. For simple notation, we denote the Z×Z zero matrix OZ×Z by P-1.

Let H be the mZ×nZ matrix given by

H=Pa11Pa21Pa12Pa22⋯⋯Pa1nPa2n⋮Pam1⋮Pam2⋱⋯⋮Pamn

where aij∈{-1, 0, 1, 2, …, Z-1} are exponent indices of permutation matrices, n and m are the numbers of column and row blocks, respectively. Then the code with parity-check matrix H is referred to as a quasi-cyclic (QC) LDPC code. Furthermore, let E(H) be the expoment matrix of H given by

EH=a11a21a12a22⋯⋯a1na2n⋮am1⋮am2⋱⋯⋮amn.

An example of a parity-check matrix for a (8Z, 4Z) QC LDPC code with n=8 and m=4 is given by

H=P0OOP1OP0P4OOP2P3OOP6P5OP1P0OP0OOP0OP0OP1OP0P0OP0,   E(H)=0-1-11-104-1-123-1-165-110-10-1-10-10-11-100-10

where O is the Z×Z zero matrix and P0 is the Z×Z identity matrix.

Lifting for supporting variable length

One of advantages of QC LDPC code is supporting length compatibility. QC LDPC code of variable length can be easily obtained by adjusting the size of circulant permutation matrices in H. When adjusting the size of circulant permutation matrices according to the target code block size, each exponent indices can be easily calculated by the specified formula, such as modulo function (aij mod Z), not independently defined for each code block size [1]. For example, we can obtain the exponent matrix EHZ for the parity-check matrix HZ from the exponent matrix EH=aij for the parity-check matrix H as follows:

[Lifting method]

EHZ=aij(Z)    s.t.   aij(Z)=aij,aij≤0aij  mod Z,aij>0.

Here, HZ is the parity-check matrix consisting of Z×Z circulant permutation matrices and/or zero matrices for given integer Z and can be derived from EHZ. Figure 1 provides demonstrating the lifting technique to support code length compatibility in an efficient way.

Figure 1. Lifting technique for length compatibility

Observation 1: QC LDPC codes can support variable code length in an efficient way

Flexibility of code length and code rate

In this section, we introduce several technologies to support variable code lengths and code rates. Similar with channel coding process in LTE standard [2], segmentation, shortening and puncturing (or repetition) will be applied to accommodate the encoding of various transport block lengths and code rates. In addition to this, as a part of the methods to support variable code lengths, the lifting technique can be applied as introduced in section 2.2.

Segmentation

Segmentation is a basic technique to support input bits longer than the maximum information size for channel coding. In LTE standard, segmentation of the transport block should be performed when the transport block size is larger than the maximum information size (= 6144) of a turbo code. Similarly to this, segmentation of the information block can be applied to LDPC codes when the information block size (=Kinput) is larger than maximum information size of a LDPC code (= KLDPC_max) as illustrated in Figure 2. The information block is segmented into two or more codeblocks of which length is less than or equal to maximum information length of LDPC codes.

Figure 2. Example of segmentation method

Observation 2: For LDPC coding, segmentation can be applied as a basic technique to support variable lengths

Shortening

Shortening is an efficient way to fit the number of information size for channel coding to a given length by zero-insertion to some information bits. LTE turbo code also adopts zero-insertion technique as a shortening method to adjust to the QPP interleaver size. Similarly to this, after the segmentation and lifting with Z×Z submatrix, if the codeblock size (= K) is smaller than the maximum information size (=KLDPC(Z)) for the lifted QC LDPC code, then zeros are inserted to (KLDPC(Z) - K ) bits and not transmitted, which is also known to the receiver.

Observation 3: LDPC codes enable variable codeblock length by shortening

Puncturing and Repetition

Supporting variable code rates would be necessary since the number of bits for transmission varies based on the available resources. In addition, rate compatibility is important to achieve high throughput in HARQ (Hybrid Automatic Repeat Request) systems. After the code shortening is applied for supporting variable codeblock length, the puncturing or repetition is needed to adjust the code rate. Figure 3 shows the concept of shortening and puncturing or repetition operation to support variable code length and code rate, similarly as in IEEE 802.11n [3].

Figure 3. Shortening, Puncturing and repetition

Observation 4: LDPC codes enable variable code rate by puncturing or repetition

Performance evaluation

To evaluate the flexibility of QC LDPC codes, we set the simulation conditions in the following:

| Evaluate the block error rate (BLER) performance versus SNR |  |  |
| --- | --- | --- |
| Channel | AWGN |  |
| Modulation | QPSK, 64QAM |  |
| Coding Scheme [1] | Turbo | LDPC |
| Code rate | 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |  |
| Decoding algorithm | Log-MAP (Number of iteration = 6) | Layered Brief Propagation (Number of iteration = 15) |
| Info. block length (bits w/o CRC) | 100, 400, 1000, 2000, 4000, 6000, 8000 |  |

Table 1. Simulation conditions for turbo codes and LDPC codes

Figures 4 and 5 show the required SNR to achieve BLER of 0.1 in simulations of LDPC codes proposed in [1] and turbo codes for various information lengths when combinations of code rate and modulation are 1/2 QPSK and 2/3 64 QAM, respectively. Firstly, Figures 4 and 5 show the performance gap between LDPC and turbo codes is less than 0.3 dB, and so they have comparable performance. Moreover, the performance of LDPC code is gracefully improving as the information length goes larger, and therefore, it is expected that LDPC code can provide the code length flexibility as similar with turbo code.

Figure 4. Required SNR to achieve BLER =0.1 vs. information length (R=1/2, QPSK)

Figure 5. Required SNR to achieve BLER =0.1 vs. information length (R=2/3, 64QAM)

Figures 6 and 7 show the required SNR to achieve BLER of 0.1 in simulations of LDPC codes proposed in [1] turbo codes for various code rates when the information length is 6000 and modulation is QPSK and 64 QAM, respectively. Firstly, Figures 6 and 7 show the performance gap between LDPC and turbo codes is negligible, and so they have comparable performance. Moreover, the performance of LDPC code degrades gracefully with puncturing, and therefore, it is expected that LDPC codes can provide the code rate flexibility as similar with turbo code.

Figure 6. Required SNR vs. Code Rate vs. information length (K=6000, QPSK)

Figure 7. Required SNR vs. Code Rate vs. information length (K=6000, 64QAM)

Based on this evaluation, two aspects should be noted in comparing the turbo and LDPC codes. They have comparable performance and LDPC codes can support the code length and code rate flexibility as similar with turbo code.

Observation 5: LDPC codes and relevant specification details can be designed to support variable code block lengths and code rates

Proposal 1: Make consensus on that LDPC codes can support variable code block lengths and code rates

IR-HARQ

In this clause, we will show that an example of LDPC codes with single parity-check extension efficiently support IR-HARQ operation.

Code extension for IR-HARQ

Single-parity check code extension from the higher rate code could be a good candidate to support IR-HARQ. This structure allows us to have a good performance not only at higher code rate but also at lower code rate [4]. For the first transmission, LDPC codes with high code rate can be efficiently encoded and decoded by using only smaller portion of parity check matrix. For lower code rate, higher coding gain is guaranteed that makes successful decoding with retransmission(s). Consequently, considering the IR-HARQ, LDPC codes with the single-parity check code extension is a candidate to guarantee an operational efficiency and good performance. Figure 8 depicts an example of a parity check matrix with the single-parity check extension.

Figure 8. Example of LDPC codes with single-parity check code extension

Observation 6: LDPC codes with the single-parity check code extension can fully support IR HARQ in efficient way

Performance evaluation

To evaluate the IR-HARQ performance of QC LDPC codes, we set the simulation conditions in the following:

| Evaluate the block error rate (BLER) performance versus SNR |  |  |
| --- | --- | --- |
| Channel | AWGN |  |
| Coding Scheme [1] | Turbo | LDPC |
| Code rate and Modulation at first transmission | (1/2, QPSK) (8/9, 64QAM) |  |
| Decoding algorithm | Log-MAP (Number of iteration = 6) | Layered sum-product (Number of iteration = 15) |
| Info. block length (bits w/o CRC) | 6000 |  |

Table 2. Simulation conditions for LDPC codes and LTE turbo codes

Figure 7 and 8 show the spectral efficiency of turbo codes and LDPC codes, we have designed and the codes from [1], when the first transmission code rate and modulation is 1/2 QPSK and 8/9 64 QAM, respectively. IR-HARQ for LDPC codes with 1st transmission corresponding to rate R and n-th retransmission corresponding to rate R/n. The number of maximum retransmission is four. The performance of LDPC codes is better than that of turbo codes by about 0.2 dB, and thus they are comparable. Therefore, rate compatible LDPC code with single-parity check extension can be efficiently operated in IR-HARQ through the use of puncturing and repetition.

Figure 9. Spectral efficiency (1st transmission: 1/2 QPSK)

Figure 10. Spectral efficiency (1st transmission: 8/9 64QAM)

Observation 7: LDPC codes and relevant specification details can be designed to support IR based HARQ

Proposal 2: Make consensus on that LDPC codes can support IR-HARQ

Conclusions

In this contribution, we provide the design aspects of LDPC codes so as to fully support the code length and code rate flexibility as well as the IR based HARQ. Based on our analysis, we have the following observation and proposal:

Observation 1: QC LDPC codes can support variable code length in an efficient way

Observation 2: For LDPC coding, segmentation can be applied as a basic technique to support variable lengths

Observation 3: LDPC codes enable variable codeblock length by shortening

Observation 4: LDPC codes enable variable code rate by puncturing or repetition

Observation 5: LDPC codes and relevant specification details can be designed to support variable code block lengths and code rates

Observation 6: LDPC codes with the single parity check code extension can fully support IR HARQ in efficient way

Observation 7: LDPC codes and relevant specification details can be designed to support IR based HARQ

Proposal 1: Make consensus on that LDPC codes can fully support variable code block lengths and code rates

Proposal 2: Make consensus on that LDPC codes can fully support IR-HARQ

- References

- R1-164812, Samsung, "Preliminary evaluation results on new channel coding scheme for NR-LDPC code for high throughput," 3GPP TSG RAN WG1 #85, Nanjing, china, 23-27 May 2016.

- 3GPP, TS 36.212, “E-UTRA; Multiplexing and channel coding (Release 8)

- IEEE Std 802.11n™-2012, “Part 11: Wireless LAN medium access control (MAC) and physical layer (PHY) specifications.”

- T.-Y. Chen, K. Vakilinia, D. Divsalar, and R. D. Wesel, "Protograph-based Raptor-like LDPC codes," IEEE Trans. Commun., vol. 63, no. 5, pp. 1522-1532, May 2015.