3GPP TSG RAN WG1 Meeting #85	R1-164043

Nanjing, China 23rd – 27th May 2016

Agenda Item:	7.1.7

Source:	Huawei, HiSilicon

Title:	Considerations on vehicular communications for NR

Document for:	Discussion and decision

# Introduction

At the RAN#71 meeting, a SI on 5G was adopted [1] with one of the following objectives:

- Target a single technical framework addressing all usage scenarios, requirements and deployment scenarios defined in TR38.913 including

  - Enhanced mobile broadband

  - Massive machine-type-communications

  - Ultra reliable and low latency communications

SA1 also started discussions on eV2X for Rel-15 [2]. It was proposed for SA1 to specify service requirements to enhance 3GPP support for V2X service. A major impetus is due to the development of self-driving cars by various companies. It is anticipated that the first self-driving cars will be commercially available around 2020, with mass-scale adoption around 2025 (e.g., [3]).

Given the timeline for 5G deployment, it is thus necessary to consider the implications of connected / self-driving cars in the early stages of 5G to ensure that the wireless industry can meet the needs of the connected vehicles in the next decade.

In this contribution, which is an extension of [4], we discuss specific problems that need to be addressed by RAN1 to support self-driving / connected cars for the NR. In particular, self-driving cars are expected to be very demanding in terms of constraints on the cellular system, thus deserve particular attention and specific evaluation scenarios.

# The connected car and 5G

Generally speaking, 5G has been described as encompassing three different sets of requirements. For instance, SA1 defines the following:

- TR22.861 describes massive Internet of Things [5]

- TR22.862 describes critical communications [6]

- TR22.863 describes enhanced mobile communication [7]

It can be argued that the connected car is the intersection of these three sets, as shown in Figure 1. For the autonomous driving part, the connected car is relevant to IoT since it send/receives messages from the infrastructure and other cars without human intervention. It is a form of critical communications since very low latency and extremely high reliability are needed to ensure safety of the “driver” and passengers. Finally, since the driver relieved from driving duties, he/she will likely consume tremendous amounts of data (e.g., watching movies, working).

Figure 1. Connected cars within the 5G ecosystem

Observation 1: the connected car is relevant for all 3 sets of requirements identified by SA1.

Vehicular traffic is characterized by varying mobility patterns and density over both space and time [9]. The vehicle density can vary greatly between the areas of a traffic jam and nearby areas of flowing traffic. At different times of the day, a road location could experience both traffic extremes (i.e., very sparse and very dense vehicular traffic). From the communications network perspective, enabling high reliability without over-provisioning resources in such a highly dynamic network is a challenge.

Observation 2: NR networks will need to dynamically allocate the resources based on the vehicular traffic density and data traffic demands.

In addition, the connected car presents some challenges that are uniquely relevant for RAN1. First, wide area coverage is needed, with no/very little coverage holes. Second, at least the safety part needs to be operational under out-of-coverage conditions. Finally, it is an application that requires high data rates in high mobility conditions.

Observation 3: from a RAN1 perspective, the connected car is a challenging use case.

Note also that when under network control, the connected car will likely be in the RRC_CONNECTED state. This may have significant impact on the design of both higher layers and physical layer: at the physical layer, the eNB (RSU) may be required to send many control messages (e.g., allocation grants); as a result, there may be some capacity constraints on the control channels.

# Required capacity demand from the autonomous car

The traffic generated by autonomous cars is anticipated to be very high for downlink, uplink, and sidelink and will grow as newer features are developed. As reported in Appendix B of TR36.885 [11], even meeting the traffic demand for “basic” V2V services is challenging for the urban 15 environment scenario. Note that this scenario is not a difficult environment in terms of traffic demand, and it is anticipated that there are many deployments where higher traffic densities can be observed. Note also that the V2X SI does not consider autonomous driving.

Autonomous driving will significantly increase traffic demand due to enabling new features and functions. For instance, the “platooning” feature requires substantial data exchange between cars. Traffic regulation at an intersection will require information exchange between vehicles as well as between vehicles and infrastructure. Perhaps more, the demand for “infotainment” services will soar: the average commute time is around 1 hour/day. The driver now has an hour of time to work, watch videos, etc.; increasing traffic demand. Note in particular that the increase in demand will be in regions that are not considered hotspots, but are generally covered by macro eNBs, where the demand is currently viewed as low.

- Observation 4: the traffic demand from connected cars is anticipated to be very high.

Meeting the traffic demand from connected cars will be challenging. It will rely on both network-to-car communications, and car-to-car (V2V) communications.

## Network-to-car communications

In a RAN2 contribution [10], we described how the network can accommodate the traffic demand through the use of service-aware RAN and the massive scale deployment of Road Side Units (RSUs). RSUs are discussed in the V2X SI for safety applications. The RSU will also be used for sending traffic data. The concept of RSU is shown in Figure 2.The RSU can also be serve as a hotspot for infotainment.

Figure 2. Example of RSU deployment

Deploying these RSUs will be challenging:

- Given that they act as hotspots for traffic delivery, using millimeter wave (mmW) transmission seems appropriate, including the deployment of mmW beamforming in high mobility conditions.

- Given that RSUs would be deployed on a large scale, it is unlikely that every RSU will be connected to the network via wire. Thus, wireless backhaul techniques and protocols will need to be standardized.

Consequently, we propose the following:

Proposal 1: RAN1 needs to study how to communicate with RSUs:

  - From/to the RSU to/from the car

  - From/to the eNB to/from the RSU

In some conditions, a car that is out of coverage from the RSU could still benefit from communicating with it, either from a safety or entertainment standpoint. Thus, relaying mechanisms from a car to a RSU should be considered. Note that for LTE-V2X in Rel-14, a RSU is an application-layer functionality and is not defined as a 3GPP logical node. For NR, it remains to be seen (and studied by other 3GPP WGs) if a logical node encompassing the RSU functionalities needs to be defined.

## Car-to-car communications

In order to facilitate communications between connected cars and to alleviate the load on the cellular systems, the V2V component of the connected car solution needs to be standardized:

- V2V can provide significant safety benefits, and is being standardized in Rel-14 for LTE. This effort does not include autonomous driving. V2V is expected to be a crucial component of the autonomous car: it is a means to provide information farther than what the sensors can detect, and provides information for some environments (NLOS conditions, such as in an urban intersection).

- V2V can also provide high-throughput, low latency safety and traffic efficiency applications between cars.

In order to provide both safety and traffic management services for the connected car, high-throughput, high reliability, low-latency V2V links will need to be supported. They will require improvements from what is being standardized in Rel-14: in particular, much higher bit rates will be needed. This may require the deployment of MIMO techniques, either in mmW or microwave domain. Relatively large dimensions of vehicles allow antenna placement that can enable spatial diversity and interference reduction for particular use cases. Having multiple antennas placed on different locations on a vehicle can ensure that shadowing caused by other vehicles is alleviated or even mitigated. In addition to antennas on the roof, vehicles can employ bumper antennas for directional bumper-to-bumper communication for specific applications such as platooning, thus reducing the interference on other antennas.

In addition, it is likely that the current broadcast mechanism will not be sufficient. Unicast links will need to be supported, with feedback from the destination to the source.

Proposal 2: RAN1 studies MIMO techniques and unicast transmission with feedback for high bit rate V2V techniques.

TDD may be more suitable than FDD for the V2V link since it would reduce the need for CSI feedback.

## RAN1 evaluation work

As described, the connected car requires RSU nodes and a strong V2V component. In addition, the connected car is a challenging 5G application in terms of latency, mobility, and coverage. It is thus necessary for RAN1 to conduct a thorough analysis of the connected car to make sure that 5G can meet the needs of this market. In addition to basic scenarios and KPIs for V2X discussed in [8], future connected car scenarios (e.g., cooperative lane merging [14], intersection control [15], platooning [16]) have considerably more stringent KPIs. Therefore, RAN1 should define a set of challenging simulation evaluation scenarios, detailed KPIs, and evaluation methodology (in particular, realistic channel models) for the connected cars covering all anticipated deployment scenarios, as proposed in [12].

Proposal 3: RAN1 to define evaluation scenarios, methodology, parameters, and channel models in order to evaluate communication with the connected car.

# Flexible frame and adaptive transmission for ultra-reliable V2X communication

Improvement in safety will be a key requirement for a large number of use cases of the connected car. For such kinds of critical V2X communication, ultra-reliable low-latency communication will be required to guarantee QoS. However, providing a certain amount of data with a very high reliability and within a predefined time limit will be challenging, especially due to the highly time-variant and dispersive radio channel, potentially insufficient coverage and dynamic network topology. Targeting ultra-reliable low-latency transmission on L1 by using a frame structure as in legacy LTE cellular systems faces following shortcomings:

- Long frame duration, which does not meet low latency requirements but usually does allow packet retransmissions.

- Significant drop of the spectral efficiency due to the required robust modulation and coding schemes to ensure low enough packet error rates.

- High overhead for feedback and signaling for adapting to the highly time-variant V2X channels.

Therefore, the frame structure and transmission scheme should be flexible in order to meet reliability and latency requirements without sacrificing spectral efficiency while being adjustable to the V2X channel conditions. Some analysis of the reliability-latency tradeoffs and a flexible pilot allocation method can be found in [17]. To reduce feedback and signaling overhead, transmission schemes using channel reciprocity during short consecutive bidirectional V2V transmissions are good candidates for deployment.

Observation 5: legacy cellular frame structure is suboptimal for ultra-reliable low-latency V2X communication and cannot address latency requirements for the connected car.

Therefore, we propose the following [13]:

Proposal 4: RAN1 needs to investigate a flexible frame structure and adaptive transmission schemes for ultra-reliable low-latency communication over V2X channels.

# Interworking between 5G NR and LTE

As stated in the previous sections, for connected car applications wide area coverage is needed, with no/very little coverage gaps. This is especially true for safety applications and cooperative driving. However, the initial deployment of 5G NR base stations will not have adequate coverage. Consequently, it is necessary to consider inter-working between 5G NR and LTE so as to provide ubiquitous services to vehicles even in situations that 5G NR is not available.

It is envisioned that the autonomous car will have multiple radios, including GNSS/GPS, NR, 4G LTE, and 802.11p, as shown in Figure 3. These radios are connected to the application layers through a connection manager that selects (or receives) data accordingly.

- The GNSS/GPS provides location and position information as well as timing for the radios.

- The 802.11p radio provides legacy service because there may be many vehicles with 802.11p radios when deployment of NR begins.

- The LTE radio can provide legacy V2V service as well as additional support through LTE.

Automated car app

Infotainment app

middleware

Connection manager

GNSS / GPS

NR

4G

802.11p

Figure 3. Radios for connected car

It is likely that the current DSRC protocol will be enhanced to support the increasing features needed for the autonomous car. Although the protocol is beyond the scope of RAN1, the effects of the enhancements will primary impact the NR because 802.11p and 4G are unlikely to meet the physical layer requirements of larger payloads, lower latency, and better performance.

Observation 6: Interworking between 5G NR and LTE is needed so as to provide ubiquitous services to vehicles.

Proposal 5: To consider interworking between 5G NR and LTE (including both Uu and sidelink) for vehicle communications in the study.

# Coordination with SA1

Sections above give considerations from RAN1 view. As have been mentioned that SA1 has also started discussion on eV2X for Rel-15. In order not to have duplicated and possibly conflicting work from different working groups, we suggest RAN1 to align with or wait SA1 work for connected cars.

Proposal 6: RAN1 to align with or wait SA1 work for connected cars.

# Conclusions

Connected cars were discussed for 5G services. The following observations were drawn:

Observation 1: the connected car is relevant for all 3 sets of requirements identified by SA1

Observation 2: NR networks will need to dynamically allocate the resources based on the vehicular traffic density and data traffic demands.

Observation 3: from a RAN1 perspective, the connected car is a challenging use case.

Observation 4: the traffic demand from connected cars is anticipated to be very high.

Observation 5: legacy cellular frame structure is suboptimal for ultra-reliable low-latency V2X communication and cannot address latency requirements for the connected car.

Observation 6: Interworking between 5G NR and LTE is needed so as to provide ubiquitous services to vehicles.

We propose the following:

Proposal 1: RAN1 needs to study how to communicate with RSUs:

  - From/to the RSU to/from the car

  - From/to the eNB to/from the RSU

Proposal 2: RAN1 studies MIMO techniques and unicast transmission with feedback for high bit rate V2V techniques.

Proposal 3: RAN1 to define evaluation scenarios, methodology, parameters, and channel models in order to evaluate communication with the connected car.

Proposal 4: RAN1 needs to investigate a flexible frame structure and adaptive transmission schemes for ultra-reliable low-latency communication over V2X channels.

Proposal 5: To consider interworking between 5G NR and LTE (including both Uu and sidelink) for vehicle communications in the study.

Proposal 6: RAN1 to align with or wait SA1 work for connected cars.

# References

RP-160671, “Study on NR New Radio Access Technology.”

S1-160111, “Preparations for eV2X Rel-15”, LG Electronics Inc.

http://www.businessinsider.com/report-10-million-self-driving-cars-will-be-on-the-road-by-2020-2015-5-6

R1-162167, “Considerations on vehicular communications for 5G”, Huawei, HiSilicon, RAN1#84b, Apr. 2016.

3GPP TR 22.861, “Feasibility Study on New Services and Markets Technology Enablers for Massive Internet of Things; Stage 1”, v0.3.0, Feb. 2016.

3GPP TR 22.862, “Feasibility Study on New Services and Markets Technology Enablers - Critical Communications; Stage 1”, v0.3.0, Feb. 2016.

3GPP TR 22.863, “Feasibility Study on New Services and Markets Technology Enablers - Enhanced Mobile Broadband; Stage 1”, v0.3.0, Feb. 2016.

3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”.

D. Helbing and B. A. Huberman, “Coherent moving states in highway traffic,” Nature, vol. 396, no. 6713, pp. 738–740, 1998.

R2-162787, “Emerging Vehicular Communication Applications”, Huawei, HiSilicon, RAN2#93bis, Apr. 2016.

3GPP TR 36.885, “Study on LTE-based V2X Services”

R1-164044, “Evaluation assumption for non-eMBB deployment scenarios”, Huawei, HiSilicon, RAN1#85, May 2016.

R1-164032, “Discussion on frame structure for NR”, Huawei, HiSilicon, RAN1#85, May 2016.

S1-152251, “Use Case Cooperative Lane Merging (CLM)”, 3GPP TSG-SA WG1 Meeting #71, Aug. 2015.

S1-152252, “Use Case Cooperative Longitudinal & Lateral Control for Collision Avoidance (CoLCA)”, 3GPP TSG-SA WG1 Meeting #71, Aug. 2015.

S1-152253, “Use Case Cooperative Short Distance Platooning (CoSdP)”, 3GPP TSG-SA WG1 Meeting #71, Aug. 2015.

M. Ibrahim and W. Xu, “Pilot allocation trade-off for low latency and high reliability transmission,” in Proc. 23rd Int. Conf. Telecommunications (ICT), Thessaloniki, Greece, 2016.