3GPP TSG RAN WG1 Meeting	#85										R1-164115

Nanjing, 23rd - 27th May 2016

### Agenda Item:	6.2.10.1

### Source:	Mitsubishi Electric

### Title:	Reducing UL RS overhead for sPUSCH

Document for:	Discussion

# Introduction

The study item on Latency reduction techniques [1] approved at RAN#67 aims at significantly reducing the packet data latency over the LTE Uu air interface for an active UE and the packet data transport round trip latency for UEs in connected state. From a RAN1 perspective, this can be achieved through TTI shortening and reduced processing time. With the prerequisite of preserving backwards compatibility by allowing operation of normal TTI and sTTI on the same carrier, RAN1 needs to:

- Assess specification impact and study feasibility and performance of TTI lengths between 0.5ms and one OFDM symbol, taking into account impact on reference signals and physical layer control signalling

Due to TTI shortening, reference signal redesign may be desirable in order to keep good channel estimation performance, or even mandatory to enable very short TTI lengths. Pilot symbols can be included with different periodicity in the time/frequency domain.

In this contribution, we assess means of UL DM-RS overhead reduction.

# Discussion

Up to Rel. 13, LTE uplink employs a fixed TTI length of 1ms with 12/14 SC-FDMA symbols for extended/normal CP. Full pilot symbols based on Zadoff-Chu sequences [2] are employed for avoiding any PAPR increase due to multiplexing data and pilot symbols within an SC-FDMA symbol. The presence of two reference signal symbols within a TTI is useful for tracking time-varying channels and for fine frequency offset estimation.

## UL RS design criteria for sTTI

Several problems related to UL RS design arise when shortening the TTI length. Design criteria should take into account channel estimation performance, pilot overhead and PAPR issues [3][4].

Pilot density in the time domain should be calibrated so as to offer sufficient channel estimation performance. For long sTTIs (e.g. 7 symbols), having two pilots in the sTTI to allow channel tracking might be necessary. For shorter sTTIs, in the cases when the UL DM-RS might be transmitted outside the sTTI boundary (given that the resource allocation remains the same for a UE along several sTTIs), the time lapse between the UL DM-RS and the sTTI should be limited to avoid channel estimation degradation. Channel estimation should be based on the UL DM-RS preceding the sTTI for obtaining latency reduction. Pilot density in the time domain may need to be increased with respect to non-sTTI pilot density.

Pilot density in the frequency domain may need to be relaxed with respect to non-sTTI to enable overhead reduction. Current design with full RS symbols leads to an excessive RS overhead, especially in the case of very short sTTI lengths and/or high mobility scenarios. When mixing data subcarriers and RS subcarriers in the same OFDM symbol, the impact on PAPR needs to be considered.

Observation 1:

Channel estimation performance, pilot overhead and PAPR need to be taken into account in UL DM-RS design for sPUSCH.

## UL DM-RS overhead reduction

To improve throughput, UL DM-RS overhead needs to be reduced. UL DM-RS overhead reduction can operate in the time or in the frequency domain.

### DM-RS sharing in the time domain

In the time domain, DM-RS overhead can be controlled through DM-RS sharing between several consecutive sTTIs for sTTI sizes below 7. The method of overhead reduction through DM-RS sharing is dependent on the sTTI length and the desired degree of scheduling flexibility.

For sTTIs of size 2, several consecutive sTTIs may use the same DM-RS for channel estimation, thus reducing DM-RS overhead. DM-RS position may be outside the sTTI boundary. An example is given in Figure 1. This method is only applicable with scheduling constraints (same resource allocation kept for several sTTIs). If the UL DM-RS position is not fixed (e.g. dynamic pilot insertion [5]), extra signalling is needed to convey the pilot position. The number of sTTIs sharing the same DM-RS is limited by the channel estimation performance (distance between the sTTI and the pilot position, especially at medium/high mobility, and/or number of multiplexed DM-RS onto the same position when the sTTIs belong to different UEs).

| a) Same user having DM-RS only in some sTTIs | b) Different users multiplexing DM-RS onto the same DM-RS position |
| --- | --- |

Figure 1 – Multiple sTTIs using the same DM-RS position for channel estimation

For sTTIs of size 3/4, DM-RS symbol can be shared by multiple sTTIs as in Figure 2. Sharing can be done if FDM or CDM. To keep the scheduling flexibility and allow successive sTTIs to have independent resource allocation with CDM sharing, the issue of orthogonality loss between DM-RS sharing the same position needs to be solved [6]. Pilot position at sTTI edge is not favourable for medium/high mobility.

Figure 2 - DM-RS symbol shared by multiple short-TTIs within the same subframe

### Hybrid pilot/data symbols

In the frequency domain, DM-RS overhead reduction can be achieved by allowing the use of hybrid symbols, mixing data and pilots into the same OFDM symbol. This allows having a very flexible structure without any scheduling restrictions and applicable to any sTTI size. Pilot position within the sTTI can be adapted to optimize channel estimation performance. Pilot position within the legacy frame becomes variable and there is no inter-cell pilot interference control. Nevertheless, this effect is rather limited because UEs employing sTTI will mostly be located near the cell center.

An option is to allow mixed data/pilot design in order to reduce pilot overhead for very short sTTI sizes, and/or to allow better time domain channel tracking for longer sTTIs. Data and pilots can be mixed before or after the DFT. In the first case, single carrier-like PAPR is conserved at the expense of channel estimation performance degradation. Interleaving data and pilot after the DFT may bring some PAPR increase. PAPR can be controlled by choosing appropriate pilot/data interleaving patterns [4]. For example, hybrid data/pilot symbol, i.e., half data - half RS symbol with interleaved subcarrier mapping, has good PAPR properties. Further decreasing the pilot density within an OFDM symbol leads to irregular subcarrier mapping of the data part, breaking the single-carrier property and further increasing the resulting PAPR. Methods for PAPR reduction when combining data and pilot in the same SC-FDMA symbol exist.

For sTTIs of 7 symbols, there are three alternatives: either current UL DM-RS is kept (Figure 3 a), or a hybrid pilot is used to increase throughput (Figure 3 b), or second UL-DM-RS is introduced (Figure 3 c) to improve performance in medium/high mobility with an overhead similar to current full pilot design. In the first case, OCC between the two UL DM-RS needs to be removed. The impact on channel estimation needs to be evaluated. In the third case, pilot position (symbols 1/7, 2/6, etc) needs to be investigated to optimize performance.

For shorter sTTIs (1, 2, 3, 4 symbols), a design is proposed in Figure 4.

| a) Full pilot | b) 1 hybrid pilot | c) Two hybrid pilots |
| --- | --- | --- |

Figure 3 – Self-contained DM-RS for sTTI of 7 symbols

Figure 4 - Self-contained DM-RS for very short sTTIs

Note that for solutions like the one in Figure 1a can be applied to the structure in Figure 4b whenever the scheduling constraints allow it in order to further decrease DM-RS overhead.

# Evaluation results

## PAPR evaluation of hybrid UL RS design

In order to evaluate the envelope variations of different signals, we use the CCDF of the instantaneous normalized power (INP). The CCDF of INP indicates the probability that the INP at a sample level exceeds a certain threshold γ2. In order to have a good approximation of the signal after digital to analogue conversion, the signal must be oversampled at least 4 times [7] before any evaluation of the envelope fluctuations.

Figure 5 presents the CCDF of INP for the ZC sequence (full pilot symbol), SC-FDMA (full data symbol), the hybrid symbol, a scattered symbol mixing 1 pilot subcarrier within data subcarriers every 3 subcarriers, and OFDMA.

The hybrid symbol exhibits limited PAPR. There is almost no PAPR increase with 64QAM. Methods of further decreasing the resulting PAPR exist.

The use of hybrid symbols enables the use of very short TTI (1-2 symbols) with non-prohibitive RS overhead. It avoids strong channel estimation degradation in mobility scenarios for longer sTTIs. We propose to investigate hybrid half data/half RS symbols for UL RS design for sTTI.

Channel estimation performance needs to be evaluated. The necessity of further reducing the PAPR of the hybrid symbol needs to be further evaluated.

Observation 2:

The hybrid symbol (1/2 pilots 1/2 data) presents limited PAPR and enables DM-RS overhead reduction.

Figure 5 – CCDF of the instantaneous normalized power for interleaved data/pilot patterns

## Performance evaluations

The simulation parameters considered are selected according to [8].  The carrier frequency is 2.0GHz, the system bandwidth is  20MHz with 50 PRBs allocated for a given UE, the transmission channel is the EPA channel with a normal CP length considering a UE speed of either 3kmph or 120 kmph. In addition, the antenna configuration is a SIMO 1x2 under the assumption of transmission mode 1. The Wiener filtering with linear time interpolation is used as a practical channel estimation coupled with an MMSE equalizer. Finally, the modulation and coding schemes are QPSK 1/3, 16QAM ¾ and 64QAM 5/6.

The following notations are used

| Legacy TTI | Legacy structure used as reference |
| --- | --- |
| sTTI 7, F.P. | Slot sTTI, full pilot in position 3, as in Figure 3 a |
| sTTI 7, 2 H.P. | Slot sTTI, 2 hybrid pilots in positions 1 and 7 as in Figure 3 c |
| sTTI 7, 1 H.P. | Slot sTTI with 1 hybrid pilot in position 4 as in Figure 3b |
| sTTI 4(3). | sTTI 4(3) with DM-RS multiplexing as in Figure 2, but equal allocated bands |
| sTTI 4, 2 H.P. | sTTI 4 with 2 hybrid pilots in positions 1 and 4 as in Figure 4 d |
| sTTI 4, 1 H.P. | sTTI 4 with 1 hybrid pilots in position 3 as in Figure 4 e |
| sTTI 3, F.P. | sTTI 3, full pilot in position 3 |
| sTTI 3, H.P. | sTTI 3, 1 hybrid pilot in position 3 as in Figure 4 c |
| sTTI 2, F.P. | sTTI 2, full pilot in position 1 |
| sTTI 2, H.P. | sTTI 2, hybrid pilot in position 1 as in Figure 4 b |

Performance is presented in tables giving the SNR and the throughput at 10% FER. Cases in grey were not simulated.

FER and throughput curves are given in the Annex.

### QPSK 1/3

UL sPUSCH performance with different pilot design and sTTI lengths is given in Table 1 for QPSK 1/3.

For sTTI lengths of size 2, using a hybrid pilots brings 50% throughput increase over using a full pilot at 10% target FER at the expense of SNR loss of less than 0.9dB on EPA and EVA channels, due to the less performing channel estimation based on a smaller number of pilot subcarriers. Loss on ETU channel is inferior to 1.5 dB and can be further reduced by using a longer Wiener filter (low complexity filtering was used).

For sTTI lengths of size 3, using a hybrid pilots brings 25% throughput increase over using a full pilot at 10% target FER at the expense of SNR loss of less than 0.9dB on EPA and EVA channels, and up to 1.5dB on ETU channel.

Table 1 - UL sPUSCH performance with different pilot design and sTTI lengths, QPSK 1/3

| QPSK 1/3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SNR/THP at 10% FER | EPA | EVA |  | ETU |  |
|  | 3 kmph | 60 kmph | 120 kmph | 60 kmph | 120 kmph |
| sTTI 14 | 1.12dB 4.32Mbps | 0.25dB 4.32Mbps | 0.21dB 4.32Mbps | 0.23dB 4.32Mbps | 0.20dB 4.32Mbps |
| sTTI 7, F.P. | 1.25dB 4.32Mbps | 0.51dB 4.32Mbps | 0.68dB 4.32Mbps | 0.51dB 4.32Mbps | 0.66dB 4.32Mbps |
| sTTI 7, 2 H.P. | 1.48dB 4.32Mbps | 0.77dB 4.32Mbps | 0.78dB 4.32Mbps | 1.33dB 4.32Mbps | 1.32dB 4.32Mbps |
| sTTI 3, F.P. | 1.23dB 3.36Mbps | 0.47dB 3.36Mbps | 0.49dB 3.36Mbps | 0.48dB 3.36Mbps | 0.52dB 3.36Mbps |
| sTTI 3, H.P. | 1.90dB 4.20Mbps | 1.32dB 4.20Mbps | 1.35dB 4.20Mbps | 2.04dB 4.20Mbps | 2.07dB 4.20Mbps |
| sTTI 2, F.P. | 1.28dB 2.52Mbps | 0.57dB 2.52Mbps | 0.59dB 2.52Mbps | 0.61dB 2.52Mbps | 0.64dB 2.52Mbps |
| sTTI 2, H.P. | 1.93dB 3.78Mbps | 1.39dB 3.78Mbps | 1.42dB 3.78Mbps | 2.08dB 3.78Mbps | 2.10dB 3.78Mbps |

### 16QAM 3/4

UL sPUSCH performance with different pilot design and sTTI lengths is given in Table 1 for 16QAM ¾.

For sTTI lengths of size 2, using a hybrid pilots brings 50% throughput increase over using a full pilot at 10% target FER at the expense of SNR loss of less than 0.4dB on EPA and EVA channels, due to the less performing channel estimation based on a smaller number of pilot subcarriers. Loss on ETU channel is inferior to 1 dB and can be further reduced by using a longer Wiener filter (low complexity filtering was used).

For sTTI lengths of size 3, using a hybrid pilots brings 25% throughput increase over using a full pilot at 10% target FER at the expense of SNR loss of less than 0.4dB on EPA and EVA channels, and up to 1dB on ETU channel.

For sTTI lengths of size 4, pilot sharing with sTTI 3(4) and sTTI of size 4 with hybrid symbol have similar performance on EPA and EVA channels. Inserting a second hybrid pilot improves SNR performance of more than 2 dB at high speed but with a throughput penalty of 14%.

For sTTI lengths of size 7, a hybrid pilots brings 8.7% throughput increase over using a full pilot at 10% target FER at the expense of SNR loss of less than 0.4dB on EPA and EVA channels, and up to 1dB on ETU channel. For sTTI lengths of size 7, using 2 hybrid pilots (and thus the same pilot overhead as full pilot design) brings a SNR gain of 2 to 3 dBs in high mobility. Performance is equivalent for low/medium mobility.

Table 2 – UL sPUSCH performance with different pilot design and sTTI lengths, 16QAM ¾

| 16QAM 3/4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SNR (dB)/ THP at 10% FER | EPA | EVA |  | ETU |  |
|  | 3 kmph | 60 kmph | 120 kmph | 60 kmph | 120 kmph |
| Legacy TTI | 13.43 dB 19.4 Mbps | 13.65 dB 19.4 Mbps | 17.08 dB 19.4 Mbps | 13.29 dB 19.4 Mbps | 15.98 dB 19.4 Mbps |
| sTTI 7, F.P. | 13.36 dB 19.4 Mbps | 13.33 dB 19.4 Mbps | 16.10 dB 19.4 Mbps | 13.05 dB 19.4 Mbps | 15.41 dB 19.4 Mbps |
| sTTI 7, 2 H.P. | 13.4 dB 19.4 Mbps | 12.93 dB 19.4 Mbps | 13.09 dB 19.4 Mbps | 13.15 dB 19.4 Mbps | 13.24 dB 19.4 Mbps |
| sTTI 7, 1 H.P. | 13.4 dB 21.1 Mbps | 13.83 dB 21.1 Mbps | 16.54 dB 21.1 Mbps | 14.16 dB 21.1 Mbps | 16.36 dB 21.1 Mbps |
| sTTI 4(3). | 13.39 dB 19.4 Mbps | 13.14 dB 19.4 Mbps | 15.67 dB 19.4 Mbps |  |  |
| sTTI 4, 2 H.P. | 13.1 dB 17.0 Mbps | 12.84 dB 17.0 Mbps | 12.82 dB 17.0 Mbps |  |  |
| sTTI 4, 1 H.P. | 13.75 dB 19.8 Mbps | 13.50 dB 19.8 Mbps | 14.41 dB 19.8 Mbps | 13.92 dB 19.8 Mbps | 14.76 dB 19.8 Mbps |
| sTTI 3, F.P. | 13.28 dB 15.1 Mbps | 12.64 dB 15.1 Mbps | 12.95 dB 15.1 Mbps | 12.52 dB 15.1 Mbps | 12.80 dB 15.1 Mbps |
| sTTI 3, H.P. | 13.58 dB 18.9 Mbps | 13.10 dB 18.9 Mbps | 13.36 dB 18.9 Mbps | 13.58 dB 18.9 Mbps | 13.80 dB 18.9 Mbps |
| sTTI 2, F.P. | 13.24 dB 11.3 Mbps | 12.68 dB 11.3 Mbps | 12.94 dB 11.3 Mbps | 12.59 dB 11.3 Mbps | 12.83 dB 11.3 Mbps |
| sTTI 2, H.P. | 13.63 dB 17.0 Mbps | 13.11 dB 17.0 Mbps | 13.36 dB 17.0 Mbps | 13.66 dB 17.0 Mbps | 13.82 dB 17.0 Mbps |

### 64QAM

UL sPUSCH performance with different pilot design and sTTI lengths is given in Table 3 for 64QAM 5/6.

With single pilot per TTI, 64 QAM never reaches FER 10% at high mobility for sTTI sizes of length 4 and 7 on both EVA and ETU channels. Inserting two hybrid pilots enables transmission with sufficient performance at high speed with the same pilot overhead as full pilot design, for both sTTI sizes of length 4 and 7.

For sTTI lengths of size 2, using a hybrid pilots brings 49% throughput increase over using a full pilot at 10% target FER at the expense of SNR loss of less than 0.4dB on EPA and EVA channels, and up to 1dB on ETU channel.

For sTTI lengths of size 3, using a hybrid pilots brings 25% throughput increase over using a full pilot at 10% target FER at the expense of SNR loss of less than 0.5dB on EPA and EVA channels, and around 1dB on ETU channel.

For sTTI lengths of size 4, using a hybrid pilots brings 16.5% throughput increase over using a full pilot at 10% target FER and at medium mobility at the expense of SNR loss of less than 0.5dB on EPA and EVA channels, and around 1dB on ETU channel.

For sTTI lengths of size 7, inserting two hybrid pilots allows improving SNR at FER 10% by at least 6dB at medium speed with respect to full pilot design having the same pilot overhead.

Table 3 - UL sPUSCH performance with different pilot design and sTTI lengths, 64QAM 5/6

| 64QAM 5/6 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SNR/ THP at 10% FER | EPA | EVA |  | ETU |  |
|  | 3 kmph | 60 kmph | 120 kmph | 60 kmph | 120 kmph |
| Legacy TTI | 20.43dB 32.4Mbps | 31.96dB 32.4Mbps | ∞ dB 0 Mbps | 28.21dB 32.4Mbps | ∞ dB 0 Mbps |
| sTTI 7, F.P. | 20.39dB 32.4Mbps | 28.18dB 32.4Mbps | ∞ dB 0 Mbps | 26.25dB 32.4Mbps | ∞ dB 0 Mbps |
| sTTI 7, 2 H.P. | 20.52dB 32.4Mbps | 20.27dB 32.4Mbps | 20.50dB 32.4Mbps | 20.52dB 32.4Mbps | 20.71dB 32.4Mbps |
| sTTI 4, F.P. |  | 21.54dB 28.4Mbps | ∞ dB 0 Mbps | 21.27dB 28.4Mbps | ∞ dB 0 Mbps |
| sTTI 4, 1 H.P. |  | 21.78dB 33.1Mbps | ∞ dB 0 Mbps | 22.19dB 33.1Mbps | ∞ dB 0 Mbps |
| sTTI 4, 2 H.P. | 20.25 dB 27.88 Mbps |  |  |  | 19.75 dB 26.6Mbps |
| sTTI 3, F.P. | 20.21dB 25.2Mbps | 20.19dB 25.2Mbps | 22.27dB 25.2Mbps | 20.02dB 25.2Mbps | 21.88dB 25.2Mbps |
| sTTI 3, H.P. | 20.67dB 31.5Mbps | 20.75dB 31.5Mbps | 22.55dB 31.5Mbps | 21.22dB 31.5Mbps | 22.80dB 31.5Mbps |
| sTTI 2, F.P. | 20.19dB 18.9Mbps | 20.16dB 18.9Mbps | 22.24dB 18.9Mbps | 19.99dB 18.9Mbps | 21.85dB 18.9Mbps |
| sTTI 2, H.P. | 20.55dB 28.3Mbps | 20.51dB 28.3Mbps | 22.77dB 28.3Mbps | 20.97dB 28.3Mbps | 22.12dB 28.3Mbps |

# Conclusions

Observation 1: Channel estimation performance, pilot overhead and PAPR need to be taken into account in UL DM-RS design for sPUSCH.

Observation 2: The hybrid symbol (1/2 pilots 1/2 data) presents limited PAPR and enables DM-RS overhead reduction.

Observation 3: UL DM-RS design with one hybrid symbol (1/2 pilots 1/2 data) provides significant throughput increase over full pilot design (from 8.7% to 50%) for all MCS at target FER 10%.

Observation 4: UL DM-RS design with one hybrid symbol (1/2 pilots 1/2 data) provides significant throughput increase at the expense of SNR loss of less than 0.4dB on EPA and EVA channels for 16QAM and 64QAM at target FER 10%.

Observation 5: UL DM-RS design with one hybrid symbol (1/2 pilots 1/2 data) for sTTI of length 4 has similar performance with UL DM-RS design with shared pilots sTTI 3(4) with 16QAM ¾ on EPA and EVA channels.

Observation 6: UL DM-RS design with one single full pilot for sTTI of size 4 or 7 symbols does not offer support for UEs with high mobility and high modulation orders.

Observation 7: UL DM-RS design with two hybrid symbols (1/2 pilots 1/2 data) provides sufficient performance for sTTI of size 4 or 7 symbols.

Observation 8: UL DM-RS design with two hybrid symbols (1/2 pilots 1/2 data) provides significant SNR gain at target FER 10% (2-8dBs) for 16QAM at medium/high mobility and 64QAM at medium mobility for sTTI 7.

# References

- RP-150456	New SI proposal: Study on Latency reduction techniques for LTE, 3GPP TSG RAN#67, Shanghai, China, March 9-12, 2015.

- 3GPP TR 36.211 v13.0.0 (2015-12), “Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation (Release 13)”

- R1-162542, “Design of UL RS for PUSCH with sTTI,” Mitsubishi Electric, RAN1#84b, Busan.

- R1- 160699, “Design of UL reference signal for PUSCH with sTTI,” Mitsubishi Electric, RAN1#84, Malta.

- R1-163316, “Link evaluation for PUSCH for short TTI,” Ericsson, RAN1#84b, Busan.

- R1-160965, “UL aspects of TTI shortening”, NTT DOCOMO, Malta, Feb. 2016.

- J. Tellado, Multicarrier Modulation with Low Peak to Average Power Applications to xDSL and Broadband Wireless, Boston/Dordrecht/London: Kluwer Academic Publishers, 2000.

- R1-157806, WF on evaluation methodology for latency reduction, RAN1#83.

# Annex