3GPP TSG RAN WG1 Meeting #85                                                               	     R1-164185

Nanjing, China, 23-27 May 2016

Agenda item:	7.1.5.1

Source: 	Intel Corporation

Title: 	Polar code constructions for evaluations

Document for:	Discussion

- Introduction

In this contribution, we provide polar code constructions for performance evaluation.

Constructing a polar code is equivalent to selecting a set of bit-channel indices to place the data bits.  The exact computation of bit-channel statistics may be an exponentially difficult problem, but several approximate calculation methods have been developed.  In [1] Monte Carlo based techniques were proposed to estimate bit-channel qualities.  [2] proposed a method to approximate the bit-channels by quantized versions in order to keep the complexity bounded.  Gaussian approximation, familiar from the analysis of LDPC codes, has also been proposed to identify the bit-channel qualities [3].

- Design features

Our code constructions are given in the attached text file (intelPC1.txt). They are for codeword lengths {64, 128, 256, 512, 1024, 2048, 4096, 8192}.Performance results are in companion contribution [8].

The codes were designed based on the quantization technique given in [4], but with slightly modified choice of quantization bins. Some features of the codes constructed are as follows:

- Nested structure:  A single permutation vector of length N is used to represent all codes of codeword length N and information block length-K (K <N).  The permutations were designed considering the performance at arbitrary code rates (e.g. fixed N and variable K).

- CRC-less design: The proposed code family does not require CRC to improve error performance, especially for the native codeword length (2n). As shown in companion contribution [5], the performance with CRC-less list decoding is often competitive with known constructions, e.g. [2], with List+CRC decoding.  Nevertheless, CRC can still be used in decoding if so desired, in which case the information block size will be K-24, if number of CRC bits is 24.

- Flexible Codeword Size: Very fine code rate support for a given codeword length is a key property of polar codes. However, these codes’ native codeword lengths are limited to 2n. As discussed in companion contribution [5], shortening can provide flexibility in codeword length (i.e. to support N that is not a power of 2).

- Polar code constructions

Each construction is represented by a permutation vector PN of length equal to the codeword length N.  The polar code of all rates at codeword length N can be obtained from PN.  A polar code of rate K/N is generated from PN by placing the K data bits in to the positions indicated by the first K entries of PN and setting the remaining positions to 0 (frozen bits)

Example 1: See Figure 1.  Suppose that for length-8 codes, P8=8 7 6 4 5 3 2 1.

- If the desired code rate is 3/8, then the data bits b1,b2, b3 are placed in the 8th, 7th, and 6th bit positions, and the remaining 5 positions are frozen (i.e. to binary 0).  That is, U8=b1, U7=b2, U6=b3 and U1=…=U5=0.  Then X18 is computed from these and transmitted.

- If the desired code rate is 4/8, then data bits b1,b2, b3,b4 are placed in the 8th, 7th, 6th, and 4th bit positions, and the remaining 4 positions are frozen. That is, U8=b1, U7=b2, U6=b3 , U4=b4  and U1=U2=U3=U5=0.  Then X18 is computed from these and transmitted.

- (shortening) If the desired code rate is 5/6 at (codeword length=6), then the last two bits are set to zero, U7=U8=0, and the 5 data bits are placed in the first five remaining positions of P8, i.e., U6=b1,U4=b2,U5=b3,U3=b2,U2=b_1, and U1=0 (frozen).

X1

X2

X3

X4

X5

X6

X7

X8

U1

U2

U3

U4

U5

U6

U7

U8

Figure 1. An (8,K) Polar code. LHS is input bits (K  bits carry data, 8-K are frozen,), and RHS is codeword.

- Conclusion

We present polar code designs for performance evaluations in this document. Some main features of the designs include a nested structure for support different input block size with a single construction, facilitates CRC-less List decoding (or CRC-based List decoding), and flexibility in codeword size that may not be natively supported by polar constructions.

References

[1] 	E. Arikan, "Channel polarization: a method for constructing capacity-achieving codes for symmetric binary-input memoryless channels," IEEE Transactions on Information Theory, 2009.

[2] 	I. Tal and A. Vardy, "How to construct polar codes," IEEE Trans. on Information Theory, 2013.

[3] 	P. Trifonov, "Efficient design and analysis of polar codes," IEEE Trans. on Communications, 2012.

[4] 	E. Sasoglu, Polarization and Polar Codes, now Publishers, 2012.

[5] 	R1-164184 Polar code design for NR, Intel Corporation, RAN1 #85

[6] 	R1-164182 Comparison of Coding Schemes for NR, Intel Corporation,"RAN1#85

[7] 	I. Tal and A. Vardy, "List decoding of polar codes," IEEE Trans. on Information Theory, 2013.

[8] 	R1-164186 Performance evaluation of coding schemes for NR, Intel Corporation," RAN1#85