3GPP TSG RAN WG1 Meeting #85		                                                              R1-164211

Nanjing, China, 23rd - 27th May 2016

Source:	CATT

Title:	Discussion on synchronization issues of physical design

Agenda Item:	6.2.2.7.2

Document for:	Discussion and Decision

Introduction

In RAN1 #83 meeting, the following agreements were achieved [1]:

- Priority of synchronization source includes at least transmission timing reference.

- FFS whether there is any differentiation depending on whether eNB is synchronized to GNSS in the corresponding SLSS transmissions

- SLSS transmitted from out-coverage UE directly synchronized with GNSS or GNSS equivalent with sufficient reliability is differentiated from SLSS_net with in coverage indicator 1

And in R1-157764 [2], the following agreements and working assumption are listed:

- At least reuse Priority order SLSS_net with in coverage indicator 1, SLSS_net with in coverage indicator 0, SLSS_oon

  - FFS: any new priorities can be defined if benefits are shown

  - FFS: GNSS or GNSS equivalent Priority

- Working assumption: Priority of SLSS transmitted from in-coverage UE directly synchronized with GNSS or GNSS equivalent with sufficient reliability is the same as that of  SLSS_net with in coverage indicator 1

In WID RP-152293 [3], the detailed objectives of V2X synchronization are as follows:

- To specify enhancement to sidelink synchronization procedure necessary for V2V services [RAN1, RAN2]

  - Low priority is given to enhancements to Rel-12/13 SLSS-based synchronization.

In RAN1 #84 meeting, following agreements were achieved [4][5]：

Working assumption:

- No new synchronization signal sequence is defined in V2V

The enhancements to physical layer design for synchronization in PC5-based V2V were presented in [6]. The modified synchronization procedure and mechanisms are presented in [7]. As a companion contribution, considerations on SLSS and PSBCH transmission of UE are discussed in this one. We present how to reuse the SLSS and PSBCH in GNSS or GNSS-equivalent based and eNB based synchronization for V2V communication respectively.

Discussion

  - SLSS and in coverage

In [7], the priority order of synchronization source priorities related to PC5-based V2V UEs is proposed as follows:

- Priority 1:

- eNB-based synchronization: in-coverage UEs synchronize with eNB;

- GNSS-based synchronization: in-coverage UEs synchronize with GNSS directly;

- Priority 2:

- out-of-coverage UEs synchronize with GNSS directly;

- GNSS-based synchronization: partial-coverage UEs synchronize with GNSS directly;

- Priority 3:

- out-of-coverage UEs synchronize with GNSS indirectly;

- GNSS-based synchronization: partial-coverage UEs synchronize with GNSS indirectly;

- eNB-based synchronization: partial-coverage UEs;

- Independent synchronization source.

Because V2V and D2D can be distinguished by different carriers [8], reusing R12 D2D scheme, the synchronization source priorities of PC5-based V2V can be distinguished with the combination of SLSSIDs and in-coverage indicator in GNSS-based synchronization:

- The fixed value X1 is selected from id_net for Priority 1/2 UEs can synchronize with GNSS directly in GNSS-based synchronization. Because the coverage state may be in-coverage, partial-coverage or out-of-coverage, reusing R12 D2D, in coverage indicator is set as TRUE for in-coverage and in coverage indicator is set as FALSE for partial-coverage and out-of-coverage;

- The detailed design of SLSSIDs for UE Priority 3 is proposed as follows:

  - If Priority 3 UE selects Priority 1/2 UE synchronize with GNSS directly in GNSS-based synchronization as SyncRef UE, the coverage state of Priority 3 UE may be in-coverage, partial-coverage or out-of-coverage. In order to simplify the process, in coverage indicator is set as FALSE for all coverage states. Reusing partial-coverage SLSS process in R12 D2D, SLSSID is set as X1 plus 168.

  - If Priority 3 UE synchronizes with a) Priority 3UE, reusing that SyncRef UE is partial-coverage SLSS process in R12 D2D, SLSSID is same as the selected SyncRef UE.

  - If Priority 3 UE synchronize with b) or c) Priority 3 UE, reusing that SyncRef UE is partial-coverage SLSS process in R12 D2D, SLSSID is same as the selected SyncRef UE.

  - Independent synchronization source: reusing R12 D2D, UE randomly selects an SLSSID from id_oon sequences using a uniform distribution. The SLSSID is in id_oon sequence.

The selection of SLSSID in eNB-based synchronization is reusing R12 D2D scheme. SLSSID is selected from id_net sequences except X1 or X2 configured by system information in coverage. SLSSID of partial-coverage UE is set as SLSSID of SyncRef UE (Priority 1 UE) plus 168.

In summary, the detailed design of SLSSIDs for UE in PC5-based V2V is proposed as follows:

- Priority 1：

- GNSS-based synchronization: SLSSID is fixed value X1 selected from id_net sequences, and in coverage is set as TRUE;

- eNB-based synchronization: reusing R12 D2D, SLSSID is selected from id_net sequences except X1, and in coverage is set as TRUE;

- Priority 2 ：SLSSID is fixed value X1 selected from id_net sequences, and in coverage is set as FALSE;

- Priority 3 ：

  - GNSS-based synchronization: If the SyncRef UE is Priority 1/2 UE, SLSSID is selected from id_oon sequences with X1 plus 168, and in coverage is set as FALSE.

  - GNSS-based synchronization: If the SyncRef UE is Priority 3 UE, SLSSID is same as the selected SyncRef UE.

  - eNB-based synchronization: SLSSID of Partial-coverage UE is set as SLSSID of synchronization reference UE (Priority 1 UE) plus 168.

  - Independent synchronization source: reusing R12 D2D, UE randomly selects an SLSSID from id_oon sequences using a uniform distribution. The SLSSID is in id_oon sequence except X1.

Proposal 1: the detailed design of SLSSIDs for UE in PC5-based V2V is proposed as follows:

- Priority 1：

- GNSS-based synchronization: SLSSID is fixed value X1 selected from id_net sequences, and in coverage is set as TRUE;

- eNB-based synchronization: reusing R12 D2D, SLSSID is selected from id_net sequences except X1, and in coverage is set as TRUE;

- Priority 2 ：SLSSID is fixed value X1 selected from id_net sequences, and in coverage is set as FALSE;

- Priority 3 ：

  - GNSS-based synchronization: If the SyncRef UE is Priority 1/2 UE, SLSSID is selected from id_oon sequences with X1 plus 168, and in coverage is set as FALSE.

  - GNSS-based synchronization: If the SyncRef UE is Priority 3 UE, SLSSID is same as the selected SyncRef UE.

  - eNB-based synchronization: SLSSID of Partial-coverage UE is set as SLSSID of synchronization reference UE (Priority 1 UE) plus 168.

  - Independent synchronization source: reusing R12 D2D, UE randomly selects an SLSSID from id_oon sequences using a uniform distribution. The SLSSID is in id_oon sequence except X1.

Figure 1 shows an example of the combination of SLSSID and in-coverage indication for different synchronization source priorities:

Figure 1 Example of SLSSID and in-coverage indicator in PC5-based V2V synchronization

Conclusion

Proposal 1: the detailed design of SLSSIDs for UE in PC5-based V2V is proposed as follows:

- Priority 1：

- GNSS-based synchronization: SLSSID is fixed value X1 selected from id_net sequences, and in coverage is set as TRUE;

- eNB-based synchronization: reusing R12 D2D, SLSSID is selected from id_net sequences except X1, and in coverage is set as TRUE;

- Priority 2 ：SLSSID is fixed value X1 selected from id_net sequences, and in coverage is set as FALSE;

- Priority 3 ：

  - GNSS-based synchronization: If the SyncRef UE is Priority 1/2 UE, SLSSID is selected from id_oon sequences with X1 plus 168, and in coverage is set as FALSE.

  - GNSS-based synchronization: If the SyncRef UE is Priority 3 UE, SLSSID is same as the selected SyncRef UE.

  - eNB-based synchronization: SLSSID of Partial-coverage UE is set as SLSSID of synchronization reference UE (Priority 1 UE) plus 168.

  - Independent synchronization source: reusing R12 D2D, UE randomly selects an SLSSID from id_oon sequences using a uniform distribution. The SLSSID is in id_oon sequence except X1.

References

  - Draft Minutes 3GPP TSG RAN WG1 Meeting #83, RAN1 Chairman’s Notes V0.3.0;

  - R1-157764, “WF on Synchronization source Priority”, CATT, LGE, NTT DOCOMO

  - RP-152293, “New WI proposal: Support for V2V services based on LTE sidelink”, LG Electronics, Huawei, HiSilicon, CATT, CATR

  - Draft Minutes 3GPP TSG RAN WG1 Meeting #84, RAN1 Chairman’s Notes V0.2.5;

  - R1-161288, “Way Forward on SLSS for V2V PC5”, Huawei, HiSilicon, CATT, CATR, ITRI, Potevio;

  - R1-162277, “Discussion on physical layer design for synchronization in PC5-based V2V”, CATT, 3GPP RAN1#84bis

  - R1-164210, “Synchronization enhancements in PC5-based V2V”, CATT, 3GPP RAN1#85

  - R1-164212, “Discussion on PSBCH”, CATT, 3GPP RAN1#85