3GPP TSG RAN WG1 Meeting #85		                                                       R1- 164212

Nanjing, China, 23rd - 27th May 2016

Source:	CATT

Title:	Discussion on PSBCH

Agenda Item:	6.2.2.7.2

Document for:	Discussion and Decision

Introduction

In RAN1 #83 meeting, following agreements were achieved [1]:

- SLSS and PSBCH transmission of UE is supported for PC5 based V2V.

- UE capability of SLSS transmission will be discussed later.

- Rel. 12/13 physical format of SLSS/PSBCH is the starting point.

- FFS number and location of PSBCH DM RS

- FFS PSSS root index, SLSS ID

- Rel. 12/13 sync procedure (e.g., sync reference priority) is the starting point.

- FFS PSBCH contents

And in RAN1 #84 meeting, following agreements were achieved [2]:

- Sensing with semi-persistent transmission is supported

  - UE transmits PSSCH (when data is available) on a selected set of periodically occurring resources until a resource reselection occurs

- Working assumption:

  - No new synchronization signal sequence is defined in V2V

The modified synchronization procedure and mechanisms are presented in [3]. As a companion contribution, considerations on the content of PSBCH and transmission of PSBCH are discussed in this one. We present how to utilize PSBCH in GNSS or GNSS-equivalent based and eNB based synchronization for V2V communication to transmit the timing information and configuration parameters.

Discussion

  - Background

In R12 D2D, PSBCH includes the following 40 bits four types of content [4]:

- System configuration: system bandwidth is 3 bits and TDD-config is 3 bits;

- Timing configuration: DFN is 10 bits and DFN subframe is 4 bits;

- In coverage: 1 bit;

- Reserved: 19 bits;

It is required for R12 D2D communications that SLSS and PSBCH are transmitting simultaneously. The receiving UEs can obtain the system configuration and timing configuration by PSBCH, and can distinguish the synchronization source priorities with the combination of SLSS id and in coverage indication in PSBCH.

A UE shall transmit PSBCH and SLSS id when the following conditions are met [4]:

Table 1 Scenarios of UE shall transmit PSBCH and SLSS id

| Coverage State | RRC connection state | Network control flag networkControlledSyncTx | Transmitting threshold | RSRP measurement |  |
| --- | --- | --- | --- | --- | --- |
| In coverage | RRC_CONNECTED | on | none |  | none |
|  |  | off | syncTxThreshIC in SIB18 |  | RSRP measurement of the cell chosen for sidelink communication transmission is below the value of syncTxThreshIC |
|  | RRC_IDLE | none |  |  |  |
| Out of coverage | none | none | syncTxThreshOoC in SL-Preconfiguration |  | S-RSRP measurement result of the selected SyncRef UE is below the value of syncTxThreshOoC |
|  |  |  |  |  | UE has no selected SyncRef UE |

From above table, there are four possible PSBCH transmission scenarios:

- In coverage and RRC_CONNECTED state, UEs transmit continuously.

- In coverage and RRC_CONNECTED state, UEs are at the cell edge.

- In coverage and RRC_IDLE state, UEs are at the cell edge.

- Out of coverage, UE is independent synchronization source.

Through the transmission of synchronization information, the synchronization range can be extended and the number of synchronization clusters will be reduced.

  - PSBCH content

Because UEs synchronize with GNSS directly can be synchronization source, the synchronization priorities of V2V is different from D2D:

- In-coverage UEs will synchronize with GNSS directly or indirectly in GNSS-based synchronization without consideration of synchronizing with eNB; while in-coverage UEs in R12 D2D only synchronize with eNB;

- The priority of out-of-coverage UEs synchronizing with GNSS directly is highest of out-of-coverage UEs; while out-of-coverage UEs are same priority in R12 D2D .

Because GNSS is introduced as synchronization source, the process of synchronization source priorities is different for in-coverage, and out-of-coverage from R12 D2D. Because no new sequence is added, V2V and D2D can be distinguished with different carriers.

Proposal 1: Because GNSS is introduced as synchronization source, different carriers are used to distinguish V2V and D2D.

From different carriers of V2V and D2D, UEs can determine the current synchronization information from V2V or D2D UEs. It is unnecessary to indicate V2V or D2D in PSBCH.

Proposal 2: It is unnecessary to indicate V2V or D2D in PSBCH.

Because eNB instruct eNB-based and GNSS-based synchronization, the combination of SLSSIDs and in coverage indicator can be utilized to distinguish the synchronization type of eNB-based or GNSS-based [5]. Thus, it is unnecessary to indicate the synchronization type is eNB-based or GNSS-based in PSBCH。

Proposal 3: It is unnecessary to indicate the synchronization type is eNB-based or GNSS-based in PSBCH.

Reusing R12 D2D, with the combination of SLSS id and in coverage indicator, the synchronization source priorities can be distinguished [5].  It is unnecessary to indicate the synchronization information is from GNSS in PSBCH.

Proposal 4: It is unnecessary to indicate the synchronization information is from GNSS in PSBCH；

Because a separate VFN (PC5 V2V Frame Number) is suggested to be used for PC5 V2V [6], VFN should be included in PSBCH.

Proposal 5: It is suggested that VFN is included in PSBCH for PC5 V2V；

  - PSBCH period

The synchronization period is 40ms and typical service is VoIP in R12 D2D, but the typical services of V2V is periodic broadcast messages and event-triggered messages with 100ms.

If synchronization period is still 40ms, the following problems are listed:

- Overhead is too high (2.5%~5%) to be efficient resource utilization.

- With stronger computing, synchronization period can be configured enough long to satisfying the requirement of reliability of V2V communications [7].

- If the synchronization period is set as 40ms, five synchronization periods is equal to two service period. In other word, 200ms the synchronization subframe will collide with service subframe. With the agreements of RAN1#84 meeting, resouce allocation of SPS+sensing will increase the possibility of collision and resource reselection. The system performance of resource utilization and service reliability will be deteriorated.

It is suggested that synchronization period of V2V is not reusing R12 D2D as 40ms but as N*100ms (N>=1). Because synchronization subframe is not considered in resource allocation of services, the collision can be avoided efficiently. With N*100ms, the synchronization period of V2V can be multiple synchronization period of D2D, and the compatibility of synchronization period with V2V and D2D can be achieved.

Based on the simulation assumption and the performance of possible DMRS location in [8], the performance of PSBCH is presented in Figure 1.

| - SNR= -1 | - SNR=0 |
| --- | --- |

| - SNR=2 | - SNR=4 |
| --- | --- |

Figure 1 Link-level performance of V2X synch signal

At 0 dB SNR (Figure 1 b)), within two synch periods, PSBCH with 3 DMRS #3/6/9 of speed 280kmh is decoded with probability 0.9875. The performance of PSBCH with 3 DMRS #3/6/9 of higher speed 500kmh is little worse with probability 0.9669. The performance of PSBCH with 2 DMRS #3/10 is worst with probability 0.5774. The performance of PSBCH is improved with increasing SNR (SNR >= 0) with the same trend. But at -1 dB SNR (Figure 1 a)), within two synch periods, PSBCH with 3 DMRS #3/6/9 of speed 500kmh is decoded with probability 0.9387 which is lower than probability 0.95.

Observation 1: The performance of PSBCH with within two synch periods can guarantee the reliability of PSBCH of 95% with SNR>=0 and 3 DMRS #3/6/9 with 280kmh or 500kmh.

Proposal 6：Synchronization period of V2V is N*100ms (N>=1).

Conclusion

Proposal 1: Because GNSS is introduced as synchronization source, different carriers are used to distinguish V2V and D2D.

Proposal 2: It is unnecessary to indicate V2V or D2D in PSBCH.

Proposal 3: It is unnecessary to indicate the synchronization type is eNB-based or GNSS-based in PSBCH.

Proposal 4: It is unnecessary to indicate the synchronization information is from GNSS in PSBCH；

Proposal 5: It is suggested that VFN is included in PSBCH for PC5 V2V；

Observation 1: The performance of PSBCH with within two synch periods can guarantee the reliability of PSBCH of 95% with SNR>0 and 3 DMRS #3/6/9 with 280kmh or 500kmh.

Proposal 6：Synchronization period of V2V is N*100ms (N>=1).

References

  - Draft Minutes 3GPP TSG RAN WG1 Meeting #83, RAN1 Chairman’s Notes V0.3.0;

  - Draft Minutes 3GPP TSG RAN WG1 Meeting #84, RAN1 Chairman’s Notes V0.2.5;

  - R1-164210,  “Synchronization enhancements in PC5-based V2V”, CATT, 3GPP RAN1 #85;

  - 3GPP TS 36.331 V13.0.0 (2016-01) "Radio Resource Control (RRC); Protocol specification"

  - R1-164211, “Discussion on physical layer design for synchronization in PC5-based V2V”, CATT, 3GPP RAN1 #85;

  - R1-164213, “Timing alignment for V2V”, CATT, 3GPP RAN1 #85;

  - R1-161072, “Distributed Synchronization Procedure for V2X over PC5”, Ericsson;

  - R1-162266, “DMRS enhancement for PSBCH”, CATT, 3GPP RAN1 #84bis;