3GPP TSG RAN WG1 Meeting #85	     R1-164266

Nanjing, P.R.China, 23rd - 27th May 2016

# Source: 	ZTE Corp, ZTE Microelectronics

# Title:	Simulation results of new waveform in case 1a and 1b

# Agenda Item:	7.1.3.1

# Document for:	Discussion

# Introduction

A study item on Next Generation New Radio Access Technology was agreed in RAN Plenary #71 meeting [1]. In this SID, waveform design based on OFDM is one of fundamental areas that need to focus on. In RAN1#84bis meeting, the evaluation assumption was discussed and some agreements were made in [2][3] plus the following agreements reached in follow-up email discussion:

Proposal 1 on MIMO mode: Companies should bring results for SISO at RAN1#85. 
Proposal 2 on Channel Model:

ETU/EPA/EVA are optional (as already agreed at RAN1#84bis) 
 Mobility: 3km/h or 30 km/h or 120 km/h, higher speed is not precluded

This contribution gives the simulation results and comparisons between CP-OFDM, CP-OFDM with WOLA and FB-OFDM in case 1a and case 1b.

# Evaluation case

The case 1a and case 1b in [2] are briefly described as following:

  - Case 1a, 1b: single numerology case

    - 1a (as in Fig. 1): Downlink

    - 1b (as in Fig. 2): Uplink, only one UE with narrow bandwidth is located at the edge of wide frequency band. It is assumed that no wide-band filter upon the whole frequency band.

In both Fig 1 and Fig 2, Gsys is the system guard band and depends on the OOB of new waveform.

In the simulation, the numerology of CP-OFDM is the same as in LTE system, and FB-OFDM is cut and windowed as shown in Fig. 12 in Appendix 1. Without loss of generality, in CP-OFDM with WOLA (W-OFDM), the shape of the edge in time domain is raised cosine window with length of 1/2 CP length. More simulation parameters are listed in Appendix 2.

Fig.1 Case 1a: Downlink single numerology case

Fig. 2 Case 1b: Uplink single numerology case

# PAPR

Fig. 3 shows the comparison of the baseband PAPR's among CP-OFDM, CP-OFDM with WOLA and FB-OFDM without clipping.

Fig.3 PAPR of CP-OFDM, W-OFDM(CP-OFDM with WOLA) and FB-OFDM without clipping

Observation 1: PAPR performance of CP-OFDM, W-OFDM and FB-OFDM are very close to each other.

# PSD

Fig. 4 shows the comparison of PSD among CP-OFDM, CP-OFDM with WOLA and FB-OFDM with/without clipping, given the data transmission bandwidth equal to 50 RBs.

Fig.4(a) PSD of CP-OFDM, W-OFDM(CP-OFDM with WOLA) and FB-OFDM with/without clipping(50RBs)

Fig.4(b) Amplified figure region in Fig.4(a)

In order to reach <-35dB OOB power leakage for data transmission bandwidth of 50 RBs in case 1a, the system guard band Gsys is 1MHz for CP-OFDM, 0.610 MHz CP-OFDM with WOLA or 0.09MHz for FB-OFDM.

Fig. 5 shows the comparison of PSD among CP-OFDM, CP-OFDM with WOLA and FB-OFDM with/without clipping in transmitter, given the data transmission bandwidth equal to 1 RBs.

Fig.5(a) PSD of CP-OFDM, W-OFDM(CP-OFDM with WOLA) and FB-OFDM with/without clipping(1RB)

Fig.5(b) Amplified figure region in Fig.5(a)

In order to reach <-35dB OOB power leakage for data transmission bandwidth of 1 RBs in case 1b, the system guard band Gsys is 0.5MHz (≈33 subcarriers) for CP-OFDM, 0.28MHz (≈18 subcarriers) for CP-OFDM with WOLA or 0.042MHz (≈3 subcarriers) for FB-OFDM.

Observation 2: The OOB energy of FB-OFDM falls much more rapidly than CP-OFDM and CP-OFDM with WOLA, which results in narrower system guard band and less guard-tones between subbands than the other two waveforms.

# Spectrum efficiency formula

The spectrum efficiency is calculated as [2], where  denotes the number of correctly received bits by target user, T is the transmission time of the target user. The value of  is determined as following:

- Case 1a (data transmission bandwidth of 50 RBs):  =BWcarrier is the whole bandwidth including system guard band (see above figure).

- For CP-OFDM,  = BWcarrier;=10 MHz

- For CP-OFDM with WOLA,  = BWcarrier;= 15K*300+15K*1 (DC))+0.61 M = 9.61 MHz

- For FB-OFDM,  = BWcarrier;=15K*300+15K*1 (DC))+90K=9.105 MHz

- Case 1b (data transmission bandwidth of 1 RB):  is the bandwidth of 1RB plus guard tones.

- For CP-OFDM,  = (12+33)*15K = 675 KHz

- For CP-OFDM with WOLA,  = (12+18)*15K = 450 KHz

- For FB-OFDM,  = (12+3)*15K = 225 KHz

The simulations use the waveform transmission/reception as described in [5] for CP-OFDM with WOLA, and the waveform transmission/reception as described in [4] for FB-OFDM. In the simulations of FB-OFDM, zero-forcing is used in polyphase filter on receiver side to suppress inter-symbol interference, and a subframe-level cutting window as shown in Appendix 1 is applied on transmitter side.

The spectrum efficiency and BLER performance are shown in Fig.6~Fig.8 for case 1a, and in Fig.9~Fig.11 for case 1b. It can be observed from these figures that,

- In both case 1a and case 1b, the BLER performances of CP-OFDM, W-OFDM and FB-OFDM are very close to each other; but due to different widths required on system guard band, the spectrum efficiency of FB-OFDM is higher than that of W-OFDM, which is further higher than that of CP-OFDM.

- Because the ratio of guard band/guard-tones to the data bandwidth in case 1b is much higher than in case 1a, the larger differences among spectrum efficiencies for three waveforms are observed in case 1b.

Fig.6 Spectrum efficiency and BLER comparison: case 1a (50RBs), 16QAM 1/2, ETU30kmh

Fig.7 Spectrum efficiency and BLER comparison: case 1a (50RBs), 64QAM 1/2, EVA30kmh

Fig.8 Spectrum efficiency and BLER comparison: case 1a (50RBs), 256QAM 1/2, EPA3kmh

Fig.9 Spectrum efficiency and BLER comparison: case 1b (1RB), 16QAM 1/2, ETU30kmh

Fig.10 Spectrum efficiency and BLER comparison: case 1b (1RB), 16QAM 1/2, EVA30kmh

Fig.11 Spectrum efficiency and BLER comparison: case 1b (1RB), 16QAM 1/2, EPA3kmh

Observation 3: Under the selected simulation assumptions, the BLER performances of FB-OFDM are very close to (or almost the same as) that of CP-OFDM and CP-OFDM with WOLA.

Observation 4: The smaller system guard band or less guard-tones required by FB-OFDM contributes to its higher spectrum efficiency than CP-OFDM and CP-OFDM with WOLA, in both case 1a and case 1b.

Proposal: FB-OFDM should be taken as a waveform candidate in NR.

# Conclusion

This contribution concludes with the following observations and proposal:

Observation 1: PAPR performance of CP-OFDM, W-OFDM and FB-OFDM are very close to each other.

Observation 2: The OOB energy of FB-OFDM falls much more rapidly than CP-OFDM and CP-OFDM with WOLA, which results in narrower system guard band and less guard-tones between subbands than the other two waveforms.

Observation 3: Under the selected simulation assumptions, the BLER performances of FB-OFDM are very close to (or almost the same as) that of CP-OFDM and CP-OFDM with WOLA.

Observation 4: The smaller system guard band or less guard-tones required by FB-OFDM contributes to its higher spectrum efficiency than CP-OFDM and CP-OFDM with WOLA, in both case 1a and case 1b.

Proposal: FB-OFDM should be taken as a waveform candidate in NR.

# References

- 3GPP RP-160671, New SID Proposal: Study on New Radio Access Technology, NTT DOCOMO

- 3GPP R1-163558, WF on evaluation assumptions for waveform, Huawei, HiSilicon

- 3GPP R1-163935, WF on evaluation assumptions for waveform, Huawei, HiSilicon, InterDigital

- 3GPP R1-164265, Discuss on FB-OFDM of New Waveform for NR, ZTE

- 3GPP R1-162199, Waveform candidates, Qualcomm

- Datta, Rohit, et al. "FBMC and GFDM interference cancellation schemes for flexible digital radio PHY design." Digital System Design (DSD), 2011 14th Euromicro Conference on. IEEE, 2011.

# Appendix 1

1ms

+

A raise cosine function window

1ms

Fig.12 Applying a raise cosine function window to the sub-frame with the extended energy

# Appendix 2

Table 1 Parameters for case 1a/1b

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Duplex | FDD/TDD (FDD and TDD have the same performance in these simulation results in this contribution) |
| System Bandwidth | 10 MHz |
| TTI length | 1 ms |
| Subcarrier spacing | 15KHz |
| Guard time interval | 4.7us (interval of LTE normal CP). The symbol interval of FB-OFDM is 1/14ms just the same as LTE |
| FFT size | 1024 |
| Data transmission bandwidth | 4 and 1 PRBs for the bandwidth of target UE in case 1b |
| Antenna configuration | 1T1R |
| MIMO mode | SISO |
| Rank per UE | Fixed single rank |
| MCS | Fixed. 16QAM: 1/2 or 2/3; 64QAM: 1/2 or 3/4; 256 QAM: 1/2 or 3/4 |
| Control Overhead | Zero |
| Channel estimation | Ideal |
| Channel Model | ETU 30km/h for 16QAM; EVA 30km/h for 64QAM; EPA 3km/h for 256QAM |
| Others | The window process of CP-OFDM with WOLA in transmitter and receiver is the same as that described in [5]. |