3GPP TSG RAN WG1 Meeting #85	R1-164370

Nanjing, China, May 23-27, 2016

Agenda Item:	7.1.3.2

Source:	Huawei, HiSilicon

Title:	SLS evaluation methodology for grant-free based multiple access

Document for:	Discussion and decision

# Introduction

In RAN1-84bis meeting [1], it was agreed that

- Non-orthogonal multiple access should be investigated for diversified NR usage scenarios and use cases

- At least for UL mMTC, autonomous/grant-free/contention based non-orthogonal multiple access should be studied.

To study the system-level performance on the grant-free contention based non-orthogonal multiple access schemes, the evaluation assumptions and methodology should first be agreed to facilitate meaningful and fair comparisons of the different multiple access schemes. Due to the wide range of services, including mMTC, URLLC, etc. each with its own deployment scenarios [2], the system level performance evaluation can become quite challenging and extensive.

In this contribution, we discuss the evaluation methodology of grant-free contention based non-orthogonal multiple accesses for the UL. It covers for non-eMBB services, which include the mMTC and URLLC usage scenarios. Note that since URLLC service is both latency- and reliability-constrained, its uplink data transmission would benefit from a grant-free access scheme that can provide a low latency transmission.

# Discussion

For the initial evaluations, it is challenging for each company to study all the different non-orthogonal MA schemes and be able to provide direct performance comparisons among them. It would be more reasonable that each company compares its proposed schemes with a baseline system. We have identified two possible baseline systems.

Baseline 1 is the LTE orthogonal multiple access for MTC support as given in [5]: This is a grant-based transmission, in which arriving user packets will first use PRACH channel for random access.  If there is no RACH collision, the packet transmissions are assumed successful; if there is a random access collision (i.e., more than one user to take a same preamble for a concurrent access), the packets will fail. The performance metrics should include the supported device connection density, as defined in [2] (and additional signalling overhead required over grant-free transmissions).  It can be seen that the number of users supported will depend on the configuration of PRACH channels and preambles. Reference configuration parameters are proposed in Table 1.

Table 1: Evaluation configurations and assumptions

| Parameter | Setting |
| --- | --- |
| PRACH configuration index | 14 |
| Preamble format | 0 |
| Total # of preambles used for RA | 64 |
| Traffic model | e.g., [2],[6] |
| Capacity metrics: | Traffic load to support ~ 99% successful pkt rate |

Baseline 2 is the LTE orthogonal multiple access under grant-free contention based transmission framework (instead of current grant-based UL transmissions), which enables data packets to “arrive and go” [3]. This baseline provides a framework that allows considerations on the possible signalling and messages overhead savings and to check if the achieved performance can meet the NR requirements as agreed in [2]. More details on grant-free contention based schemes are given in [3].

Proposal 1: The LTE orthogonal MA scheme should be considered as a baseline, defined as either Baseline 1 or Baseline 2, when evaluating a non-orthogonal multiple access scheme.

For mMTC, urban coverage for massive connection scenario will be used for evaluation, with parameters and scenarios, including ISD and traffic model, aligned with [2][6]. For URLLC, more detailed evaluation scenarios and assumptions are discussed ongoing, and FFS.

For evaluation of an MA scheme, we should provide a few fundamental intermediate results,  in order to calibrate and fairly compare the performance. Towards this end, Table 2 provides a guideline for grant-free contention based multiple access evaluation on top of the evaluation assumptions in [6].

Table 2: Simulation items and published intermediate results

| Evaluation in-process results | Significance and Notice |
| --- | --- |
| Single user AWGN curves (link level) | To provide common background in understanding single user performance. For both baseline or MA scheme with only a few MCSs (see: Pre-defined MCS below and also in [3]). |
| Multiple user MA performance: PHY abstraction for SLS simulation from LL | This is to demonstrate how performance degradation incurred by multiple user access. |
| Post-SiNR in SLS | This is a good point for calibration before detection. |
| Open loop power control: α and P0 | Note that we should keep the same power consumption per symbol per user between two MAs for fair comparison. |
| Receiver type and complexity estimation | This will provide reception type and complexity. |
| Pre-defined MCS: TB size and how many RB(s) used in each transmission per user | Using a pre-defined MCS is for reduced complexity in grant-free blind and joint signal detection. |
| KPI performance - mMTC: Connection density / connection efficiency [2] - URLLC (FFS): Latency & reliability | This is served as a starting point, more KPIs can be evaluated after , e.g., UE battery life for mMTC, etc. Note that eMBB short packets in UL transmissions can also be evaluated under this framework [3]. |

In summary, system level evaluation should provide both a baseline and proposed non-orthogonal multiple access schemes. With above in-process results, fair MA performance comparisons are possible.

Proposal 2: SLS evaluations should be performed for a baseline and the proposed non-orthogonal multiple access scheme(s), accompanied by the intermediate results as proposed in Table 2 for calibration purposes.

# Conclusion

In this contribution, we have proposed a baseline system that can be adopted for the SLS evaluation of a  grant-free contention based non-orthogonal multiple access and some additional proposals on intermediate check points that would be useful calibrations for a fair performance comparisons among the different MA schemes.

Proposal 1: The LTE orthogonal MA scheme should be considered as a baseline, defined as either Baseline 1 or Baseline 2, when evaluating a non-orthogonal multiple access scheme.

Proposal 2: SLS evaluations should be performed for a baseline and the proposed non-orthogonal multiple access scheme(s), accompanied by the intermediate results as proposed in Table 2 for calibration purposes.

# References

3GPP Chairman Notes, RAN1#84bis, Busan, Korea, April 2016.

3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”.

R1-164036, “Multiple access for UL small packets transmission,” Huawei, HiSilicon, RAN1#85, Nanjing, China, May 2016.

R1-164037, “LLS results for uplink multiple access,” Huawei, HiSilicon, RAN1#85, Nanjing, May 2016.

3GPP TR 37.868 v11.0.0 (2011-09).

R1-164045, “SLS parameters for evaluation scenarios for multiple access evaluation,” Huawei, HiSilicon, RAN1#85, Nanjing, China, May 2016.