3GPP TSG RAN WG1 Meeting #85	R1-164390

Nanjing, China, May 23-27, 2016

Agenda Item:	7.1.3.2

Source:	Huawei, HiSilicon

Title:	Transceiver implementation and complexity analysis for SCMA

Document for:	Discussion

# Introduction

In RAN1#84bis, the evaluation method for multiple access evaluation was discussed and the following metrics for link level evaluation have been agreed [1]:

- For LLS, consider the following as evaluation metrics

  - General

    - BLER vs SNR reported for UL and DL calibration

    - BS and UE receiver complexity reported

  - UL:

    - Sum throughput v.s. SNR at given BLER level under different overloading factor*.

    - Link budget (MCL with specific data rate)

    - Other metrics FFS (e.g., Maximum overloading factor v.s. SNR at given user throughput)

  - DL:

    - Rate region

    - Optional: Sum normalized user throughput (normalized by throughput in orthogonal case); Sum throughput with minimum throughput constraint for some users

In this contribution, we will discuss about the transceiver implementation of SCMA [2] to strike a good balance between link performance and implementation complexity.

# General Description on SCMA Transceiver Structure

For downlink SCMA transmission, the abstracted transceiver structure can be expressed in Figure 1. The only difference in uplink will be the separate SCMA modulator after FEC encoding for each user.

- Abstracted transmitter structure for downlink SCMA

- Abstracted receiver structure for downlink SCMA

Figure 1: Abstract transceiver structure of SCMA.

As we can see, the major modules that distinguish SCMA from LTE in the link implementation are the SCMA modulator and demodulator, which are very similar for both uplink and downlink. Also note the transceiver design can be well combined with different types of FEC encoder/decoder, different waveforms, as well as different MIMO modes.

# Transmitter Implementation and Complexity Analysis

At the transmitter side, the major change is the replacement of QAM modulation with SCMA modulator, which maps the coded bits directly to the multi-dimensional codeword of pre-designed codebooks, as shown in Figure 2. Therefore, as long as we have the codebooks in hand, SCMA modulation is a codeword selection procedure with negligible complexity.

Figure 2: Example transmitter diagram of LTE and SCMA for UL transmission.

In practical implementations, the codebooks are pre-stored at both transmitter and receiver sides just like the current modulation (bits-to-symbol mapping) tables. The selection of the codeword is similar to the selection of MCS configuration, whose index can be indicated through scheduling information from base station to a target user in granted transmission case, or it can be pre-assigned or randomly selected in grant-free transmission case.

Observation 1: At the transmitter side, the major change introduced by SCMA implementation is the replacement of QAM modulation to SCMA modulator, which is a codebook mapping process with negligible complexity.

In the design of bits to codeword mapping (reflected in the codebook design), instead of traditional linear spreading in which the symbols mapped on the non-zero tones are repetitions of a QAM symbol with some phase and/or amplitude adjustment (m1=m2), we consider non-repetition based bits to symbol-vector mapping (m1≠m2). Figure 3 illustrates the difference between the two ways. The benefits of considering such non-repetition based design are 2-folds:

- It enhances the distance properties of the joint constellation, which could then bring obvious shaping/coding gain over the repetition based mapping, as shown in [3]. This gain is due to the fact of using the full 4-dimensional real space for the codebook design, instead of only using the bisector plane of a 4-dimensional real space as in the repetition case.

- It allows for the possibility of having lower number of projections (e.g., instead of 16 QAM, M=16, only 9 projections in the constellation, Mp=9, but still with 4 bits mapping), which greatly reduces the demodulation complexity at the receiver [4].

Figure 3: Different ways of bits to symbol-vector mapping.

Therefore, when symbol level spreading is considered, it is suggested to consider non-repetition based bits to symbol-vector mapping, which is included in the codebook design and will not increase the implementation complexity at the transmitter.

Proposal 1: When spreading is considered, non-repetition based bits to symbol-vector mapping is encouraged to be used for better utilization of available resources, and to achieve enhanced link performance and reduced demodulation complexity at the receiver.

# Consideration on Receiver Design and Complexity Analysis

At the receiver side, as similar to all the other non-orthogonal multiple access schemes, multi-user detector is needed to decode multiple data layers superposed together. Specifically, the single user channel equalization and QAM de-mapper are replaced by SCMA demodulator, which jointly detect the superposed data layers and output separate LLR results to the turbo decoders of each layer. Figure 4 illustrates the receiver diagrams for LTE and SCMA and their difference.

# Candidate Receivers

Different receivers can be applied to SCMA. Among all the candidates, the simplest is the SIC receiver (SLIC or CWIC) that decodes users/data layers one after another. It works well when the received SNR among users/data layers are quite different from each other. However, it suffers when the SNR difference is not obvious between users/data layers, in which case error propagation happens. Moreover, when there is correlation between user channels, the performance of SIC receiver also degrades.

On the other extreme of the candidates is the full MPA receiver that jointly decodes all users/data layers. Thanks to the sparseness in the codebook design of SCMA, we can use MPA receiver to achieve close to joint ML detection performance, while the complexity of MPA receiver compared with the original ML receiver is greatly reduced. Moreover, it does not have constraint of SNR difference at the receiver, and is more robust to channel correlations between users, as compared with SIC receiver. However, the complexity of full MPA receiver is higher than pure SIC receiver.

To strike a good balance between link performance (close to ML detection and robust to channel imperfection) and implementation complexity (the complexity of the receiver), we could actually combine the features of SIC with MPA to have SIC-MPA receiver. More specifically, MPA is first applied to a limited number of layers, so that the number of colliding layers over each RE does not exceed a given threshold value df, which are referred to as MPA layers. Then, the successfully decoded MPA layers are removed by SIC and the procedure continues until all layers are successfully decoded or no new layer gets successfully decoded by MPA. Due to the fact that MPA is used for a very limited number of layers instead of all the layers, the decoding complexity is greatly reduced.

On the other hand, in the downlink, due to the fact that at each user’s detector all the data layers go through the same channel, techniques such as IQ separation can be applied to reduce the complexity for SCMA decoder, i.e., the IQ-MPA receiver. It can be theoretically proved that the performance of MPA and IQ-MPA is exactly the same.

The IQ separation technique used in IQ-MPA can be further combined with SIC-MPA to reduce receiver complexity to a larger extent, which gives SIC-IQ-MPA receiver.

All candidate receivers mentioned above are described with complexity order analysis in the appendix.

Proposal 2: To strike a good balance between link performance and implementation complexity, SIC-MPA can be applied for UL SCMA, while IQ-MPA or SIC-IQ-MPA can be applied for DL SCMA.

Figure 4: Example receiver diagram of LTE and SCMA for UL transmission.

# Complexity Analysis

In the following, we summarize the complexity analysis results. In particular, we analyze the order of complexity for SIC-MPA receiver, IQ-MPA receiver, SIC-IQ-MPA receiver, and compare with MMSE-SIC receiver and full MPA receiver as baselines.

Table 1 gives the explanation of the key parameters, and Table 2 summarizes the complexity analysis results for different receiver types. Note that the complexity order provided in Table 2 only includes the dominant terms, and the detailed number of calculation depends heavily on the specific hardware implementation, which is too early to be discussed for now.

Table 1: Key parameters in the complexity order analysis.

| Parameters | Description | Example values |
| --- | --- | --- |
| NRx | Number of receiver antennas | 2, 4, 8, etc. |
| SF | Spreading length of a spreading block | 4, 8, etc. |
| NUE | Number of users/data layers | UL: 6, 12, etc. DL: 2, 3, etc. |
| Niter | Including inner-loop and number of MPA rounds | 6, 9, 18, etc. |
| Mp | Number of projection points on the constellation for log2M bits mapping (M≥Mp) | 4 (M=8), 9 (M=16) |
| d(i) | degree-of-freedom of FN i in full-MPA implementation, representing the number of symbol collisions on the ith RE in the spreading block | 3, 6, etc. |
| df | Maximum degree-of-freedom allowed in SIC-MPA receiver, representing the maximum number of symbol collisions left for MPA decoding in the SIC-MPA receiver | 1, 2, 3, etc. (note df=1, equivalent to SIC, df=maxid(i), equivalent to MPA) |

Table 2: Summary of complexity analysis of different receiver types.

| Rx Type | Operation Complexity Order per spreading block (Only dominant part considered) |
| --- | --- |
| MMSE-SIC (baseline) | O(NRx3SF3NUE) |
| MPA (baseline) | ONiteri=1SFMpd(i) |
| SIC-MPA | OSF∙NiterMpdf |
| IQ-MPA (DL only) | O2Niteri=1SFMpd(i)2 |
| SIC-IQ-MPA（DL only） | O2SF∙NiterMpdf2 |

In the following, we give two numerical examples to show the complexity comparison between different receivers for UL and DL, respectively.

Example 1: UL SIC-MPA v.s. MMSE-SIC

Table 3: Typical values for SCMA parameters and results for complexity order comparison.

| SCMA Parameter | NRx | SF | NUE | Niter | Mp | d(i) | df | MMSE-SIC | MPA | SIC-MPA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value Set 1 | 2 | 4 | 6 | 9 | 4 | 3 | 2 | O(3072) | O(2304) | O(576) |
| Complexity ratio compared with baseline |  |  |  |  |  |  |  | 18.75% / 5.3x less | 25% / 4.0x less | -- |
| Value Set 2 | 4 | 4 | 12 | 18 | 4 | 6 | 2 | O(49152) | O(294912) | O(1152) |
| Complexity ratio compared with baseline |  |  |  |  |  |  |  | 2.34% / 43.5x less | 0.4% / 256.0x less | -- |

To validate the performance of SIC-MPA, we do the LLS with the two set of parameters in Table 3 for 12RB bandwidth and 4RB bandwidth, respectively. The target spectrum per user is set to 0.25, and C84 (M=8, Mp=4) codebook is used. According to Figure 11 and Figure 12, the SIC-MPA receiver implies almost no performance penalty compared to the full MPA receiver for both cases.

Figure 11：Performance comparison between full MPA receiver and SIC-MPA receiver in UL, Value set 1 and 12RB bandwidth.

Figure 12：Performance comparison between full MPA receiver and SIC-MPA receiver in UL, Value set 2 and 4RB bandwidth.

Observation 2: The complexity of SIC-MPA is lower than MMSE-SIC for the scenarios with application interest, while the performance gap between SIC-MPA and MPA is negligible in these cases.

Example 2: DL IQ-MPA v.s. full MPA

Consider DL scenarios where 4 layers of a 6-by-4 SCMA codebook are used by 2 users with each user taking 2 layers. Given the C169 (M=16, Mp=9) codebook, with IQ-MPA receiver, about 10 times of complexity reduction can be achieved compared with the original full MPA receiver. And the performance of IQ-MPA receiver can be approved to be exactly the same as full MPA receiver.

Table 4: Example complexity of MPA and IQ-MPA w and w/o low projection codebook.

| SCMA Parameter | Mp | df | Niter | Overall Complexity order per tone | Complexity Ratio/times |
| --- | --- | --- | --- | --- | --- |
| MPA | 16 | 2 | 9 | O9×162=O(2304) | -- |
| MPA with low projection codebook | 9 | 2 | 9 | O9×92=O(729) | 31.6% / 3.2x less |
| IQ-MPA with low projection codebook | 9 | 1 | 9 | O2×9×91=O(162) | 7% / 14.2x less |

Observation 3: The complexity of IQ-MPA is reasonable for device implementation, and is about 10 times lower than full MPA with low projection codebooks.

# Conclusions

In this contribution, we discussed the transceiver implementation and complexity analysis of SCMA. We obtained the following observations.

Observation 1: At the transmitter side, the major change introduced by SCMA implementation is the replacement of QAM modulation to SCMA modulator, which is a codebook mapping process with negligible complexity.

Observation 2: The complexity of SIC-MPA is lower than MMSE-SIC for the scenarios with application interest, while the performance gap between SIC-MPA and MPA is negligible in these cases.

Observation 3: The complexity of IQ-MPA is reasonable for device implementation, and is about 10 times lower than full MPA with low projection codebooks.

From the discussion and the observations, we draw the following proposals.

Proposal 1: When spreading is considered, non-repetition based bits to symbol-vector mapping is encouraged to be used for better utilization of available resources, and to achieve enhanced link performance and reduced demodulation complexity at the receiver.

Proposal 2: To strike a good balance between link performance and implementation complexity, SIC-MPA can be applied for UL SCMA, while IQ-MPA or SIC-IQ-MPA can be applied for DL SCMA.

# References

Huawei, HiSilicon, “Way forward on assumptions for multiple access evaluation”, R1-163560, Busan, Korea, April 11-15, 2016.

Huawei, HiSilicon, “Sparse Code Multiple Access (SCMA) for 5G radio transmission”, R1-162155, Busan, Korea, April 11-15, 2016.

R1-162163, “Applying Multi-Dimensional Modulation to Spreading Sequence Design,” Huawei, HiSilicon, RAN1#84bis, Busan, South Korea.

A. Bayesteh, H. Nikopour, M. Taherzadeh, H. Baligh, and J. Ma, “Low Complexity Techniques for SCMA Detection”, in IEEE GlobeCom 2015, San Diego, CA, Dec. 2015.

# Appendix: Discussion on Different Receiver Types

# SIC Receiver

SIC receiver is one class of multi-user (or data layer) receiver that treat all the other undecoded users/layers as interference when decoding one target user/layer. Depending on in which step the cancelation is done, it can be implemented as either symbol level IC (SLIC) or codeword level IC (CWIC).

In general, for the decoding of each target layer, the following steps are needed:

- User selection. Ordering (the selection of decoding order for each user/layer) is important for any type of SIC receivers. This is because the eariler a user/layer to be decoded, the larger the residual interference it faces with and the higher would be the probability of error propagation.

- MIMO equalization. Depending on whether the equalization takes into consideration the cross-tone correlations, there are single tone MIMO equalization and multi-tone MIMO equalization.

- Demapping. After MIMO equalization, the remaining procedure is single user/data layer demapping.

The abstract diagram for SIC receiver with  CWIC implementation is shown in Figure 5.

# MMSE-SIC Receiver

This receiver takes advantage of the correlation both over antenna domain and code domain, provided that the Tx signal is correlated over the spreading block. MMSE-SIC would be a good choice when linear spreading is used. In this case, the Tx covariance matrix can be written in the form of SSH, where S denotes the signature vector. MMSE-SIC is the baseline Rx for our complexity analysis. The decoding complexity of MMSE-SIC is mainly dominated by MIMO equalization, which involves calculation of the inverse covariance matrix of size NRxSF×NRxSFfor all spreading blocks with the total complexity of O(NRx3SF3NUE) per spreading block, where SF denotes the spreading factor and NRx denotes the number of Rx antennas, and NUE denotes the total number of users to be decdoed.  It can be seen that the complexity of MMSE-SIC grows rapidly with the number of Rx antennas.

Figure 5: The abstract structure of the CWIC SIC receiver

# Full MPA Receiver

Instead of decoding the users one by one as in SIC receiver, MPA receiver decodes all users/data layers jointly. Thanks to the sparseness in the codebook design of SCMA, we can use MPA receiver to achieve close to joint ML detection performance for multi-users with greatly reduced complexity.

Figure 6: The abstract structure of the MPA receiver

The MPA algorithm is performed on the tanner graph constructed by the codebook. Every FN (representing RE) and VN (representing data layer) which is connected in the tanner graph makes a FN-VN pair. MPA starts with the initial conditional probability calculation at each FN. Received signal on each RE, channel estimation on each RE from each user, as well as the noise estimation on each RE are all needed as input in this step. Then it enters message passing iterations between FNs and VNs along the edges. For each iteration, there are two steps, known as FN update and VN update, respectively, which is done independently for each FN-VN pair. After enough iterations, log-likelihood-ratio (LLR) for the coded bits are calculated based on the codeword probability and output at the VN and serve as the input to the turbo decoder thereafter. The MPA iteration process is represented in the following figure.

Figure 7: The iteration process of MPA.

Though MPA receiver can achieve close to ML performance with orders less complexity compared with ML receiver, the original MPA receiver still has much higher complexity than SIC receiver, especially when the modulation order is high or the number of users is high.

The main complexity of MPA receiver is determined by calculating exp(.) terms for all combinations at each FN (before the message passing algorithm) and FN update (during message passing algorithm) which depends on the number of layers colliding over each FN. The complexity order of the receiver, per spreading block, can be expressed as Niteri=1SFMpd(i) , where Niter denotes the total number of iterations (inner and outer loop),  Mp denotes the number of projection points on the constellation for log2M bits mapping (M≥Mp), and d(i) denotes the number of collisions over the ith FN.

# Reduced Complexity MPA (R-MPA) Receivers

# SIC-MPA Receiver

The SIC-MPA receiver is the combination of SIC receiver and MPA receiver, and is developed as a reduced complexity implementation of the original MPA receiver. More specifically, MPA is first applied to a limited number of layers, so that the number of colliding layers over each FN does not exceed the threshold value df, which are referred to as MPA layers. Then, the successfully decoded layers are removed by SIC and the procedure continues until all layers are successfully decoded or no new layer gets successfully decoded in MPA. The receiver structure is illustrated in Figure 8. Due to the fact that MPA is used for a very limited number of layers instead of all the layers, the decoding complexity is greatly reduced.

Figure 8: Abstract structure of SIC-MPA receiver.

The main complexity of SIC-MPA receiver is determined by calculating exp(.) terms for all signal combinations for the MPA layers at each FN (before the message passing algorithm) and FN update (during message passing algorithm) for the MPA layers which depends on the number of layers colliding over each FN. These calculations are for each MPA round. The complexity order of the receiver, per spreading block, can be expressed as OSF∙NiterMpdf, where Niter denotes the total number of iterations (including inner-loop and number of MPA rounds), Mp denotes the number of projection points on the constellation for log2M bits mapping (M≥Mp).

# IQ-MPA Receiver

IQ-MPA receiver is another type of reduced complexity MPA receiver, which is mainly used in the downlink. In the downlink, due to the fact that at each user’s detector all the data layers go through the same channel, techniques such as IQ separation can be applied to reduce the complexity for SCMA decoder. Figure 9 gives the abstract structure of IQ-MPA receiver.

Figure 9: Abstract structure of IQ-MPA receiver.

The calculation of complexity of IQ-MPA receiver is the same as the regular MPA receiver, with the following modifications:

- The total number of signal combinations at each FN, only depends on the number of projections over the real/imaginary axis instead of all projection points over the whole complex plain. If the total number of projection points is denoted by Mp, the number of projections over each real/imaginary axis would be Mp. Therefore, the decoding complexity per MPA is of order 
ONiteri=1SFMpd(i)2.

- There are two separate MPAs for the I and Q projections. So, the overall complexity can be expressed as O2Niteri=1SFMpd(i)2.

# SIC-IQ-MPA Receiver

The IQ separation technique used in IQ-MPA can be further combined with SIC-MPA to reduce receiver complexity to a larger extent, which gives SIC-IQ-MPA receiver. Figure 10 gives the abstract structure of SIC-IQ-MPA receiver.

Figure 10: Abstract structure of SIC-IQ-MPA receiver.

Noting the complexity calculations for MPA, SIC-MPA and IQ-MPA receivers, the complexity order of  SIC-IQ-MPA receiver can be expressed as  O2SF ∙NiterMpdf2, where Niter denotes the total number of iterations (including inner-loop and number of MPA rounds), NUE denotes the number of UEs, Mp denotes the total number of projection points on the complex domain for log2M bits mapping (M≥Mp), and df denotes the maximum allowed collision over each FN in the MPA.