3GPP TSG RAN WG1 Meeting #85                                        	     R1-164483

Nanjing, China 23rd - 27th May 2016

Agenda item:	6.2.3.1.1

Source:	NEC

Title:               Discussion on CSI-RS with CDM8

Document for:	Discussion and Decision

- Introduction

In RAN#71 meeting, a new WID of enhancement on Full-Dimension (FD) MIMO for LTE in Rel-14 was approved [1]. One of the detailed objectives is CSI-RS enhancement as below:

- Specify enhancements on reference signal in the following areas [RAN1]

  - Non-precoded CSI-RS, extending the existing numbers {1, 2, 4, 8, 12, 16} of CSI-RS antenna ports for support of {20, 24, 28, 32} CSI-RS ports with mechanism for reducing the overhead for CSI-RS transmission

In the current Rel-13 standard, OCC=4 is introduced to achieve the benefit derived from both CDM4 and full power utilization for non-precoded CSI-RS. However, for up to 32 ports, full power boosting gain may not be achieved anymore. So we propose higher order OCC in order to solve this issue in this contribution.

- Discussion

With 6dB power boosting, CDM4 of CSI-RS may have full power utilization for 16 CSI-RS ports in Rel-13, where signals of 4 port CSI-RS are transmitted on the same four REs as described in the Figure 1.

Figure1 16 port CSI-RS with CDM4

However, by CDM4, full power utilization may not be achieved for more than 16 CSI-RS ports because of 6dB power boosting limitation in RAN4 as described in the Figure 2 for 24 port CSI-RS example with OCC=4.

Figure2 24 port CSI-RS with CDM4

To achieve full power boosting utilization and meet the requirement of 6dB limitation, we propose to introduce CDM8 to non-precoded CSI-RS for more than 16 ports.

Proposal 1: CDM8 should be introduced for more than 16 CSI-RS port.

There are two possible schemes to aggregate one CDM8 group.

- Alt.1 8 REs in one CDM8 group are from one legacy 8 port CSI-RS configuration resource, as described in Figure 3a (left figure).

- Alt.2 8 REs in one CDM8 group are partitioned into two groups of 4REs, and these two group of 4REs are from two CSI-RS configuration resources, as described in Figure 3b (right figure).

Figure 3a and 3b CSI-RS pattern with CDM8

Proposal 2: Both the above two alternatives should be studied for CDM8 resource aggregation.

It is noted that CDM8 based scheme can combine with lower density scheme, e.g. 0.5 RE/RB/port.

Especially for Alt.2, similar with the discussion in [2], sharing resource elements with Rel-13 UEs may be possible, and the detailed design can be further studied.

Proposal 3: Sharing resource elements with Rel-13 UEs should be FFS.

- Conclusion

In this contribution, we discussed design for non-precoded CSI-RS extension with CDM8 and proposed:

Proposal 1: CDM8 should be introduced for more than 16 CSI-RS port.

Proposal 2: Both the above two alternatives should be studied for CDM8 resource aggregation.

Proposal 3 Sharing resource elements with Rel-13 UEs should be FFS.

- References

- RP-160623, "New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE", Samsung, 3GPP TSG RAN#71, Göteborg, Sweden, 7th - 10th March, 2016

[2]  R1-156679, "Proposals for CSI-RS with OCC=4", NEC, 3GPP TSG RAN WG1 Meeting #83, Anaheim, USA,

15th - 22nd November 2015