3GPP TSG RAN WG1 Meeting #85                                                             R1-164637

Nanjing, China, 23rd – 27th May 2016

Source:	ZTE, ZTE Microelectronics

Title:	System-level performance evaluation for TDD

Agenda Item:	6.2.10.3

Document for: 	Discussion and Decision

# Introduction

In the Release 13 Study Item on Latency reduction techniques for LTE [1], RAN1 needs to study the TTI shortening and reduced processing times, including the following aspects:

- Assess specification impact and study feasibility and performance of TTI lengths between 0.5ms and one OFDM symbol, taking into account impact on reference signals and physical layer control signaling.

- Backwards compatibility shall be preserved (thus allowing normal operation of pre-Rel-13 UEs on the same carrier).

During the RAN#71, it is further clarified that the study on latency reduction techniques for LTE in 2016 Q2 is focused for: frame structure type 1 and frame structure type 2.

This contribution provides system-level evaluation results of latency reduction for F2 discussed in [2].

# Evaluation assumptions

As discussed in [2], the latency in single TDD carrier is not reduced in a way proportional to the TTI length reduction due to the restriction of UL/DL configuration, which is different from latency reduction in FDD. To achieve latency reduction comparable to FDD sTTI, new switching points may be considered on top of the existing TDD UL/DL configurations.

In order to evaluate the short TTI gains in FS2, legacy TDD DL/UL configuration#2 and special subframe configuration 8(DpPTS:GP:UpPTS =11:1:2) are compared.

For TTI shortening case, it keeps legacy configuration#2, and all downlink/uplink/special subframes are fixed as downlink/uplink/special subframes. Besides, up to 2 symbols can be configured for UpPTS for uplink transmission and can only be used for SRS and PRACH transmission.

For new subframe type case, the location of the new subframe type may affect the time delay of frame alignment and HARQ retransmission. So, we simulate two examples of the new subframe type that are recommended in [2] as shown in Figure 1 and Figure 2. The SC-FDMA symbols in UpPTS can be used for sPUCCH/sPUSCH transmission in the new subframe type.

Figure1: sTTI=7 symbols with different subframe type

Figure2: sTTI=2 symbols with different subframe type

The evaluations in this contribution are performed in an ITU macro layout of 7 macro sites and 3 cells per site, with wrap-around feature turned on. The FTP traffic model2 and TCP traffic are used with model parameters given in Table 1.

Table 1 FTP traffic model 2 and TCP model parameters

| Parameter | Assumption |
| --- | --- |
| FTP traffic model | FTP Traffic Model 2 [3] - Reading Time: Exponential Distribution, with parameter TBD for specific traffic loads |
| File size/ reading time D | 100KB / D = 1s / D= 1/2s / D= 1/4s 500KB / D = 5s / D = 3s / D = 2s |
| TCP model | - SSThresh 65535 Bytes - Initial window size 1460 Bytes - MSS 1460 Bytes - 40 Bytes TCP header + IP header - The three way handshake is not modelled as baseline. |

In every successful LTE transmission, one uplink TCP ACK per maximum segment size (MSS) with a fixed uplink TCP ACK delay can be generated for TCP traffic as can be seen in Table 2.

Table 2: Uplink TCP ACK transmission latency calculation

| Step | Description | Delay |
| --- | --- | --- |
| 1. | Encoding of (SR) sPUCCH | UL Frame Alignment |
| 2. | UE Transmission of (SR) sPUCCH | 1 TTI |
| 3. | eNB Decoding of (SR) sPUCCH + Encoding UL grant | 1.5TTI+1TTI+DL Frame Alignment |
| 4. | eNB Transmission of UL grant (assumed always error free) | 1 TTI |
| 5. | UE processing delay (decoding Scheduling grant + L1 encoding of data) | 1.5TTI+1TTI+UL Frame Alignment |
| 6. | UE sends UL transmission | (1 + p*8) TTI where p is initial BLER. p=0 |
| 7. | eNB receives and decodes the UL data | 1.5 TTI |
| Sum |  | 9.5TTI+DL Frame Alignment + 2*UL Frame Alignment |

As shown in table 3, DL/UL frame alignment in case of the different TTI length is given.

Table 3: DL/UL frame alignment for different sTTI length

| TTI length | UL Fame Alignment | D L Fame Alignment |
| --- | --- | --- |
| 14OS | 2.5TTI | 0.7TTI |
| 7OS | 4.1TTI | 0.8TTI |
| 2OS | 11.3TTI | 1.53TTI |
| Example 1 of 7OS | 2.7TTI | 0.9TTI |
| Example 2 of 2OS | 4.5TTI | 1.59TTI |

# Evaluation results

## 3.1 Performances of TTI shortening with existing UL/DL configuration

| Parameter | Assumption |
| --- | --- |
| CN delay | 6ms |
| UL TCP ACK delay | 9.5TTI+DL Frame Alignment + 2*UL Frame Alignment |
| CSI feed back | CSI delay: 6TTI |
| UE speed | 3km/h |
| File size | 0.1Mbyte, 0.5Mbyte |
| TTI length | 1ms, 0.5ms, 2OS |

The following evaluation results aim to show the impact of different resource utilizations varying from 23% to 75.6%, assuming the shortened TTI length equal to 14OS, 7OS, 2OS and FTP file size equal to 0.1Mbyte, or 0.5Mbyte.

Table 4 and Table 5 show that short TTI gain under different loads and files. There are obvious gains with mean, 50% and 95% of UPT. However, for medium and high load, the 5% UPT (representing the cell edge performance) even has negative gain.

Table 4~Table 5 also show the gains of latency reduction under different loads. It can be seen that the mean, 5%, 50% and 95% of latency gains decrease with the increased RU. Nevertheless, TTI shortening always reduces the latencies at all percentage points.

Table 4: File size=0.1Mbyte, UPT and latency gain of under different RUs

| Reported Parameters |  | Low load |  |  | Medium load |  |  | High load |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14OS | 7OS | 2OS | 14OS | 7OS | 2OS | 14OS | 7OS | 2OS |
| UPT CDF [Mbps] | 5% Gain(%) | 3.7273 (0) | 4.3272 (+16.1) | 4.5044 (+20.8) | 3.2014 (0) | 3.7982 (+18.6) | 3.5345 (+10.4) | 2.881 (0) | 3.1407 (+9) | 2.5885 (-10.2) |
|  | 50% Gain(%) | 4.1588 (0) | 5.1108 (+23) | 6.3673 (+53.1) | 4.1387 (0) | 5.0572 (+22.2) | 6.0686 (+46.6) | 4.0938 (0) | 4.9064 (+19.8) | 5.4057 (+32) |
|  | 95% Gain(%) | 4.4846 (0) | 5.3799 (+20) | 6.6079 (+47.3) | 4.4799 (0) | 5.3674 (+19.8) | 6.5565 (+46.4) | 4.4788 (0) | 5.3559 (+19.6) | 6.4604 (+44.2) |
|  | Mean Gain(%) | 4.1519 (0) | 5.0448 (+21.5) | 6.1278 (+47.5) | 4.0839 (0) | 4.908 (+20.2) | 5.7346 (+40.4) | 3.9482 (0) | 4.6714 (+18.3) | 5.0658 (+28.3) |
| Delay CDF [s] | 5% Gain(%) | 0.1871 (0) | 0.1559 (-16.7) | 0.1269 (-32.2) | 0.1872 (0) | 0.1562 (-16.6) | 0.128 (-31.7) | 0.1872 (0) | 0.1564 (-16.5) | 0.1299 (-30.6) |
|  | 50% Gain(%) | 0.202 (0) | 0.1643 (-18.7) | 0.132 (-34.7) | 0.2033 (0) | 0.1661 (-18.3) | 0.1393 (-31.5) | 0.2056 (0) | 0.1722 (-16.2) | 0.158 (-23.2) |
|  | 95% Gain(%) | 0.2261 (0) | 0.1925 (-14.9) | 0.1803 (-20.3) | 0.2571 (0) | 0.218 (-15.2) | 0.2292 (-10.9) | 0.286 (0) | 0.2656 (-7.1) | 0.3324 (+16.2) |
|  | Mean Gain(%) | 0.2035 (0) | 0.1677 (-17.6) | 0.1399 (-31.3) | 0.2084 (0) | 0.1746 (-16.2) | 0.1556 (-25.3) | 0.2199 (0) | 0.1892 (-14) | 0.191 (-13.3) |
| RU(%) |  | 23 | 24.3 | 28.1 | 41 | 42.6 | 51.3 | 62 | 65.5 | 75.7 |
| λ |  | 1 |  |  | 2 |  |  | 4 |  |  |

Table 5 File size=0.5Mbyte, UPT and latency gain of under different RUs

| Reported Parameters |  | Low load |  |  | Medium load |  |  | High load |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14OS | 7OS | 2OS | 14OS | 7OS | 2OS | 14OS | 7OS | 2OS |
| UPT CDF [Mbps] | 5% Gain(%) | 7.0814 (0) | 7.3624 (+4) | 6.1689 (-12.9) | 6.3052 (0) | 6.1219 (-2.9) | 5.1637 (-18.1) | 5.0804 (0) | 5.0075 (-1.4) | 4.0992 (-19.3) |
|  | 50% Gain(%) | 10.3584 (0) | 12.8161 (+23.7) | 15.2424 (+47.2) | 10.1512 (0) | 12.4954 (+23.1) | 12.9008 (+27.1) | 10.0251 (0) | 11.5342 (+15.1) | 11.5781 (+15.5) |
|  | 95% Gain(%) | 11.3794 (0) | 13.6825 (+20.2) | 16.9325 (+48.8) | 11.3632 (0) | 13.6735 (+20.3) | 16.8127 (+48) | 11.3593 (0) | 13.6124 (+19.8) | 16.2753 (+43.3) |
|  | Mean Gain(%) | 10.0803 (0) | 12.1552 (+20.6) | 13.7741 (+36.6) | 9.7447 (0) | 11.703 (+20.1) | 12.445 (+27.7) | 9.2832 (0) | 10.86 (+17) | 11.0248 (+18.8) |
| Delay CDF [s] | 5% Gain(%) | 0.3684 (0) | 0.3065 (-16.8) | 0.2476 (-32.8) | 0.3686 (0) | 0.3067 (-16.8) | 0.2493 (-32.4) | 0.3689 (0) | 0.3079 (-16.5) | 0.2569 (-30.4) |
|  | 50% Gain(%) | 0.4052 (0) | 0.3271 (-19.3) | 0.28 (-31) | 0.4137 (0) | 0.3371 (-18.5) | 0.3361 (-18.8) | 0.4213 (0) | 0.3692 (-12.4) | 0.3855 (-8.5) |
|  | 95% Gain(%) | 0.576 (0) | 0.5207 (-9.6) | 0.6915 (+20.1) | 0.6587 (0) | 0.6598 (+0.2) | 0.8025 (+21.8) | 0.7916 (0) | 0.808 (+2.1) | 1.0624 (+34.2) |
|  | Mean Gain(%) | 0.4283 (0) | 0.3616 (-15.6) | 0.3436 (-19.8) | 0.4549 (0) | 0.3884 (-14.6) | 0.4112 (-9.6) | 0.5055 (0) | 0.4467 (-11.6) | 0.5032 (-0.5) |
| RU(%) |  | 24.9 | 26 | 30.2 | 39 | 37.9 | 47 | 55.6 | 57.6 | 65 |
| λ |  | 1/5 |  |  | 1/3 |  |  | 1/2 |  |  |

Observation1: TTI shortening brings latency reduction gain in case of legacy TDD DL/UL configuration.

Observation2: Regardless the value of RU and file size, 2 symbols TTI provides the best performance in case of non-cell-edge UEs, followed by 7 symbols.

## 3.2 Performances of TTI shortening with new subframe type

| Parameter | Assumption |
| --- | --- |
| CN delay | 6ms |
| UL TCP ACK delay | 9.5TTI+DL Frame Alignment + 2*UL Frame Alignment |
| CSI feed back | CSI delay: 6TTI |
| UE speed | 3km/h |
| File size | 0.1Mbyte, 0.5Mbyte |
| TTI length | 0.5ms, example1 , 2OS, example2 |

As shown in table 6 and 7, we compare the influence of new subframe type of different RU and file sizes for short TTI performance. 7 symbols TTI length with existing UL/DL configuration 2 is used to compare with example 1of new frame structure, and 2 symbols TTI length with existing UL/DL configuration 2 is used to compare with example 2 of new frame structure.

From the results shown in Table 6 and Table 7, when the TTI length is 7 symbols, we did not observe over 8% gain of the new frame structure example 1 comparing with TTI shortening with existing UL/DL configuration 2 in mean UPT. What’s more, the 5% UPT point has negative gain. For 2 symbols TTI length, we can see that the new frame structure example 2 brings 15% more  gain than TTI shortening with existing UL/DL configuration 2 in mean UPT.

Table 6: File size=0.1Mbyte, UPT and latency gain of different RUs for new subframe type

| Reported Parameters |  | Low load |  |  |  | Medium load |  |  |  | High load |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7OS | Enhanced 7OS | 2OS | Enhanced 2OS | 7OS | Enhanced 7OS | 2OS | Enhanced 2OS | 7OS | Enhanced 7OS | 2OS | Enhanced 2OS |
| UPT CDF [Mbps] | 5% Gain(%) | 4.3272 (0) | 4.2936 (-0.8) | 4.5044 (0) | 4.8637 (+8) | 3.7982 (0) | 3.4995 (-7.9) | 3.5345 (0) | 3.8172 (+8) | 3.1407 (0) | 2.772 (-11.7) | 2.5885 (0) | 2.363 (-8.7) |
|  | 50% Gain(%) | 5.1108 (0) | 5.5549 (+8.7) | 6.3673 (0) | 7.4219 (+16.6) | 5.0572 (0) | 5.4519 (+7.8) | 6.0552 (0) | 7.0183 (+15.9) | 4.9064 (0) | 5.1022 (+4) | 5.4057 (0) | 5.9849 (+10.7) |
|  | 95% Gain(%) | 5.3799 (0) | 5.827 (+8.3) | 6.6079 (0) | 7.8379 (+18.6) | 5.3674 (0) | 5.8067 (+8.2) | 6.5563 (0) | 7.8094 (+19.1) | 5.3559 (0) | 5.7776 (+7.9) | 6.4604 (0) | 7.6468 (+18.4) |
|  | Mean Gain(%) | 5.0448 (0) | 5.4087 (+7.2) | 6.1278 (0) | 7.067 (+15.3) | 4.908 (0) | 5.1873 (+5.7) | 5.7334 (0) | 6.5576 (+14.4) | 4.6714 (0) | 4.8309 (+3.4) | 5.0658 (0) | 5.6769 (+12.1) |
| Delay CDF [s] | 5% Gain(%) | 0.1559 (0) | 0.144 (-7.6) | 0.1269 (0) | 0.1069 (-15.8) | 0.1562 (0) | 0.1443 (-7.6) | 0.128 (0) | 0.107 (-16.2) | 0.1564 (0) | 0.1452 (-7.2) | 0.1299 (0) | 0.1098 (-15.5) |
|  | 50% Gain(%) | 0.1643 (0) | 0.1511 (-8.03) | 0.132 (0) | 0.1128 (-14.5) | 0.1661 (0) | 0.1538 (-7.4) | 0.1397 (0) | 0.1209 (-5.5) | 0.1722 (0) | 0.1643 (-4.6) | 0.158 (0) | 0.1451 (-8.2) |
|  | 95% Gain(%) | 0.1925 (0) | 0.1967 (+2.2) | 0.1803 (0) | 0.1711 (-5.1) | 0.218 (0) | 0.2391 (+9.7) | 0.2292 (0) | 0.2197 (-4.1) | 0.2656 (0) | 0.305 (+14.8) | 0.3324 (0) | 0.3416 (+2.8) |
|  | Mean Gain(%) | 0.1677 (0) | 0.1573 (-6.2) | 0.1399 (0) | 0.123 (-12.1) | 0.1746 (0) | 0.1679 (-3.8) | 0.1556 (0) | 0.1395 (-10.3) | 0.1892 (0) | 0.1893 (+0.05) | 0.1906 (0) | 0.1776 (-6.8) |
| RU(%) |  | 24.3 | 25.3 | 28.1 | 30 | 42.6 | 49 | 51.3 | 52.5 | 65.5 | 71.2 | 75.7 | 78.2 |
| λ |  | 1 |  |  |  | 2 |  |  |  | 4 |  |  |  |

Table 7: File size=0.5Mbyte, UPT and latency gain of different RUs for new subframe type

| Reported Parameters |  | Low load |  |  |  | Medium load |  |  |  | High load |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7OS | Enhanced 7OS | 2OS | Enhanced 2OS | 7OS | Enhanced 7OS | 2OS | Enhanced 2OS | 7OS | Enhanced 7OS | 2OS | Enhanced 2OS |
| UPT CDF [Mbps] | 5% Gain(%) | 7.3624 (0) | 7.0403 (-4.4) | 6.1689 (0) | 6.3707 (+3.3) | 6.1219 (0) | 5.9283 (-3.2) | 5.1637 (0) | 4.903 (-5.1) | 5.0075 (0) | 4.0092 (-19.9) | 4.0992 (0) | 3.8225 (-6.8) |
|  | 50% Gain(%) | 12.8161 (0) | 13.5486 (+5.7) | 15.2424 (0) | 17.1841 (+12.7) | 12.4954 (0) | 12.8387 (+2.7) | 12.9008 (0) | 14.6503 (+13.6) | 11.5342 (0) | 11.6998 (+1.4) | 11.5781 (0) | 12.5073 (+8) |
|  | 95% Gain(%) | 13.683 (0) | 14.831 (+8.4) | 16.9325 (0) | 20.11 (+18.8) | 13.6735 (0) | 14.775 (+8.1) | 16.8127 (0) | 20.0442 (+19.2) | 13.6124 (0) | 14.6516 (+7.6) | 16.2753 (0) | 19.9025 (+22.3) |
|  | Mean Gain(%) | 12.1552 (0) | 12.6118 (+3.8) | 13.7741 (0) | 15.5882 (+13.2) | 11.703 (0) | 11.9836 (+2.4) | 12.445 (0) | 13.8772 (+11.5) | 10.86 (0) | 10.6747 (-1.7) | 11.0248 (0) | 12.5639 (+14) |
| Delay CDF [s] | 5% Gain(%) | 0.3065 (0) | 0.2825 (-7.8) | 0.2476 (0) | 0.2085 (-15.8) | 0.3067 (0) | 0.2833 (-8) | 0.2493 (0) | 0.2091 (-16.1) | 0.3079 (0) | 0.2839 (-7.8) | 0.2569 (0) | 0.2098 (-18.3) |
|  | 50% Gain(%) | 0.3271 (0) | 0.3114 (-4.8) | 0.28 (0) | 0.2462 (-12.1) | 0.3371 (0) | 0.3318 (-1.6) | 0.3361 (0) | 0.2996 (-10.9) | 0.3692 (0) | 0.3851 (+4.3) | 0.3855 (0) | 0.3469 (-10) |
|  | 95% Gain(%) | 0.5207 (0) | 0.574 (+10.3) | 0.6915 (0) | 0.6312 (-8.7) | 0.6598 (0) | 0.6709 (+1.7) | 0.8025 (0) | 0.815 (+1.6) | 0.808 (0) | 1.0732 (+32.8) | 1.0624 (0) | 1.1379 (+7.1) |
|  | Mean Gain(%) | 0.3616 (0) | 0.3575 (-1.1) | 0.3436 (0) | 0.3203 (-6.7) | 0.3884 (0) | 0.3953 (+1.8) | 0.4112 (0) | 0.3875 (-5.8) | 0.4467 (0) | 0.4965 (+11.1) | 0.5032 (0) | 0.4712 (-6.4) |
| RU(%) |  | 26 | 29 | 30.2 | 28.5 | 37.9 | 43.3 | 47 | 50 | 57.6 | 63.5 | 65 | 65 |
| λ |  | 1/5 |  |  |  | 1/3 |  |  |  | 1/2 |  |  |  |

Observation3: In case of 7 symbols TTI, there is small latency reduction gain of new subframe type compared to TTI shortening with existing UL/DL configuration 2.

Observation4: In case of 2 symbols TTI, there is more latency reduction gain of new subframe type compared to TTI shortening with existing UL/DL configuration 2.

# Conclusion

This contribution provides the system-level simulation results for TTI shortening of different schemes on TDD, together with the following observations:

Observation1: TTI shortening brings latency reduction gain in case of legacy TDD DL/UL configuration.

Observation2: Regardless the value of RU and file size, 2 symbols TTI provides the best performance in case of non-cell-edge UEs, followed by 7 symbols.

Observation3: In case of 7 symbols TTI, there is small latency reduction gain of new subframe type compared to TTI shortening with existing UL/DL configuration 2.

Observation4: In case of 2 symbols TTI, there is more latency reduction gain of new subframe type compared to TTI shortening with existing UL/DL configuration 2.

# References

- RP-150465, “SI: Study on Latency reduction techniques for LTE”.

- R1-164638, “Latency reduction for TDD” ZTE.

- 3GPP TR 36.912, Feasibility study for Further Advancements for E-UTRA (LTE-Advanced).

- R1-163919, “WF on TDD”, LG Electronics, Huawei, HiSilicon.

# Appendix1: L1 Overhead

Table 8 L1 Overhead

| TTI length | 14OS | 7OS | 2OS | Example 1 | Example 2 |
| --- | --- | --- | --- | --- | --- |
| Scheduled UE number per TTI | 2 |  |  |  |  |
| CRS ports | 2 |  |  |  |  |
| System bandwidth (MHz) | 10 |  |  |  |  |
| 2 CCE for each user (RE) | 72 |  |  |  |  |
| Number of PRBs | 50 |  |  |  |  |
| RE of GP overhead per PRB per radio frame | 12*2*50 | 12*2*50 | 12*2*50 | 12*4*50 | 12*6*50 |
| RE of PDCCH per radio frame | 12*8*50 | 12*8*50 | 12*8*50 | 12*1*50 | 12*1*50 |
| RE of sPDCCH overhead outside PDCCH region per radio frame | 0 | (6*1+1*2)*144 | (6*6+5*2)*144 | (1*1+2*2+2*1+2*3)*144 | (7+6+6*6)*144 |
| RE of CRS per radio frame | (12*6+8*2)*50 | (12*6+8*2)*50 | (12*6+8*2)*50 | (12+3*16+12*2+8*2)*50 | (12+16+12*6)*50 |
| Total overhead per radio frame | 10400 | 11552 | 17024 | 9872 | 16256 |
| Total RE for DL and GP per radio frame | 64800 | 64800 | 64800 | 64800 | 64800 |
| Overhead ratio | 16.05% | 17.83% | 26.27% | 15.23% | 25.09% |

# Appendix2: Simulation parameters

Table 9 General parameters

| Parameter | Assumptions |
| --- | --- |
| Layout | 7 macro eNB sites, 3 sectors per site |
| System bandwidth per carrier | 10MHz |
| Carrier frequency | 2GHz |
| Inter-site distance | 500m |
| Total BS TX power (Ptotal per carrier) | 46dBm |
| TTI length | 2/7 symbols |
| RS and control signaling overhead | 14OFDM symbols: 16.05% overhead 7OFDM symbols: 17.83% overhead 2OFDM symbols: 26.27% overhead 7OFDM symbols of new subframe: 15.23% overhead 2OFDM symbols of new subframe: 25.09% overhead |
| TBS determination | Scalable with TTI length as baseline |
| HARQ RTT | scaled with TTI length shortening |
| Scheduler | Proportional fairness |
| Distance-dependent path loss | ITU UMa[referring to Table B.1.2.1-1 in TR36.814], with 3D distance between an eNB and a UE |
| Penetration | For indoor UEs: 20dB+0.5din (din: independent uniform random value between [ 0, min(25,d) ] for each link) For outdoor UEs: 0 dB |
| Shadowing | ITU UMa |
| Antenna pattern | 3D |
| Antenna Height | 25m |
| UE antenna Height | 1.5m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | 2Tx(eNB) Cross-polarized 2Rx(UE), Cross-polarized |
| Number of UEs | 10 UEs per macro cell |
| UE dropping | Randomly and uniformly dropped throughout the macro geographical area 20% UEs are outdoor and 80% UEs are indoor |
| CSI report delay | 6TTIs |
| UE receiver | MMSE-IRC |
| eNB noise figure | 5dB |
| UL antenna configuration | 2Rx(eNB), 1Tx(UE) |
| UE noise figure | 9dB |
| UE speed | 3Km/h |
| Duplex mode | TDD |
| Network synchronization | Synchronized |
| Performance metrics | Mean, 5%, 50% and 95% user perceived throughput Mean, 5%, 50% and 95% user packet delay |