3GPP TSG RAN WG1 #85		R1-164679

May 23rd - 27th, 2016; Nanjing, China

Agenda item:	7.1.2

Source: 		Qualcomm Incorporated

Title: 			Highway and urban grid evaluation scenarios

Document for:	Discussion and Decision

# Introduction

With regard to the Rel-14 RAN study and Technical Report on Scenarios and Requirements for Next Generation Access Technologies [1], this paper provides some discussion and proposal on few open points related to attributes/parameters of the Highway and Urban grid scenarios, relevant for NR V2X evaluations.

# Discussion and Proposals

There were some initial proposals and discussions at the last RAN1 meeting ([2]) on the Highway scenario, but no final agreement.

The RAN TR ([1]) includes both Highway and Urban Grid scenarios, with some attributes still FFS (see Annex A for more details). A RAN email discussion is ongoing, with a certain expectation from RAN1 to suggest a way forward on some of those TBD values.

Taking the two references above as baseline, the following table proposes a list of evaluation parameters for both Highway and Urban Grid scenarios, for RAN1 discussion

| Parameters | Highway | Urban Grid | Notes |
| --- | --- | --- | --- |
| Layout | Option 1: Macro only Option 2: Macro + RSUs | Option 1: Macro only Option 2: Macro + RSUs |  |
| Inter-BS distance | Macro cell: ISD = 500m Inter-RSU distance = [100m] NOTE5 | Macro cell: ISD = 500m RSU at each intersection (Option 2) |  |
| Carrier frequency | Macro 6 GHz RSU: 6 GHz | Macro 6 GHz RSU: 6 GHz |  |
| Aggregated system bandwidth | Up to 100MHz (DL+UL) | Up to 100MHz (DL+UL) | Given the higher traffic (from 10 to 50 msg per sec), compared to LTE, it seems necessary to consder larger BWs (e.g. 5x larger) 80MHz may be considered (e.g. similar to 802.11) |
| Simulation bandwidth | 20 MHz | 20 MHz |  |
| Channel model | Follow TR36.885 | Follow TR36.885 |  |
| Tx power | RSU: FFS UE: 23 dBm | RSU: FFS UE: 23 dBm |  |
| BS antenna configuration | Up to 32 Tx /Rx antenna elements | Up to 32 Tx /Rx antenna elements |  |
| BS antenna pattern | FFS | FFS |  |
| BS antenna height | Macro: 25m RSU: FFS | Macro: 25m RSU: FFS |  |
| BS antenna element gain + connector loss | 8 dBi | 8 dBi |  |
| BS antenna downtilt | FFS | FFS |  |
| BS receiver noise figure | 5 dB | 5 dB |  |
| UE antenna elements | RSU Tx: Up to 32 Tx RSU Rx: Up to 32 Rx Vehicle Tx: Up to 8 Tx Vehicle Rx: Up to 8 Rx | RSU Tx: Up to 32 Tx RSU Rx: Up to 32 Rx Vehicle/Pedestrian Tx: Up to 8 Tx Vehicle/Pedestrian Rx: Up to 8 Rx | To be discussed |
| UE antenna height | Follow TR36.885: 1.5 m for vehicle UE, 5 m for UE type RSU | Follow TR36.885: 1.5 m for vehicle UE and pedestrian UE, 5 m for UE type RSU |  |
| UE antenna gain | Follow TR36.885: 3 dBi for vehicle UE and UE type RSU | Follow TR36.885: 3 dBi for vehicle UE and UE type RSU, 0 dBi for pedestrian UE |  |
| Traffic model | 50 messages per 1 second with absolute average speed of 100-250 km/h (relative speed: 200 – 500km/h) | 50 messages per 1 second at high speed (e.g. 60km/h or higher), 10 messages per 1 second at lower speeds (e.g. 15km/h) | SA1 should provide review/inputs, based on relevant use cases |
| Traffic load (Resource utilization) | FFS | FFS |  |
| UE distribution | 100% in vehicles Average inter-vehicle distance (between two vehicles’ center) in the same lane is 1sec * average vehicle speed (average speed: 100-250km/h]) | Urban grid model (car lanes and pedestrian/bicycle sidewalks are placed around a road block. 2 lanes in each direction, 4 lanes in total, 1 sidewalk, one block size: 433m x 250m) Average inter-vehicle distance (between two vehicles’ center) in the same lane is 1sec * average vehicle speed (average speed 15 – 120km/h) Pedestrian/bicycle dropping: average distance between UEs is 20m | SA1 should provide review/inputs, based on relevant use cases |
| UE receiver | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. |  |
| Feedback assumption | FFS | FFS | Not relevant for broadcast traffic. If SA1 determines unicast traffic should be considered, then feedback channel becomes relevant. |
| Channel estimation | Realistic | Realistic |  |

# Conclusions

It is proposed to discuss the above parameters for the evaluation of Highway and Urban grid scenarios.

# 5. References

[1] 3GPP TR 38.913 V0.3.0: SI TR on Scenarios and Requirements for Next Generation Access Technologies

[2] R1-163867, Updated Summary of evaluation assumptions for NR, CMCC

# Annex – From RAN TR

This section includes scenarios attributes currently defined in the RAN TR [1]. Attributes and parameters which are FFS (in square brackets) are highlighted in yellow, for convenience.

Note: a RAN email discussion (RAN#71-03) is ongoing, aiming at resolving those FFS.

========================== Text from TR 38.913 =================================

### Highway Scenario

The highway deployment scenario focuses on scenario of vehicles placed in highways with high speeds. The main KPIs evaluated under this scenario would be reliability/availability under high speeds/mobility (and thus frequent handover operations).

Some of its attributes are listed in Table 6.1.9-1.

[Editor’s notes: It is TBD whether eMBB requirements for eV2X would be evaluated under this scenario or another scenario. Examples of eMBB requirements for eV2X are video streaming and video calls]

[Editor’s notes: This scenario can be further updated to reflect practical highway scenarios.]

Table 6.1.9-1: Attributes of Highway

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro only: Below 6GHz (Around 6GHz) Macro + RSUs NOTE2: 1) For BS to RSU: Below 6 GHz (Around 6 GHz) NOTE3 2) RSU to vehicles or among vehicles: Below 6GHz |
| Aggregated system bandwidth NOTE4 | [TBD] MHz (DL+UL) |
| Layout | Option 1: Macro only Option 2: Macro + RSUs NOTE2 |
| ISD | Macro cell: ISD = 500m Inter-RSU distance = [100m] NOTE5 |
| BS antenna elements | Tx: Up to [32 Tx] Rx: Up to [32 Rx] |
| UE antenna elements | RSU Tx: Up to [32 Tx] RSU Rx: Up to [32 Rx] Vehicle Tx: Up to [8 Tx] Vehicle Rx: Up to [8 Rx] |
| User distribution and UE speed | 100% in vehicles Average inter-vehicle distance (between two vehicles’ center) in the same lane is [1sec * average vehicle speed] (average speed: [100-300km/h]) |
| Traffic model | [50 messages] per 1 second with absolute average speed of [100-250 km/h] (relative speed: 200 – 500km/h) |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 24 GHz – 40 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range.  A range of bands from 66 GHz – 86 GHz identified for WRC-19 are currently being considered and around 70 GHz is chosen as a proxy for this range.

NOTE2:	SA1 defines RSU as a logical entity that combines V2X application logic with the functionality of an eNB (referred to as eNB-type RSU) or UE (referred to as UE-type RSU). Therefore a RSU can communicate with vehicles via D2D link or cellular DL/UL

NOTE3:	This frequency may or may not be evaluated depending on communication type between eNB and RSU.

NOTE4:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE5:	If above 6 GHz is considered for communication between RSU and vehicles, inter-RSU distance can be [30 meter].

Illustrative diagram of freeway mode is as follows (from [3GPP TR 36.885])

Figure 6.1.9-1: Road configuration for highway scenario

### Urban Grid for Connected Car

The urban macro deployment scenario focuses on scenario of highly densely deployed vehicles placed in urban area. It could cover a scenario where freeways lead through an urban grid. The main KPI evaluated under this scenario are reliability/availability/latency in high network load and high UE density scenarios.

Some of its attributes are listed in Table 6.1.10-1.

[Editor’s notes: It is TBD whether eMBB requirements for eV2X would be evaluated under this scenario or another scenario. Examples of eMBB requirements for eV2X are video streaming and video calls]

Table 6.1.10-1: Attributes of urban grid for connected car

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro only: Below 6GHz (Around 6GHz) Macro + RSUs NOTE2: 1) For BS to RSU: Below 6GHz (Around 6GHz) NOTE3 2) RSU to vehicles or among vehicles/pedestrians: Below 6GHz |
| Aggregated system bandwidth NOTE4 | [TBD]MHz (DL+UL) |
| Layout | Option 1: Macro only Option 2: Macro + RSUs NOTE2 |
| ISD | Macro cell: ISD = 500m RSU at each intersection for Option 2 |
| BS antenna elements | Tx: Up to [32 Tx] Rx: Up to [32 Rx] |
| UE antenna elements | Vehicle Tx: Up to [8 Tx] Vehicle Rx: Up to [8 Rx] Pedestrian/bicycle Tx: Up to [8 Tx] Pedestrian/bicycle Rx: Up to [8 Rx] |
| User distribution and UE speed NOTE5 | Urban grid model (car lanes and pedestrian/bicycle sidewalks are placed around a road block. [2 lanes] in each direction, [4 lanes] in total, [1 sidewalk], one block size: [433m x 250m]) Average inter-vehicle distance (between two vehicles’ center) in the same lane is [1sec * average vehicle speed ] (average speed [15 – 120km/h]) Pedestrian/bicycle dropping: average distance between UEs is [20m] |
| Traffic model | [TBD] messages per 1 second with [120km/h], [50 messages] per 1 second with [60km/h], [10 messages] per 1 second with [15km/h] |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 24 GHz – 40 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range.  A range of bands from 66 GHz – 86 GHz identified for WRC-19 are currently being considered and around 70 GHz is chosen as a proxy for this range

NOTE2:	SA1 defines RSU as a logical entity that combines V2X application logic with the functionality of an eNB (referred to as eNB-type RSU) or UE (referred to as UE-type RSU). Therefore a RSU can communicate with vehicles via D2D link or cellular DL/UL

NOTE3:	This frequency may or may not be evaluated depending on communication type between eNB and RSU.

NOTE4:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE5:	More detail information can be found in [3GPP TR 36.885].

Illustrative diagram of urban grid model with UE distribution are the follows (from [3GPP TR 36.885]).

Table 6.1.10-2: Details of vehicle UE drop and mobility model

| Parameter | Urban case | Freeway case |
| --- | --- | --- |
| Number of lanes | 2 in each direction (4 lanes in total in each street) | 3 in each direction (6 lanes in total in the freeway) |
| Lane width | 3.5 m | 4 m |
| Road grid size by the distance between intersections | 433 m * 250 m. NOTE1 | N/A |
| Simulation area size | Minimum [1299 m * 750 m] | Freeway length >= 2000 m. Wrap around should be applied to the simulation area. |
| Vehicle density | Average inter-vehicle distance in the same lane is 2.5 sec * absolute vehicle speed. Baseline: The same density/speed in all the lanes in one simulation. |  |
| Absolute vehicle speed | 15 km/h, 60 km/h, 120 km/h | 250 km/h, 140 km/h, 70 km/h |

Figure 6.1.10-1: Road configuration for urban grid

NOTE1:	3 m is reserved for sidewalk per direction (i.e., no vehicle or building in this reserved space).