3GPP TSG RAN WG1 Meeting #85	R1-164727

Nanjing, China, 23rd – 27th May, 2016

Agenda Item:	7.2.3.3

Source:	Huawei, HiSilicon

Title:	Discussion on remaining details on large antenna array modeling

Document for:	Discussion

# Introduction

Accurate modeling of large antenna arrays should take several aspects into account [4], [5], [6], [7]:

- Wide bandwidth (modeling propagation delay over a large array)

- Spherical wave

- High angular resolution

- 3D correlation issue

- Non-stationarity

The aspect #1 was already taken into account in Busan [7], but the others are still open issues. This contribution discusses the spherical wave modeling for the current framework of spatial consistency.

For many modeling activities in large arrays we require the position of the first or last interaction point of clusters. In the last meeting the way forward document on the channel temporal evolution R1-163480 [1] proposes a method for time evolution of delay and angles similar to what was originally proposed in Wang2015 [2] for single bounce. We propose to use this method to find the first and last interaction points.

# Time Evolution Simulation of Way Forward (R1-163480)

In the following is the time evolution concept summarized of R1-163480 [1].

# Geometry Dependencies and Obtaining the Position of First and Last Interaction Points

Figure 1: Sketch for the random surface for the UE side.

In Figure 1 we consider for simplicity only a 2D case. We focus on the UE side, where we know the angle ϕn,AoA from a random drop. A random angle ϕRS,n is selected. According to the proposal in R1-163480 [1], those two angles define the incoming and outgoing angle for the assumed specular reflection. Together with the locations of UE and BS this defines essentially the location of the reflecting surface. Investigating all the different angles, we observe that we can actually obtain all the three angles for the triangle β, α3 and α4 and together with the distance d2D we utilize the sine law to calculate

d2D,LB,n=d2Dsin(2π-2ϕRS-ϕAoA,LoS-ϕAoA,n)sin⁡(2ϕAoA,n+2ϕRS-π) .		(1)

Thus using d2D,LB,n together with the angle ϕAoA,n we can calculate the position pLB,n of the last bounce scatterer. See the blue square in Figure 1. In a similar way this can be done for the BS side where ϕn,AoD and a new random angle ϕRS,n are known. The expression for the distance d2D,FB,n reads

d2D,FB,n=d2Dsin(2ϕRS-ϕAoD,LoS-ϕAoD,n)sin⁡(π+2ϕAoD,n-2ϕRS) .		(2)

Thus again the position pFB,n.of the first bounce scatterer can be obtained.

Note that there is actually as well another important position, which is the position of the mirrored base station BS’, see Figure 2. If we consider only a single bounce reflection this would actually be the point of origin of the wave that is observed at the UE. This can be done similarly at for the BS where a mirrored MS’ exists and a relevant moved mirrored MS’ for the respective chosen angle.

Figure 2: Sketch from the mobility model considering the again the UE side.

Note that the location can easily obtained from the proposed mobility model which provides the angular change SAoA,n, and the angle ε obtained via the moving direction ϕvand the angle of arrival ϕn,AoA and the complement to a triangle with 180 degrees θ=π-ε-SAoA,n. Utilizing the moved distance in the sine law we obtain

d'2D,AoA,n=v Δtsin(θ)sin⁡(SAoA,n) .		(3)

Using the distance d'2D,AoA,n and the angle ϕn,AoA the location of the mirror source BS’ is easily obtained. This can be obtained similarly for the BS side where again the angle change SAoD,na certain moved distance and the respective angle θ are utilized.

For simplicity only the 2D case was considered here. Additional work is required to extend the work to include elevation.

# Conclusion

Observation 1: There are five important aspects to be considered in the case of a large antenna array, namely wide bandwidth, spherical wave, high angular resolution, 	3D correlation issue, and non-stationarity. So far, only wide bandwidth issue was completed by 3GPP.

Observation 2: The proposed model for mobility allows obtaining relevant locations of first and last bounce scatterers or the origin of the wave considering single bounce specular reflection.

Observation 3: In case we assume the cluster in the 3GPP model corresponds to a scatterer with a fixed location. The scatterer location pLB,nand pFB,ncan be obtained with the respective distances ( (1) and (2)) and the angles  ϕAoA,n and ϕAoD,n..

Observation 4: It should be clarified which locations are the relevant ones for the large array support, and which ones could be used for spherical wave considerations.

Proposal 1: The following aspects on large array support should be discussed in the channel model SI: spherical wave, high angular resolution, 3D correlation issue, and non-stationarity.

Proposal 2: The spherical wave modeling introduced in this document should be taken into account in the channel model SI.

# References

- Intel, InterDigital, Huawei, HiSilicon, Qualcomm, “WF on channel temporal evolution”, R1-163480, Busan, Korea, April 11-15, 2016.

- Y. Wang, L. Huang, Z. Shi, K. Liu and X. Zou, “A millimeter wave channel model with variant angles under 3GPP SCM framework,” Personal, Indoor, and Mobile Radio Communications (PIMRC), 2015 IEEE 26th Annual International Symposium on, Hong Kong, 2015, pp. 2249-2254.

- NTT Docomo, NTT, “Discussion on Modeling of Spatial Consistency”, R1-161617, Lubeljana, Slovenia, March 14-16, 2016.

- Ericsson, Nokia, Qualcomm, KT, LGE, ETRI, Fraunhofer HHI, Intel, Interdigital, NTT, NTT Docomo, CATT, NEC, AT&T, Sony, Fujitsu, Sharp, Mitsubishi, National Instrument, IITH, CEWiT, Motorola Mobility, Lenovo, Huawei, HiSilicon, ALU, ZTE, Keysight Technologies, “WF on Channel Modeling Methodology”, St. Julian’s, Malta, February 15 – 19, 2016.

- Huawei, HiSilicon, CMCC, Fraunhofer HHI, Keysight, Telecom Italia, Vodafone, Orange, ZTE, Ericsson, AT&T, “Text proposal to TR 38.900 to add channel modeling requirements”, R1-161742, R1-161611, Ljubljana, Slovenia, March 14-16, 2016.

- Huawei, HiSilicon, “On the support for large antenna array in existing 3GPP 3D channel model”, R1-161611, Ljubljana, Slovenia, March 14-16, 2016.

- LG Electronics, ETRI, InterDigital, Keysight, Samsung, Ericsson, Huawei, “WF on propagation delay modeling over large antenna array”, R1-163487, Busan, South Korea, April 11-15, 2016.