3GPP TSG RAN WG1 Meeting #85 	R1- 164730

Nanjing, China, May 23-27, 2016

Agenda Item:	7.2.2

Source:	Huawei, HiSilicon, AT&T, Ericsson, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, Samsung

Title:	Cross-correlation on LSP for Indoor office

Document for:	Discussion and decision

# Introduction

In RAN1 #84bis meeting, the LSPs and SSPs for indoor office scenario have been agreed in [1]. During the email discussion after the meeting, one issue about the cross-correlation on the LSPs was found. The cross-correlation matrix for LOS case defined by the parameters in [1] is not positive definite which will bring difficulty for the implementation of the correlation matrix.

In this contribution, by introducing new results based on latest measurement campaign, the cross correlation results in [1] were updated. And the non-positive definite issue can be fixed.

# 2	Cross-correlation results based on latest measurement campaign

After Busan meeting, some measurement data as introduced in [2] were further processed to derive the cross-correlation on LSPs. Two more frequency bands were investigated, including 3.5GHz and 8GHz. The results can be found in Table 1

Table 1. Latest cross-correlation results on LSP

| Parameters |  | # 3.5GHz1 |  | # 20GHz2 | # 28GHz3 | # 73GHz4 | # 3.5GHz5 |  | # 8GHz5 |  | # 28GHz5 |  | # 73GHz5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | LOS | Hybrid | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| Cross Correlation | ASD vs DS | 3GPP | 3GPP | 0.26 | 0.481 | 0.47 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ASA vs DS | 3GPP | 3GPP | TBD | 0.5433 | 0.28 | 0.56 | 0.171 | 0.179 | 0.103 | 0.83 | -0.03 | 0.69 | -0.1 |
|  | ASA vs SF | TBD | TBD | TBD | 0.0454 | TBD | TBD | TBD | TBD | TBD | 0.07 | -0.55 | -0.41 | -0.13 |
|  | ASD vs SF | TBD | TBD | -0.68 | 0.3111 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | DS vs SF | TBD | TBD | -0.33 | 0.3318 | TBD | TBD | TBD | TBD | TBD | 0.44 | 0.35 | -0.24 | 0.6 |
|  | ASD vs ASA | 3GPP | 3GPP | TBD | 0.6236 | 0.15 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ASD vs K | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ASA vs K | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | DS vs K | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | SF vs K | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSD vs SF | 0.18 | 0 | 0.02 | 0.3749 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSA vs SF | 0.34 | 0 | TBD | 0.3313 | TBD | TBD | TBD | TBD | TBD | -0.52 | -0.01 | -0.68 | -0.27 |
|  | ZSD vs K | 0 | N/A | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSA vs K | 0.1 | N/A | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSD vs DS | 0 | -0.27 | 0.24 | 0.1752 | 0.07 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSA vs DS | 0 | -0.06 | TBD | 0.4963 | -0.01 | 0.28 | -0.01 | 0.13 | 0.06 | -0.2 | -0.12 | 0.41 | -0.24 |
|  | ZSD vs ASD | 0.65 | 0.35 | 0.1 | -0.3449 | 0.21 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSA vs ASD | 0 | 0.23 | TBD | 0.5116 | 0.1 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSD vs ASA | 0 | -0.08 | TBD | -0.0418 | 0.22 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSA vs ASA | 0.62 | 0.43 | TBD | 0.6429 | 0.06 | 0.42 | 0.01 | 0.01 | 0.4 | -0.03 | -0.28 | 0.61 | -0.37 |
|  | ZSD vs ZSA | 0.05 | 0.42 | TBD | 0.0975 | 0.44 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

- From BUPT based on measurement

- From DOCOMO based on measurement

- From KT based on measurement

- From Nokia/NYU based on ray-tracing

- From Huawei based on measurement

Based on the results in Table 1, the zenith domain related cross-correlation results in [1] can be updated as in Table 2. Some of the results in [1] were changed based on new data.

Table 2. Cross correlation on LSP for indoor office

| Scenarios |  | LOS | NLOS |
| --- | --- | --- | --- |
| Cross-Correlations | ASD vs DS | 0.6 | 0.4 |
|  | ASA vs DS | 0.8 | 0 |
|  | ASA vs SF | –0.5 | –0.4 |
|  | ASD vs SF | –0.4 | 0 |
|  | DS vs SF | –0.8 | –0.5 |
|  | ASD vs ASA | 0.4 | 0 |
|  | ASD vs K | 0 | N/A |
|  | ASA vs K | 0 | N/A |
|  | DS vs K | -0.5 | N/A |
|  | SF vs K | 0.5 | N/A |
|  | ZSD vs SF | 0.2 | 0 |
|  | ZSA vs SF | -0.1 | -0.1 |
|  | ZSD vs K | 0 | N/A |
|  | ZSA vs K | 0.1 | N/A |
|  | ZSD vs DS | 0.1 | -0.1 |
|  | ZSA vs DS | 0.2 | -0.1 |
|  | ZSD vs ASD | 0.2 | 0.3 |
|  | ZSA vs ASD | 0.2 | 0.2 |
|  | ZSD vs ASA | 0.1 | 0.1 |
|  | ZSA vs ASA | 0.3 | 0 |
|  | ZSD vs ZSA | 0.2 | 0.4 |

Alternative 1: the cross-correlation on LSP for indoor office in Table 2 will be adopted.

Considering the fact that the newly introduced results in zenith domain are lowly correlated, <0.4.	 Another approach for simplification can be treating all cross-correlation related to zenith domain as 0. Then we have the results in Table 3.

Table 3. Cross correlation on LSP for indoor office

| Scenarios |  | LOS | NLOS |
| --- | --- | --- | --- |
| Cross-Correlations | ASD vs DS | 0.6 | 0.4 |
|  | ASA vs DS | 0.8 | 0 |
|  | ASA vs SF | –0.5 | –0.4 |
|  | ASD vs SF | –0.4 | 0 |
|  | DS vs SF | –0.8 | –0.5 |
|  | ASD vs ASA | 0.4 | 0 |
|  | ASD vs K | 0 | N/A |
|  | ASA vs K | 0 | N/A |
|  | DS vs K | -0.5 | N/A |
|  | SF vs K | 0.5 | N/A |
|  | ZSD vs SF | 0 | 0 |
|  | ZSA vs SF | 0 | 0 |
|  | ZSD vs K | 0 | N/A |
|  | ZSA vs K | 0 | N/A |
|  | ZSD vs DS | 0 | 0 |
|  | ZSA vs DS | 0 | 0 |
|  | ZSD vs ASD | 0 | 0 |
|  | ZSA vs ASD | 0 | 0 |
|  | ZSD vs ASA | 0 | 0 |
|  | ZSA vs ASA | 0 | 0 |
|  | ZSD vs ZSA | 0 | 0 |

Alternative 2: the cross-correlation on LSP for indoor office in Table 3 will be adopted.

Proposal 1: Adopt Alternative 1 or Alternative 2 for cross-correlation on LSP for indoor office.

# 3	Conclusion

By introducing new results based on latest measurement campaign, the cross correlation results in [1] were updated. Two different alternatives were provided to resolve the non-positive definite issue. Based on the results, the following is proposed:

Alternative 1: the cross-correlation on LSP for indoor office in Table 2 will be adopted.

Alternative 2: the cross-correlation on LSP for indoor office in Table 3 will be adopted.

Proposal 1: Adopt Alternative 1 or Alternative 2 for cross-correlation on LSP for indoor office.

# References

R1-163472, “Large-scale and small-scale channel modeling for indoor hotspot”, Huawei, HiSilicon, AT&T, CMCC, Ericsson, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, Samsung, ETRI, CATT, Busan, April. 11-15, 2016.

R1-161608, “Frequency dependency for large scale parameters”, Huawei, HiSilicon, Ljubljana, March. 14-16, 2016.