3GPP TSG RAN WG1 Meeting #85	R1-164736

# Nanjing, China, 23rd - 27th May 2016

Agenda item:	6.1.4

Source: 	Samsung

Title: 	Clarifications on non-PMI based CSI reporting in TS 36.213

Document for:	Discussion and Decision

Introduction

In RAN1#83 meetings, agreements on non-PMI based Class B CSI reporting had been drawn [1]:

Agreements: - When K=>1 and Nk = {2,4,8}, RI, CQI and CRI (only if K>1) feedback is supported in non-PMI feedback modes PUSCH 2-0 & 3-0 and PUCCH 1-0 & 2-0. - In principle, CQI, PMI and RI derivation is the same as for PMI based feedback modes. - PMI is not reported - Note: PMI can be restricted to a single precoder configured by CBSR

However, the corresponding descriptions in the current specification may cause a misunderstanding especially for the highlighted sentences in the above agreements [2]. This contribution discusses on the potential issues in the current aperiodic and periodic CSI reporting mode descriptions and provides possible alternatives to capture the RAN1 agreements correctly.

Configuration of CBSR for non-PMI feedback mode

As highlighted by yellow in Section 1, although it was agreed in principle that CQI, PMI, and RI derivation for non-PMI based feedback mode is the same as for PMI based feedback modes, there is no clear agreement to define which one is the baseline among single or multiple PMI type. However, the current specification forces eNB and UE to assume a single PMI for non-PMI feedback mode. The corresponding descriptions in the current specification are highlighted by yellow in Table 1 and Table 2. Since there are no RAN1 agreement that eNB shall restrict a single precoder via CBSR for non-PMI reporting mode for ‘CLASS B’ eMIMO-Type, corresponding specification should be revised more generally at least for PUSCH mode 3-0 and PUCCH mode 2-0, which support subband CQI reporting. Consequently, we have the following two options for UE assumptions on precoding matrix with PUSCH mode 3-0 and PUCCH mode 2-0 when ‘CLASS B’ eMIMO-Type is configured:

Observation: Two options for UE assumption on precoding matrix for PUSCH mode 3-0 and PUCCH mode 2-0 are available.

- Alternative 1: Remove all assumptions on precoding matrix in non-PMI based CSI reporting mode descriptions.

- Alternative 2: Release the assumption that ‘a single precoding matrix is selected from the CBSR’ and clarify that ‘for each subband a preferred precoding matrix is selected from the codebook subset assuming transmission only in the subband’ in non-PMI based CSI reporting mode descriptions.

Note that based on the latter statement, ‘for each subband a preferred precoding matrix is selected from the codebook subset assuming transmission only in the subband’, the same operation as the former statement, ‘a single precoding matrix is selected from the CBSR’, can be done by eNB implementation.

Proposal: One of alternatives in Observation should be down-selected to revise the corresponding descriptions in the current specification.

Our companion contribution provides the detailed revision based on the second alternative [3].

Conclusions

In this contribution, we discussed clarification on non-PMI based CSI reporting. Based on the discussion, the following observations and proposals are drawn:

Observation: Two options for UE assumption on precoding matrix for PUSCH mode 3-0/PUCCH mode 2-0 are available.

- Alternative 1: Remove all assumptions on precoding matrix in non-PMI based CSI reporting mode descriptions.

- Alternative 2: Release the assumption that ‘a single precoding matrix is selected from the CBSR’ and clarify that ‘for each subband a preferred precoding matrix is selected from the codebook subset assuming transmission only in the subband’ in non-PMI based CSI reporting mode descriptions.

Proposal: Support both options in observation and revise the corresponding descriptions in the current specification.

References

RP-151845,	“Status report for WI Elevation Beamforming/Full-Dimension (FD) MIMO for LTE; rapporteur: Samsung,”	RAN1

3GPP TS 36.213 V13.1.1: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

R1-162654, “Draft CR on non-PMI based CSI reporting in TS 36.213,”		Samsung

Appendix

Table 1. PUSCH mode 3-0 description in [2].

- Mode 3-0 description: - If a UE is configured in transmission mode 9 or 10, and UE is configured with higher layer parameter eMIMO-Type for a CSI process, and eMIMO-Type is set to ‘CLASS B’, and the number of CSI-RS antenna ports in at least one of the one or more configured CSI-RS resource is more than one, - If the number of configured CSI-RS resources is more than one, the UE shall report one wideband CRI which is calculated assuming transmission on set S subbands. - A single precoding matrix is selected from the codebook subset assuming transmission on set S subbands - A UE shall report one subband CQI value per codeword for each set S subband which are calculated assuming the use of the single precoding matrix in all subbands and assuming transmission in the corresponding subband. - A UE shall report a wideband CQI value per codeword which is calculated assuming the use of the single precoding matrix in all subbands and transmission on set S subbands - The selected precoding matrix, and reported CQI values are calculated conditioned on the reported RI. If CRI is reported, the selected precoding matrix, reported CQI, and RI values are calculated conditioned on the reported CRI otherwise, - …

Table 2. PUCCH mode 2-0 description in [2].

- Mode 2-0 description: - …. - In the subframe where CQI for the selected subbands is reported: - If the UE is configured without PMI reporting (only for transmission mode 9 or 10): - The UE shall select the preferred subband within the set of Nj subbands in each of the J bandwidth parts where J is given in Table 7.2.2-2. - A single precoding matrix is selected from the codebook subset assuming transmission on on the selected subband within the applicable bandwidth part. - The UE shall report a type 1 report per bandwidth part on each respective successive reporting opportunity consisting of: - CQI value for codeword 0 reflecting transmission only over the selected subband of a bandwidth part determined in the previous step along with the corresponding preferred subband L-bit label. - When RI>1, an additional 3-bit subband spatial differential CQI value for codeword 1 offset level - Codeword 1 offset level = subband CQI index for codeword 0 – subband CQI index for codeword 1. - The mapping from the 3-bit subband spatial differential CQI value to the offset level is shown in Table 7.2-2. - If the UE is configured with CRI reporting, - …