3GPP TSG RAN WG1 #85		R1-164803

Nanjing, China 23rd - 27th May 2016

Agenda item:	7.2.5.2

Source:	Samsung, Nokia

Title:	E-mail discussion summary of the full scale calibration

Document for:	Discussion & Decision

# Introduction

From the e-mail discussion of [84b-17] for full scale channel modeling calibration, the following has been agreed.

Agreement:

- The penetration loss (between High vs. low loss) is UE specifically generated.

- O-I penetration loss deviation generation:

  - Change the standard deviation to 4.4 and 6.5 for low loss and high loss penetration model separately

  - FFS: O-I deviation is UE specific random variable.

- ZoD offset for indoor office LOS and NLOS shall be set to 0

- Missing parameters of UMi/UMa O2I ZoD spread and ZoD offset: reuse these parameters taken from UMi O2O and UMa O2O, respectively

- Continue discussion on whether to take UE-specific or link-specific or site-specific din in Nanjing

- Continue discussion on the two alternatives to resolve the positive definite issues for the cross-correlation parameters

# Conclusion

It is proposed to discuss further 2b, 5 and 6 in this RAN1 meeting according to the agreement.

# Appendix

A discussion summary used for drawing the above agreement can be found below:

- Company’s implementation of the O-I parameters:

  - High vs. low loss – all 17 companies (UE specific)

  - din – 12 companies (UE specific); 5 companies (link specific)

  - O-I penetration loss deviation: 3 companies (UE specific); 5 companies (link-specific); 9 (link-specific, with replacing the shadow fading term)

- Company’s views on the O-I parameters:

  - din:

    - Alt 1: UE specific – supported by Nokia, Ericsson, Samsung;

    - Alt 2: link-specific – supported by LGE, Straightpath

  - O-I penetration loss deviation … the proposal by Nokia was supported by Ericsson and Samsung

    - Change the standard deviation to 4.4 and 6.5 for low loss and high loss penetration model separately (see below table from R1-163256)

    - O-I deviation is UE specific random variable. … This option was disputed by LGE.

- ZoD offset for indoor office LOS and NLOS shall be set to 0 … proposed by Huawei and supported by Xianwei;

- Missing parameters of UMi/UMa O2I ZoD spread and ZoD offset: reuse these parameters taken from UMi O2O and UMa O2O, respectively … proposed by Samsung, supported by Xinwei

- Solutions to resolve the positive definite issue:

  - Alt 1: A table from Ericsson … supported by Samsung and Nokia

  - Alt 2: A table from Xinwei

Alt 1: revised cross-correlation parameters

| UMi / UMa O2I |  |  |
| --- | --- | --- |
| Cross-Correlations | ASD vs DS | 0.4 |
|  | ASA vs DS | 0 |
|  | ASA vs SF | 0.53 |
|  | ASD vs SF | 0 |
|  | DS vs SF | -0.5 |
|  | ASD vs ASA | 0 |
|  | ASD vs  | N/A |
|  | ASA vs  | N/A |
|  | DS vs  | N/A |
|  | SF vs  | N/A |
| Cross-Correlations 1) | ZSD vs SF | 0 |
|  | ZSA vs SF | 0.4 |
|  | ZSD vs K | N/A |
|  | ZSA vs K | N/A |
|  | ZSD vs DS | 0 |
|  | ZSA vs DS | -0.53 |
|  | ZSD vs ASD | 0 |
|  | ZSA vs ASD | 0.42 |
|  | ZSD vs ASA | 0 |
|  | ZSA vs ASA | 0 |
|  | ZSD vs ZSA | 0 |

Alt 2: Revised cross-correlation parameters from Xinwei

| UMi / UMa O2I |  |  |
| --- | --- | --- |
| Cross-Correlations | ASD vs DS | 0.4 |
|  | ASA vs DS | 0 |
|  | ASA vs SF | 0.53 |
|  | ASD vs SF | 0.2 |
|  | DS vs SF | -0.5 |
|  | ASD vs ASA | 0 |
|  | ASD vs  | N/A |
|  | ASA vs  | N/A |
|  | DS vs  | N/A |
|  | SF vs  | N/A |
| Cross-Correlations 1) | ZSD vs SF | -0.15 |
|  | ZSA vs SF | 0.4 |
|  | ZSD vs K | N/A |
|  | ZSA vs K | N/A |
|  | ZSD vs DS | 0.01 |
|  | ZSA vs DS | -0.53 |
|  | ZSD vs ASD | 0 |
|  | ZSA vs ASD | 0.42 |
|  | ZSD vs ASA | -0.21 |
|  | ZSA vs ASA | 0 |
|  | ZSD vs ZSA | 0.38 |