3GPP TSG RAN WG1 Meeting #85	R1-164810

Nanjing, China, May 23-27, 2016

Agenda Item:	7.2.5.1

Source:	Samsung

Title:	Results of large-scale channel model calibration

Document for:	Discussion

Introduction

The new channel model above 6GHz is a necessary to enable 5G performance evaluation, thus it is very important to verify the channel model’s capability and align its implementation. According to the previous channel modeling study item [1], the calibration is an efficient method to examine companies’ understanding and implementation of the developed new channel models above 6GHz.

At the RAN1 #84bis meeting, several issues on channel modeling calibration have been got solved especially in terms of scenarios, parameters and calibration methods in [2]. We provided the detailed layout and parameters of calibration scenarios in [3], and in [4] and [5] we provided the view on channel model calibration assumptions and metrics.

In this contribution, we provide the initial large-scale channel model calibration results for different scenarios each frequency, which can be taken into account for comparison.

Initial calibration results for large-scale channel model

6GHz

Figure 1 Distribution of coupling loss for different scenarios (6 GHz).

Figure 2 Distribution of geometry with noise for different scenarios (6 GHz).

Figure 3 Distribution of geometry without noise for different scenarios (6 GHz).

30GHz

Figure 4 Distribution of coupling loss for different scenarios (30 GHz).

Figure 5 Distribution of geometry with noise for different scenarios (30 GHz).

Figure 6 Distribution of geometry without noise for different scenarios (30 GHz).

70GHz

Figure 7 Distribution of coupling loss for different scenarios (70 GHz).

Figure 8 Distribution of geometry with noise for different scenarios (70 GHz).

Figure 9 Distribution of geometry without noise for different scenarios (70 GHz).

Detail clarifications of O2I modelling

In this paper, we discussed the initial calibration results of phase 1 for variety scenarios above 6GHz frequency. For accurate initial calibration results of phase 1, it is necessary to ensure the assumption. Especially, detail clarifications of O2I modelling. In phase 1, the path loss of O2I model described in

Path loss= PLb+PLtw+PLin

PLb is the path loss through outdoor distance including shadow fading term. PLtw is the path loss through external wall, i.e., penetration loss. PLin is the indoor propagation loss. σP is the standard deviation of penetration loss, and the penetration is realized added on top of shadow fading term in PLb, which is realized as UE-specific. PLin is determined by din, then it means indoor-distance is modelled as UE specific, too.

Conclusions

In this paper, we discussed the initial calibration results of phase 1, large-scale channel model for UMi-SC, UMa, and InH scenarios above center frequency of 6GHz/30GHz/60GHz/70GHz, and these results should be taken into collections on the calibration comparison in this SI.

# References

- 3GPP, TR 36.873 V12.2.0, “Study on 3D Channel Model for LTE (Release 12)”, Jun. 2015.

- 3GPP, TR 38.900 V 0.2.0, “Technical Specification Group Radio Access Network; Channel model for frequency spectrum above 6 GHz”.

- NTT DOCOMO, “Scenarios for channel model verification and calibration”, R1-161618, Ljubljana, Slovenia, Mar. 14-16, 2016.

- Nokia, AT&T, CMCC, Ericsson, ETRI, Huawei, HiSilicon, Intel, KT Corporation, NTT DOCOMO, Qualcomm, Samsung, “Scenarios and simulation assumption for calibration”, R1-163477, Busan, Korea, Apr. 11-15, 2016.

- Nokia, Samsung, “WF on simulation assumption for large-scale calibration”, R1-163641 , Busan, Korea, Apr. 11-15, 2016.

# APPENDIX

Table 1 Parameters of calibration scenarios.

|  |  | Indoor hotspot | Urban micro | Urban Macro |
| --- | --- | --- | --- | --- |
| Layout |  | Indoor open office | Hexagonal grid, 19 micro sites,3 sectors per site* | Hexagonal grid, 19 macro sites,3 sectors per site* |
| UE mobility (movement in horizontal plane) |  | 3km/h | 3km/h | 3km/h |
| BS antenna height |  | 3m | 25m | 25m |
| Total BS Tx Power |  | [24] dBm per band ** | [41/44] dBm per band ** | [46/49] dBm per band ** |
| Carrier frequency |  | < 100 GHz | < 100 GHz | < 100 GHz |
| Bandwidth |  | ≤ 10% carrier frequency | ≤ 10% carrier frequency | ≤ 10% carrier frequency |
| Min. UE-eNB 2D distance |  | 0m | 10m | 35m |
| UE height (hUT) in meters | general equation | 1.0 | hUT=3(nfl – 1) + 1.5 | hUT=3(nfl – 1) + 1.5 |
|  | nfl for outdoor UEs | 1 | 1 | 1 |
|  | nfl for indoor UEs | 1 | nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) | nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| Indoor UE fraction |  | 100% | 80% | 80% |
| UE distribution (in x-y plane) | Outdoor UEs | N/A | uniform in cell | uniform in cell |
|  | Indoor UEs | uniform in cell | uniform in cell | uniform in cell |
| ISD |  | 20m | 200m | 500m |

*Note1: other sector models, e.g., 6 sectors per site are FFS.

**Note2: the tx power is highly related to the carrier frequency, bandwidth and sector number, current assumption is base on TR 36.872/ 36.873 and only for calibration.

Table 2 Simulation assumptions for phase 1 calibration.

| Parameter | Values |
| --- | --- |
| Scenarios | Indoor hospot (InH), Urban micro (UMi), Urban macro (UMa) |
| Antenna configurations | K=1, M=1 |
| Downtilt | InH: 120 degrees electrical tilt UMi/UMa: 102 degrees electrical tilt |
| Handover margin (for calibration) | 0dB |
| UE attachment | Based on pathloss considering LOS angle |
| Fast fading channel | Fast fading channel is not modelled |
| Wrapping method | Geographical distance based (except for InH) |
| Metrics | 1) Coupling loss – serving cell (based on LOS pathloss) |
|  | 2) Geometry (based on LOS pathloss) |