3GPP TSG RAN WG1 #85	R1-164813

# Nanjing, China, 23th – 27th May 2016

Agenda item:	7.1.5.1

Source: 	Samsung

Title: 	Design of LDPC code for high throughput

Document for:	Information and Discussion

Introduction

In the RAN1 #84bis meeting, several channel coding schemes were proposed for NR system and those can be classified as: LDPC code, polar code, convolutional code (LTE and/or enhanced convolutional codding) and turbo code (LTE and/or enhanced turbo coding).

In this contribution, we introduce quasi-cyclic (QC) LDPC codes with layered structure and address that they are suitable for multi-Gbps applications since the layered structure makes it easy to optimize decoding parallelization.

Quasi-Cyclic LDPC code

A QC LDPC code is characterized by the parity-check matrix which consists of small square blocks which are the zero matrix or circulant permutation (right-shifted identity) matrices. Let P=pij (0≤i,j<Z) be the Z×Z permutation matrix defined by

pij=1if i+1≡j mod Z0otherwise      or    P=001001⋯00⋮0⋮0⋮0⋯⋮11000.

Note that Pi is just the circulant permutation matrix which shifts the identity matrix I(=P0) to the right by i times for any integer i, 0≤i<Z. For simple notation, we denote the Z×Z zero matrix O by P-1.

Let H be the mZ×nZ matrix given by

H=Pa11Pa21Pa12Pa22⋯⋯Pa1nPa2n⋮Pam1⋮Pam2⋱⋯⋮Pamn

where aij∈{-1, 0, 1, 2, …, Z-1} are exponent indices of permutation matrices, n and m are the numbers of column and row blocks, respectively. Then the code with parity-check matrix H is referred to as a quasi-cyclic (QC) LDPC code. Furthermore, let E(H) be the expoment matrix of H given by

EH=a11a21a12a22⋯⋯a1na2n⋮am1⋮am2⋱⋯⋮amn.

One advantage of QC LDPC codes is length compatibility. QC LDPC codes of variable length can be easily obtained by adjusting the size of circulant permutation matrices in H. An example of a parity-check matrix for a (8Z, 4Z) QC LDPC code with n=8 and m=4 is given by

H=P0OOP1OP0P4OOP2P3OOP6P5OP1P0OP0OOP0OP0OP1OP0P0OP0,   E(H)=0-1-11-104-1-123-1-165-110-10-1-10-10-11-100-10

where O is the Z×Z zero matrix and P0 is the Z×Z identity matrix.

Layered architecture for high data throughput

In [1], Samsung proposes QC LDPC code whose parity-check matrix has a conceptual structure depicted in Figure 1. As shown in Figure 1, the proposed code can be regarded as a concatenated QC LDPC code of IEEE 802.11n-like code and many single parity-check codes.

An important characteristic of the proposed code is the row-orthogonal property (so-called layered structure). The row-orthogonal property means that each column block belonging to two or more row blocks has at most weight-1, i.e., there is no overlapped circulant permutation matrix among the row blocks, as shown in Figure 1 [3], [4]. The row-orthogonal property is first adopted in IEEE 802.11ad [3] to support very high decoder throughput. Since multiple row blocks without overlapped circulants can be regarded as one row block, the row-orthogonal property makes a highly-parallelized implementation possible. In other words, QC LDPC codes with layered structure are suitable for multi-Gbps applications.

Figure 1. Structure of proposed QC LDPC code for high data throughput

In Figure 1, 802.11n-like code (Part-1 submatrix) has a small number of layers with few Z×Z zero matrices, but Part-2 submatrix has more and sparser layers. If the decoder processes a matrix layer by layer, Part-1 submatrix can be processed in fewer sub-iterations with less hardware wasted on Z×Z zero matrices compared to Part-2 submatrix. However, Part-2 submatrix has a special structural feature; they have groups of non-overlapping row blocks. Figure 2 shows an example for parity-check matrix with 3 row blocks and 2 layers. The non-overlapping row blocks can be effectively combined together to form a compressed matrix, and each compressed layer can be processed as one.

Figure 2. Example of compressing matrices with 2 layers

Figure 3 shows conceptual operations for layered decoding based on min-sum algorithm. Consider a decoder that has the variable nodes fully parallelized and the check nodes fully layer-serialized. To process the first row block (1st layer), the variable nodes send their messages to the corresponding input on the check node. To process the second and third row blocks (2nd layer) simultaneously, the variable nodes connected to the second row block route their inputs to the top of the compare-select (CS) tree, and the variable nodes connected to the third row block route their inputs to the bottom of the tree. Figure 3 shows this process for one of the check nodes with the first row block (1st layer) being processed at the top-right of the figure and the second and third row blocks (2nd layer) being processed at the bottom-right. The variable nodes in the figure represent the one variable node out of the variable node group that connects to the check node. Consequently, Figure 3 shows that layer-structured QC LDPC codes make it possible to implement highly-parallelized, fully-pipelined LDPC decoder by appropriately combining the non-overlapping row blocks.

Figure 3. Example implementation of check node granularity

Observation: Layer-structured QC LDPC codes are suitable for multi-Gbps applications since the layer-structure makes it possible to implement highly-parallelized, fully-pipelined decoder.

Conclusion

Layer-structured QC LDPC codes are suitable for multi-Gbps applications since the layer-structure makes it possible to implement highly-parallelized, fully-pipelined decoder.

- References

- R1-164812, Samsung, "Preliminary evaluation results on new channel coding scheme for NR - LDPC code for high throughput," 3GPP TSG RAN WG1 #85, Nanjing, China, May 23-25, 2016.

- IEEE Std 802.11n™-2012, "Part 11: Wireless LAN medium access control (MAC) and physical layer (PHY) specifications."

- IEEE Std 802.11ad™-2012, "Part 11: Wireless LAN medium access control (MAC) and physical layer (PHY) specifications. Amendment 3: Enhancements for very high throughput in the 60 GHz band."

- A. Blanksby, B. Shen, and J. Trachewsky, "LDPC code set for mmWave communication," in Proc. Int’l. Wksp. mmWave Commun. From Circuits to Networks (mmCom 2010), Chicago, IL, Sept. 2010.

- M. Weiner and B. Nikolic, "A high-throughput, flexible LDPC decoder for multi-Gb/s wireless personal area networks," Technical Report No. UCB/EECS-2010-177, Electrical Engineering and Computer Sciences University of California at Berkeley, Dec. 22, 2010