3GPP TSG RAN WG1 Meeting #85		                                                R1-164838

Nanjing, China, May 23-27, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | XXXX | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on collision between SR and PDSCH with repetitions in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-05-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The subframes for PDSCH repetitions reception may partially or fully collide with the subframes for SR transmission for eMTC. For half-duplex FDD operation, a UE is not able to simultaneously transmit SR and receive PDSCH. The corresponding UE behaviour has not been specified. The interruption of PDSCH reception to the transmission of SR over PUCCH may degrade the PDSCH reception performance, as the UE needs stop the reception for at least 3 subframes due to the UL/DL retuning and UL transmission. Although PDSCH can be retransmitted, it is not efficient resource utilization. As the typical service for CE UEs is delay tolerant, the UE could drop SR transmission for PDSCH reception to save the DL resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In case of collision between SR transmission and PDSCH with repetitions, the SR transmission is dropped for a half-duplex UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undefined behavior in case of collision between SR transmission and PDSCH with repetitions for a half-duplex UE. The PDSCH reception performance may be degraded. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 10.1.1	PUCCH format information

#### < Unchanged parts are omitted >

For a BL/CE UE in half-duplex FDD operation, in case of collision between a PUCCH format 1/2 transmission including half-duplex guard subframe and a PDSCH reception with repetitions, the PUCCH format 1/2 transmission is dropped.

For a BL/CE UE, in case of collision between at least one physical resource block to be used for transmission of UCI on PUCCH (defined in [3]) and physical resource blocks corresponding to configured PRACH resources for BL/CE UEs or non-BL/CE UEs (defined in [3]) in a same subframe, the UCI is dropped.

#### < Unchanged parts are omitted >