3GPP TSG RAN WG1 Meeting #85		                                                R1-164840

Nanjing, China, May 23-27, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | XXXX | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on collision between PUCCH and PUSCH in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-05-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The agreement in RAN1 #84 for the collision handling between PUCCH and PUSCH says: - A UE drops PUSCH transmissions in the subframes that collide with the PUCCH (for HARQ-ACK or SR or CQI) transmitted by the UE. However the sentence in the specification describes the case as the collision between a UCI and a PUSCH transmission, which will cause misunderstanding that PUSCH can not carry any UCI for BL/CE UEs. This is not the actual meaning of the agreement. PUSCH can carry at least CSI report because aperiodic CSI report is supported for CEmodeA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the case description for the collision between a UCI and a PUSCH transmission to the collision between a UCI on PUCCH and a PUSCH transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will cause misunderstanding which is not the meaning of the agreement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 10.1.1	PUCCH format information

#### < Unchanged parts are omitted >

For a BL/CE UE, in case of collision between a UCI on PUCCH and a PUSCH transmission in a same subframe, the PUSCH transmission is dropped.

For a BL/CE UE in half-duplex FDD operation, in case of collision between a PUCCH format 2 transmission including half-duplex guard subframe and a PDSCH reception with repetitions, the PUCCH format 2 transmission is dropped.

For a BL/CE UE, in case of collision between at least one physical resource block to be used for transmission of UCI on PUCCH (defined in [3]) and physical resource blocks corresponding to configured PRACH resources for BL/CE UEs or non-BL/CE UEs (defined in [3]) in a same subframe, the UCI is dropped.

#### < Unchanged parts are omitted >