3GPP TSG RAN WG1 Meeting #85	R1-164867

Nanjing, China, May 23-27, 2016

Agenda Item:	6.2.10.1

Source:	Huawei, HiSilicon

Title:	CQI feedback overhead reduction in short TTI

Document for:	Discussion and decision

# Introduction

In the RAN#67 meeting, the Study Item on “Study on Latency reduction techniques for LTE” was approved [1]. The objective of this study item is:

Study enhancements to the E-UTRAN radio system in order to:

  - Significantly reduce the packet data latency over the LTE Uu air interface for an active UE

  - Significantly reduce the packet data transport round trip latency for UEs that have been inactive for a longer period (in connected state).

It is observed in [2] that UPT gain of both short CSI measurement interval and short CSI reporting period is significant and larger than that of short CSI measurement interval and long CSI reporting period. While in [3], four sources evaluated the usage of shortened TTI for obtaining faster update of CSI measurements. Therefore it is reasonable to consider that the CQI report overhead for short TTI transmission may be higher than legacy TTI transmission if 5 short TTI CSI reporting period is applied. As such, a new CQI report schemes to reduced overhead may be considered. In this contribution, considering that the dynamic range of the wireless channel state may not be so large within multiple CSI reporting intervals in case of short TTI，we will discuss a two-tier CQI report scheme which can reduce CQI reporting overhead while maintaining similar throughput performance.

# Discussion on Two-tier CQI report Scheme

## Motivation

In [4], table 7.2.3.1 and 7.2.3.2 show that the CQI index is reported by 4 bits with the efficiency from 0 to 5.55 or 7.406. There are 16 CQI index in total. However, the CQI of the channels may not fluctuate among the whole CQI table in a short reporting period. By contrast, it may fluctuate no more than several CQI indices, e.g. 8 CQI indices, which provide potential possibility to reduce the length of the CQI index feedback.

## The principle of two-tier CQI report

As we know, the legacy CQI index needs to cover a wide efficiency range. However, in a short period of time, the channel quality fluctuates in a much less range. Instead of reporting absolution CQI value each time, the CQI can be reported in two tiers. In the first tier, the UE reports an average CQI, which is the average value of a number of CQIs reported before the report time. This average CQI only needs to be reported in a very low frequency. And then in the second tier, the UE measures the channel quality and reports the difference between the instantaneous CQI and the average CQI. Mathematically, it may be represented as:

,

Where  represents the average CQI reported in the first tier while represents the difference between the instantaneous CQI and the long-term average.

The, which represents the average receive SNR can use Table 7.2.3.1 and 7.2.3.2 in [4]. While, in the second tier, the  representing difference between the UE measured instantaneous CQI and the  can use Table 7.2-2 in [4].

Figure 1(a): The One Tier CQI report scheme.

Figure 1(b): The Two Tier CQI report scheme.

Note that the 4 bit absolute CQI report scheme as shown in Fig. 1(a) is termed as One Tier CQI report scheme.

Table 1: The table 7.2-2 in [4]. Mapping differential value to offset level

| Differential CQI value | Offset level |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 4 | -4 |
| 5 | -3 |
| 6 | -2 |
| 7 | -1 |

It was agreed in [5] that multi-stream should be supported in short TTI design. In the case of multi-stream, we do not change the current report scheme of the n th stream, where n>1.

## Performance Results

In the current system, the CQI report period may be 5TTI. Therefore we set the simulation parameter CQI report period as 5TTI both for One Tier scheme and for the second tier in Two Tier scheme. In the proposed method, the control overhead is 4bit/200TTI for the first tier and 3bit/5TTI for the second tier, while the one tier scheme control overhead is 4bit/5TTI. Thus, the CQI overhead of Two Tier scheme is 77.5% of the One Tier scheme.

Figure 2: The overhead of the two schemes.

Observation 1: The two-tier CQI report scheme is capable of reducing 22.5% control overhead compared with one-tier CQI report scheme in sTTI.

Then we are wondering whether the new scheme deteriorates the throughput performances. Based on the current standard and two tier scheme, the simulation parameters [5] in table 1 of appendix and results are shown as below. The number of antenna is 2X2 and we only consider one stream. We also considered the UE speed from 3Kmh to 120Kmh and considered the sTTI=2, 3(4) and 7 symbols. It is observed that the performances of Two Tier scheme of both the cell frequency efficiency and edge 5% UE frequency efficiency are as similar as the performances of One Tier scheme. Note that the simulation is based on full buffer traffic model.

|  | TTI=2symbol |  |  | TTI=3(4)symbol |  |  | TTI=7symbol |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Speed/Kmh | Two Tier | Legacy | Difference | Two Tier | Legacy | Difference | Two Tier | Legacy | Difference |
| 3 | 1.478 | 1.478 | 0.0% | 1.470 | 1.469 | 0.1% | 1.455 | 1.452 | 0.2% |
| 60 | 1.384 | 1.387 | -0.2% | 1.244 | 1.251 | -0.6% | 1.188 | :1.193 | -0.4% |
| 120 | 1.285 | 1.289 | -0.3% | 1.197 | 1.200 | -0.3% | 1.153 | 1.160 | -0.6% |

Table 2-1: Performance Results of Cell frequency efficiency (b/s/Hz)

|  | TTI=2symbol |  |  | TTI=3(4)symbol |  |  | TTI=7symbol |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Speed/Kmh | Two Tier | Legacy | Difference | Two Tier | Legacy | Difference | Two Tier | Legacy | Difference |
| 3 | 0.0370 | 0.0377 | -1.84% | 0.0368 | 0.0377 | -2.4% | 0.0388 | 0.0397 | -2.3% |
| 60 | 0.0356 | 0.0353 | 0.85% | 0.0333 | 0.0316 | 5.3% | 0.0307 | 0.0303 | 1.3% |
| 120 | 0.0343 | 0.0333 | 3% | 0.0318 | 0.0306 | 3.9% | 0.0305 | 0.0298 | 2.3% |

Table 2-2: Performance Results of Edge 5% UE frequency efficiency (b/s/Hz)

Based on the simulation results, we observed that the two-tier CQI report scheme had the similar performance as the One Tier scheme.

Observation 2: The two-tier CQI report scheme has the similar performance as the One Tier scheme.

Based on the above discussion and observations, we have the following proposal.

Proposal 1: The two-tier CQI report scheme can be further studied in the case of sTTI.

# Conclusion

In this paper, we discuss the CQI report scheme, based on figure 2.

We observe that:

Observation 1: The two-tier CQI report scheme is capable of reducing 22.5% control overhead compared with one-tier CQI report scheme in sTTI.

Observation 2: The two-tier CQI report scheme has the similar performance as the One Tier scheme.

Based on the above observations, we have the following proposal:

Proposal 1: The two-tier CQI report scheme can be further studied in the case of sTTI.

# References

RP-150465, “New SI proposal: Study on Latency reduction techniques for LTE”, Ericsson, Huawei.

R1-162618 “Evaluation on fast link adaptation”, Huawei, HiSilicon

R1-163826 “Text proposal on findings from the system level evaluation of latency”, Ericsson

3GPP TR 36.213 V12.7.0 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures (Release 12)

R1-157806, “WF on evaluation methodology for latency reduction”, Ericsson, Huawei, HiSilicon, ZTE, CATT, Sharp, Samsung, Intel, Qualcomm, Interdigital, LGE, CMCC, ETRI

- Appendix

Table 1: Simulation Parameters

| Parameter | Assumptions |
| --- | --- |
| Layout | 19 Macro eNBs can be used, 3 sectors per site; |
| System bandwidth per carrier | 10MHz |
| Carrier frequency | 2GHz |
| Inter-site distance | 500m |
| Total BS TX power (Ptotal per carrier) | 46dBm |
| TTI length | 2/3(4)/7 symbols |
| Scheduler | Proportional fairness |
| Distance-dependent path loss | ITU UMa[referring to Table B.1.2.1-1 in TR36.814], with 3D distance between an eNB and a UE |
| Penetration | For outdoor UEs:0dB |
|  | For indoor UEs: 20dB+0.5din (din: independent uniform random value between [ 0, min(25,d) ] for each link) |
| Shadowing | ITU UMa according to Table A.1-1 of 36.819 with 3D distance for shadowing correlation distance |
| Antenna pattern | 3D, referring to TR36.819 |
| Antenna Height: | 25m |
| UE antenna Height | 1.5m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa according to Table A.1-1 of 36.819 |
| Antenna configuration | 2Tx(eNB), Cross-polarized 2Rx(UE), Cross-polarized |
| Number of UEs | 10 UEs per macro cell |
| UE dropping | Randomly and uniformly dropped throughout the macro geographical area. 20% UEs are outdoor and 80% UEs are indoor. |
| Traffic model | Full Buffer |
| CSI report period | The period of the first tier is 40 times of the period of the second tier. |
|  | The period of the second tier: 5TTI Note: Companies should provide details of CSI measurement |
| CSI report delay | 6 TTIs and milliseconds |
| UE receiver | MMSE-IRC; other UE receiver provided by companies |
| UE noise figure | 9dB |
| UE speed | 3km/h, 60km/h, 120km/h |
| Duplex mode | FDD |
| Network synchronization | Synchronized |

Table 7.2.3.1 and 7.2.3.2 from [4]