3GPP TSG RAN WG1 Meeting #85                                                     	                                       R1-164876

Nanjing, China, 23rd – 27th May 2016

Source:	Panasonic

Title: 	Design target on multiple access schemes for NR

Agenda Item:		7.1.3.2

Document for:	Discussion

Introduction

In RAN1 #84bis meeting [1], the following overview on multiple access study was agreed for NR.

Agreements:

- Study multiple access mechanisms including UL-grant less transmission, contention-based transmission, non-orthogonal multiple access

Agreements:

- Non-orthogonal multiple access should be investigated for diversified NR usage scenarios and use cases

- At least for UL mMTC, autonomous/grant-free/contention based non-orthogonal multiple access should be studied.

In this document, we discuss design targets of multiple access scheme for NR and show our initial view on multiple access study.

Discussion

Design targets for multiple access scheme

We discuss some of the design targets for multiple access below.

Spectral efficiency

NR should be designed to accommodate different numerologies optimized for specific use cases and deployment scenarios in order to support wide range of use cases and frequency bands (We proposed to use sub-RAT concept in [1]. The container optimized for each specific use case and deployment scenario is called as the “sub-RAT”). In order to support multiple sub-RATs optimized for specific use cases and deployment scenarios efficiently, the time, frequency, and space allocation of sub RATs should be more flexible. Assuming a conventional OFDMA or SC-FDMA, each sub-RAT which has different subcarrier spacing, CP length, and TTI would not be closely contained in the assigned bandwidth due to its large sidelobes. If the sidelobes to out-band can be reduced, different sub-RATs (for different use cases) can be efficiently multiplexed in the assigned bandwidth, leading to much efficient spectrum utilization. This topic would be more related to waveform design as discussed in [2]. In addition, multiplexing between different sub-RATs would be more related to numerology and forward compatibility issue as discussed in [3, 4]. The rest of the paper will treat only for the multiple access of UEs within sub-RAT.

To increase network throughput or area traffic capacity is one of key performance target for new radio systems [5]. In order to improve these, higher spectrum efficiency is required for multiple access design especially for eMBB. To increase spectral efficiency, multi-antenna technologies are essential technologies similar to LTE in new radio system. In addition, in higher frequency bands, high gain antennas with high directivity are effective to overcome the severe path loss effect. Massive MIMO technology with several tens of antenna elements and more realizes a high-gain multi-beam antenna system which can handle multiple users simultaneously. Therefore, especially for eMBB, ability to efficiently support of MIMO also needs to be considered for multiple access design.

Another approach is to increase spectral efficiency would be to introduce non-orthogonality. By sharing the same time and frequency resources intentionally by several UEs, there is the potential for enhancing the capacity of multiple access channel. The introduction of non-orthogonality between UEs for unicast channel would be possible in future phase similar to the introduction of MUST to LTE-Advanced Pro.

Observation 1: Especially for eMBB, ability to efficiently support MIMO needs to be considered for the design of multiple access scheme.

Asynchronous multiple access

In LTE, adaptive transmit timing control based on common control signals and random access channel is used in order to keep synchronization between UEs. For the services which require lower latency, if such adaptive transmit timing control is removed or alleviated, lower latency can be realized as the cost of timing offset generation and some potential frequency inefficiency. In such a use case, multiple access scheme (also waveform) which is robust against the timing offset would be useful. On the other hand, frequency synchronization aspect can be more demanding in low cost oscillator. The trade-off between frequency and time synchronization should take into account.

Observation 2: For URLLC, multiple access scheme which is robust against the timing offset would be useful.

Support small size of data efficiently

Frequency selective scheduling used in LTE improves system capacity by exploiting multiuser diversity gain at the expense of the loss of the radio resources assigned to the control channels that convey some sort of information necessary for scheduling. For mMTC, a higher number of small data burst devices should be supported and in this use case, multiple access scheme with minimal scheduling overhead is important design target.

Observation 3: For mMTC, multiple access scheme with minimal scheduling overhead is desirable in order to efficiently support small size of data.

Implementation complexity

Simple implementation complexity is important for reducing processing latency, power consumption, and cost. This is especially emphasized to target Giga-bit throughput communication system and low cost UEs. Complexity requirement can be relaxed only if sufficient gains are obtained.

Observation 4: Simple implementation complexity is one of key design targets for multiple access scheme.

Initial view on multiple access scheme for NR

We discuss multiple access schemes considering above key design targets into account.

- OFDMA / SC-FDMA

In LTE, orthogonal OFDMA and SC-FDMA is used for downlink and uplink, respectively. In orthogonal multiple access, since there is no multi-user interference within the same cell, the higher received SINR can be obtained. Therefore, assuming simple receiver without interference canceller, the throughput can be improved as it can simply support frequency-dependent scheduling and MIMO techniques.

On the other hand, from an information-theoretical point of view, orthogonal user multiplexing cannot achieve the capacity region of the multiple access channel. In addition, orthogonal multiple access does not allow asynchronous multiple access.

OFDMA and SC-FDMA would be an attractive solution for the use cases which require higher spectral efficiency.

- Non-orthogonal multiple access based on OFDMA and SC-FDMA

Introduction of non-orthogonality on the top of OFDMA and SC-FDMA by superposition of data symbols for different users would beneficial to capacity enhancement as the resource utilization could be further increased and the multiplexed users do not rely on spatial separation like MU-MIMO. This has been already studied in Rel.13 MUST SI and will be specified in Rel. 14 MUST WI for LTE.

The drawback is requiring more advanced/complicated receiver technology as it basically needs interference cancellation at the receiver. Synchronous operation is also required. In addition, signalling overhead is one of issues as notification of control signals among different UEs (such as transmit power ratio, precoding vector, and MCS information of other users…) is required.

- Other non-orthogonal multiple access schemes

As mentioned above, NR interfaces for mMTC and URLLC should be designed to accommodate the traffic requiring low latency and small size of data efficiently. In order to meet such a requirement, non-orthogonal multiple access schemes utilizing spreading technique [5-10] are useful as they has the potential of removing or alleviating the timing control and reducing scheduling overhead.

Taking the key design targets and characteristics of candidate multiple access schemes into account, we consider OFDMA and SC-FDMA as an attractive baseline multiple access scheme for NR interface in eMBB use case. The reason to say “baseline multiple access scheme” is that we envisage to introduce non-orthogonality is possible in future on top. In addition, the waveform in above OFDMA and SC-FDMA might not actually be the pure OFDM and DFT-spread OFDM. We consider other waveforms as discussed in [2].

For mMTC and URLLC, non-orthogonal multiple access schemes are further studied.

Proposal 1: Study OFDMA and SC-FDMA for new radio interface in eMBB use case as the baseline multiple access.

Conclusion

In this document we discussed design targets of multiple access scheme for NR and showed our initial view on multiple access study.

Observation 1: Especially for eMBB, ability to efficiently support MIMO needs to be considered for the design of multiple access scheme.

Observation 2: For URLLC, multiple access scheme which is robust against the timing offset would be useful.

Observation 3: For mMTC, multiple access scheme with minimal scheduling overhead is desirable in order to efficiently support small size of data.

Proposal 1: Study OFDMA and SC-FDMA for new radio interface in eMBB use case as the baseline multiple access.

Reference

[1] RWS-150008, “View on RAT and IMT-2020,” Panasonic

[2] R1-164875, “Design target and initial evaluation results on waveform for NR,” RAN1#85, Panasonic

[3] R1-164985, “Discussion on multiplexing of different numerologies,” RAN1#85, Panasonic

[4]	R1-162553, “Forward compatibility aspect of NR,” RAN1#84bis, Panasonic

[5]	R1-162226, “Discussion on multiple access for new radio interface,” RAN1#84bis, ZTE

[6] R1-163510, “Candidate NR Multiple Access Schemes,” RAN1#84bis, Qualcomm Incorporated

[7] R1-162155, “Sparse Code Multiple Access (SCMA) for 5G Radio Transmission,” RAN1#84bis, Huawei, HiSilicon

[8]	R1-163383, “Candidate Solution for New Multiple Access,” RAN1#84bis, CATT

[9] R1-162517, “Considerations on DL/UL multiple access for NR,” RAN1#84bis, LG Electronics

[10] R1-162385, “Multiple access schemes for new radio interface,” Intel Corporation