3GPP TSG RAN WG1 Meeting #85	R1-165059

Nanjing, China 23rd – 27th May 2016

Agenda Item:   7.1.5.1

Source:	  InterDigital Communications

Title:	          Performance Evaluation of Channel Coding for URLLC and mMTC

Document for:	  Discussion

# 1	Introduction

In the RAN1 meeting #84bis, a lot of contributions on the topic of channel coding design for the physical layer of the NR system were discussed. The initial agreements regarding the channel coding for NR include the following [1]:

- Candidates for 5G new RAT data transmission are identified as the following

  - LDPC code

  - Polar code

  - Convolutional code (LTE and/or enhanced convolutional coding)

  - Turbo code (LTE and/or enhanced turbo coding)

  - Note: It is RAN1 common understanding that combination of above codes is not precluded

  - Note: Outer erasure code is not precluded

- Selection of 5G new RAT channel coding scheme(s) will consider,

  - Performance

  - Implementation complexity

  - Latency (Decoding/Encoding)

  - Flexibility (e.g., variable code length, code rate, HARQ (as applicable for particular scenario(s)))

In this contribution, we present our preliminary analysis and simulation results for turbo code, LDPC code and polar code for Ultra-Reliable and Low Latency Communications (URLLC) and massive Machine-Type-Communications (mMTC) use cases. The comparison and evaluation of the performance of these codes are provided.

# 2	Discussion

To facilitate unified performance evaluation, the simulation assumptions for channel coding for URLLC and mMTC use cases have been agreed [2] (see Appendix for details). Our simulations are based on these assumptions and our performance evaluation is in terms of the Block Error Rate (BLER) vs. SNR.

## 2.1 	Candidate Channel Codes

In this section, we introduce the channel codes used in our simulation evaluations.

Polar code:

We consider a N,K polar code, where K is the information block length and N is the coded block length. Here, the value N is a set as a power of 2, i.e., N=2n for some integer n. The generator matrix of the polar code is GN=G2⨂n, where (∙)⨂n denotes the n-th Kronecker power and G2=1011. The frozen bits selection is based on the Bhattacharyya bounds (cf. [3]) with the design-SNR being 0 dB. Puncturing is applied from the bottom of the coded block. When M bits are punctured, the effective coding rate of polar codes is KN-M.

LDPC code:

We consider a (N,K) Quasi-Cyclic LDPC (QC-LDPC) code (cf. [4]), where K is the information block length and N is the coded block length. The parity check matrix H is a sparse matrix with size (N-K)×N. A QC-LDPC code can be uniquely defined by its base matrix with size J×L

B=B1,1⋯B1,L⋮⋱⋮BJ,1…BJ,L

Each component in the base matrix may be a p×p circulant permutation matrix or an all zero matrix. A positive integer value of Bi,j represents the circulant permutation matrix which is circularly right shifted Bi,j from the p×p identity matrix. Identity matrix is indicated by Bi,j=0, while a negative value of Bi,j indicates an all zero matrix. Note that N = L∙p.

We apply the row-column constraint: no two rows (or two columns) can have more than one place where they both have non-zero components. This ensures good performance as the resulting parity check matrix H is free of cycles of length 4, thus a girth of at least 6 is guaranteed.

Turbo code: We use the turbo code in LTE systems [5].

## 2.2 	Performance Comparison of Channel Codes

In our simulations, we apply the max-log-MAP decoding algorithm for turbo code with the maximum number of iterations as 8; the sum-product decoding algorithm for LDPC codes with the maximum number of iterations as 20; the CRC-aided successive cancellation list decoding algorithm with list size of 1, 4 or 32 for polar codes. We simulated the coding rates 1/3 and 1/6 specified in Table 4, with the detailed information block lengths and coded block lengths summarized in Table 1. In LTE, turbo implementation on code rate lower than 1/3 is achieved by repeating the bits from circular buffer.

Table 1: Block length used in simulations

|  | R = 1/3 |  |  | R=1/6 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LDPC | Turbo | Polar | LDPC | Turbo | Polar |
| Information block length (bits) | 168 | 168 | 170 | 170 | 168 | 170 |
| Coded block length w/o puncturing (bits) | 504 | 504 | 512 | 1020 | 1008 | 1024 |
| Punctured bits | NA | NA | 2 | NA | NA | 4 |

Figure 1 provides the BLER performance comparison of rate 1/3 codes. It is seen from the figure that the polar code with list 32 has the best performance for QPSK modulation and the turbo code has the best performance for 16QAM. LDPC code and polar code with list 1 do not perform as well as Turbo code, with the gap of about 0.5 to 1 dB.

Figure 2 provides the BLER performance comparison of rate 1/6 codes. It is seen from the figure that the polar code with list 32 always has the best performance. The polar code with list 4 has the similar performance as turbo code. Again, LDPC code and polar code with list 1 do not perform as well as turbo code.

Observation 1: For small information block length with coding rates 1/3 and 1/6, LDPC code and polar code with list 1 do not perform as well as turbo code. Polar code with list 32 outperforms turbo code in most cases, while polar code with list 4 has similar performance as turbo code.

We also simulated turbo, LDPC and polar codes at rate 1/12. One way of implementing LDPC and polar codes is to simply repeat the coded block generated from rate 1/6 LDPC and polar codes. Similarly, the (4 times) repetition of the rate 1/3 turbo codes is applied.

Figure 3 provides the BLER performance comparison of rate 1/3 codes. It is seen from the figure that the polar code with list 32 always has the best performance. The polar code with list 4 has the similar performance as turbo code. Again, LDPC code and polar code with list 1 do not perform as well as turbo code. Note that in practical scenarios, a low coding rate of 1/12 generally might not couple with high modulation order. We include the 16QAM modulation results here just for completeness.

Observation 2: For small information block length with coding rate 1/12, LDPC code and polar code with list 1 do not perform as well as turbo code. Polar code with list 32 outperforms turbo code, while polar code with list 4 has similar performance as turbo code.

For convenience, we summarize the minimum SNR (in dB) required for turbo, LDPC, and polar codes to achieve the target BLER levels, shown in Table 2. This table is derived from Figures 1-3. The numbers in green indicate the best performance, and the numbers in red indicate the worst performance.

Table 2: The minimum SNR required for turbo, LDPC, and polar codes to achieve the target BLER (unit: dB)

| Code Rate | BLER | QPSK |  |  |  |  | 16QAM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Turbo | LDPC | Polar-1 | Polar-4 | Polar-32 | Turbo | LDPC | Polar-1 | Polar-4 | Polar-32 |
| 1/3 | 1e-3 | 0.52 | 1.04 | 1.44 | 0.58 | 0.33 | 5.56 | 6.18 | 7.07 | 6.16 | 5.90 |
|  | 1e-4 | 0.94 | 1.69 | 1.94 | 1.07 | 0.78 | 5.98 | 6.98 | >7.5 | 6.79 | 6.59 |
| 1/6 | 1e-3 | -2.48 | -1.94 | -1.44 | -2.49 | -3.13 | 2.21 | 2.87 | 3.5 | 2.23 | 1.61 |
|  | 1e-4 | -1.96 | -1.51 | -0.98 | -2.13 | -2.62 | 2.70 | 3.28 | >3.5 | 2.70 | 2.06 |
| 1/12 | 1e-3 | -5.53 | -4.97 | -4.40 | -5.53 | -6.13 | -0.58 | 0.48 | 1.16 | 0.13 | -0.64 |
|  | 1e-4 | -5.15 | -4.49 | -4.03 | -5.12 | -5.62 | -0.24 | 0.81 | >1.5 | 0.28 | -0.22 |

Another way of implementing rate 1/12 LDPC and polar codes is to directly construct the corresponding codes. Specifically, we construct a (2040, 170) LDPC code and a (2048, 170) polar code with 8 bits punctured. Figure 4 compares the performance of these codes with the ones with repetition. Conclusions can be drawn from the figure that polar code with repetition outperforms the one without repetition, and the gain is as large as 1.5 dB. Similarly, the LDPC code with repetition outperforms the one without repetition for about 0.5 dB. The major gain obtained from repetition is because the corresponding SNR belongs to the power-limited regime.

Observation 3: For small information block length with rate 1/12, LDPC and polar codes with repetition outperform the ones without repetition.

Figure 1: Performance comparison for rate 1/3 codes

Figure 2: Performance comparison for rate 1/6 codes

Figure 3: Performance comparison for rate 1/12 codes

Figure 4: Performance comparison for LDPC and polar codes with or without repetition for QPSK modulation

## 2.3	Complexity Analysis

In this section, we compare the decoding complexity for the codes we have simulated. Specifically, our comparison is in terms of operations count. For turbo codes and LDPC codes, we apply the conclusions in [6], while for polar codes, we utilize the results from [7]. For the rate 1/3 LDPC code, we have the degree distribution dv=2.6667, dc=4; For the rate 1/6 LDPC code, we have the degree distributiondv=2.75, dc=3.3.

The decoding complexity comparison of turbo, LDPC and polar codes is shown in Table 3. Note that LDPC and polar codes with rate 1/12 are implemented using rate 1/6 codes with repetition 2. The repeated bits are combined at receiver side. Thus, the decoding complexity is the same as those of rate 1/6 LDPC and polar codes. Since the information block sizes of simulated turbo code are identical for rates 1/3, 1/6, 1/12, the decoding complexity of turbo code remains the same.

It is seen from the table that polar code with list 1 or 4 always has lower decoding complexity than turbo or LDPC code. LDPC code has lower decoding complexity than turbo code at rate 1/3, but has slightly higher decoding complexity than turbo code at rate 1/6. The polar code with list 32 has lower decoding complexity than turbo code at rate 1/3, but higher decoding complexity at rate 1/6. One way to reduce the decoding complexity of polar code with list 32 at rate 1/6 or 1/12 is to apply the rate 1/3 polar code with repetition.

Table 3: Decoding complexity comparison of turbo, LDPC, polar codes (unit: operations count)

|  | Turbo (log- map) | Turbo (max-log- map) | LDPC (sum- product) | LDPC (min- sum) | Polar (L=1) | Polar (L=4) | Polar (L=32) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R=1/3 | 788,928 | 229,824 | 423,360 | 114,240 | 4,778 | 20,472 | 180,096 |
| R=1/6 or R=1/12 (rep.) | 788,928 | 229,824 | 880,600 | 241,400 | 10,410 | 43,000 | 360,320 |

Taking into account both BLER performance and decoding complexity, we propose to consider polar code with list between 4 and 32 as a candidate channel code for URLLC and mMTC. At lower coding rates, the repetition of channel code might be applied.

Proposal 1: Taking into account BLER performance and decoding complexity, polar code with list between 4 and 32 could be considered as a candidate channel code for URLLC and mMTC. At lower coding rates, the repetition of channel code might be applied.

# 3	Conclusion

In this contribution, we discussed various coding schemes for URLLC and mMTC, compared their performance and complexity. Our simulation results show that:

Observation 1: For small information block length with coding rates 1/3 and 1/6, LDPC code and polar code with list 1 do not perform as well as turbo code. Polar code with list 32 outperforms turbo code in most cases, while polar code with list 4 has similar performance as turbo code.

Observation 2: For small information block length with coding rate 1/12, LDPC code and polar code with list 1 do not perform as well as turbo code. Polar code with list 32 outperforms turbo code, while polar code with list 4 has similar performance as turbo code.

Observation 3: For small information block length with rate 1/12, LDPC and polar codes with repetition outperform the ones without repetition.

Hence, we have the following proposal:

Proposal 1: Taking into account BLER performance and decoding complexity, polar code with list between 4 and 32 could be considered as a candidate channel code for URLLC and mMTC. At low coding rate of 1/12, the repetition of channel code might be applied.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #84bis, Busan, Korea, Apr. 2016.

- R1-163757, “WF on channel coding evaluation for 5G new radio,” Apr. 2016.

- H. Vangala, E. Viterbo and Y. Hong, “A comparative study of polar code constructions for the AWGN channel,” arXiv: 1501.02473v1, cs.IT, Jan. 2015.

- IEEE Std 802.11™-2012, "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

- 3GPP TS 36.212 v13.1.0, “E-UTRA multiplexing and channel coding.” Mar. 2006.

- R1-060874, “Complexity comparison of LDPC codes and Turbo codes,” Mar. 2006.

- R1-162162, “High-level comparison of candidate FEC schemes for 5G,” Apr. 2016.

# Appendix: Evaluation Assumptions for Channel Coding for URLLC and mMTC

Table 4: Simulation assumptions for URLLC and mMTC [2]

| Channel | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK, 16QAM |  |  |  |
| Coding scheme | Convolutional | LDPC | Polar | Turbo |
| Code rate | 1/12, 1/6, 1/3 |  |  |  |
| Decoding algorithm | List-X Viterbi | min-sum | List-Y | Max-log-MAP |
| Info. block length (bits w/o CRC) | 20, 40, 200, 600, 1000 |  |  |  |