3GPP TSG-RAN WG1#85	R1-165100

Nanjing, China 23rd - 27th May 2016

Source:	Ericsson

Title:	High Resolution CSI feedback

Agenda Item:	6.2.3.2.3

Document for:	Discussion and Decision

# Introduction

In the eFD-MIMO WI objective [1], it is said that RAN1 should evaluate and if needed specify CSI enhancements improving eNB precoding to support multi-user transmission, and in RAN1#84b it was decided that companies should propose and evaluate schemes for advanced CSI reporting for RAN1#85.

Conclusions:

- Evaluate proposed advanced CSI schemes

- Baseline for comparison:

  - Implicit CSI feedback based on Rel.13 codebook (as well as its extension to >16 ports) or other legacy codebooks if applicable

- Performance and feedback overhead should be provided

- Simulation scenarios:

  - FTP model 1 with 70% and 50% traffic loading

  - Evaluations should include dynamic switching between MU- and SU-MIMO

  - Other simulation details can refer to 36.897 EB/FD MIMO SI

Candidates for PMI Enhancement:

- Codebook based (implicit feedback)

  - Linear combination codebook (enhanced W2) for Non-precoded CSI-RS and beamformed CSI-RS

  - MU CSI (e.g., additional i1, i1,1, or i1,2)

  - Other new or modified codebooks

- Explicit feedback

  - Channel quantization

  - Eigenvector quantization

  - Covariance matrix quantization

- Analog feedback

- Note: other schemes are not precluded

Candidates for CQI Enhancement:

- CQI Derivation

  - Interference estimation based on NZP CSI-RS

  - MU-CQI conditioned on MU hypotheses and CSI-IM

- Reduced CQI feedback delay

- Note: other schemes are not precluded

In this contribution, we discuss and present such evaluation results for both explicit and implicit advanced CSI feedback schemes.

# Explicit feedback based schemes

It is well known that MU-MIMO can yield large performance gains if sufficiently large antenna arrays are used and if detailed CSI is available at the transmitter. Ideally, the transmitter should have the explicit channel matrix and interference covariance matrix of each user available, for each PRB (or PRG). However, this would require feeding back a vast amount of channel coefficients which is not plausible from a feedback overhead perspective. One immediate way of reducing the feedback overhead is to average the channel matrix over frequency and feed back a channel correlation matrix, the strongest eigenvector(s) or an averaged channel matrix, per frequency interval. We present such simulation results in Table 1, comparing MU-MIMO using explicit channel and interference feedback with different frequency granularity to SU- and MU-MIMO using a Rel. 13 DFT codebook extended to 32 ports. The antenna layout is an 8x4 antenna array using a 2x1 virtualization and FTP Model 1 with 500kB packet size is used in the 3D-UMi scenario. The explicit channel feedback is unquantized and calculated based on noisy CSI-RS channel estimates and is averaged over {50, 12, 6, 1} PRBs. For MU-MIMO, a SLNR-based precoding scheme with dynamic SU/MU-switching is used.

Table 1: Evaluation result comparing explicit CSI with different frequency granularity.

| Scheme | 50% RU |  | 70% RU |  |
| --- | --- | --- | --- | --- |
|  | CE UTP gain [%] | Mean UTP gain [%] | CE UTP gain [%] | Mean UTP gain [%] |
| Rel. 13 DFT Codebook SU-MIMO | 0 | 0 | 0 | 0 |
| Rel. 13 DFT Codebook MU-MIMO | 28 | 9 | 67 | 26 |
| Explicit FB MU-MIMO 50 PRB granularity | 29 | 11 | 84 | 29 |
| Explicit FB MU-MIMO 12 PRB granularity | 84 | 35 | 184 | 70 |
| Explicit FB MU-MIMO 6 PRB granularity | 114 | 47 | 242 | 87 |
| Explicit FB MU-MIMO 1 PRB granularity | 154 | 57 | 292 | 102 |

As MU-MIMO performance relies heavily on accurate nullforming to suppress interference, it makes sense that granular phase information of the channel is needed. If the channel is averaged over frequency, rapid phase-variations due to delay difference between channel taps are averaged out and the capability to form nulls by applying appropriate phase shifts to different spatial directions is lost. When the explicit channel feedback is averaged over the entire 50 PRB bandwidth, the MU-MIMO performance is similar to Rel. 13 DFT Codebook.  The explicit FB MU-MIMO scheme yields the best performance when the averaging is limited to 1 PRB.

Observation:

- Explicit channel feedback needs to be frequency-selective in order to show gains over implicit feedback using DFT codebook.

From the above observation, it is thus clear that the explicit channel feedback has to be compressed in some other way in order to have a reasonable overhead. The traditional approach of most kinds of compression algorithms is to utilize correlation between samples in order to compress them. One such way is to find a basis where the sample vector can be expressed sparsely, perform a basis change by multiplying the sample vector with a basis transformation matrix, and finally discard everything but a couple of the strongest coefficients of the sample vector expressed in that basis. If the sample vector is sparse enough, the compression loss is small.

Such a method could of course be applied to the explicit channel feedback problem, and a natural basis that could sparsely express the channel matrix H from a 2D antenna array is a 2D DFT basis. Given that the propagation channel between the 2D antenna array and the UE antenna is spatially correlated, the channel matrix, expressed in 2D-DFT beam space, could be significantly sparser than expressed in antenna space. The equivalent 2D-DFT beam space channel matrix can be derived by multiplying the antenna space channel matrix H with an orthonormal transformation matrix BNV,NH =DNH⨂DNV00DNH⨂DNV as

HDFT=HBNV,NH ,

where DN is a size N×N DFT matrix, i.e. the elements of DN  are defined as  DNk,l=1Nej2πklN.  The columns dii=12NHNV of  BNV,NH constitutes an orthonormal basis of the vector space C2NVNH and each column corresponds to a beam transmitted on one polarization.

An explicit feedback scheme could thus work by selecting the Nbeams strongest beams (which could be selected once for the entire bandwidth) and only feeding back the 2NbeamsNRX beam space channel coefficients of the reduced beam space, where NRX  is the number of receive antennas at the UE .

One can further reduce the number of beams required to express the channel matrix by allowing the beam space to be rotated, that is, multiplying the DFT matrix for each dimension with a size N×N rotation matrix RNq=diag(ej2π⋅0⋅qNej2π⋅1⋅qN⋯ej2π⋅(N-1)⋅qN) , defined for 0≤q<1. Multiplying DN with RNq from the left creates a rotated DFT matrix with entries RNqDNk,l=1Nej2πk(l+q)N. Rotating the beam space basis has an effect similar to oversampling a codebook, for example, if the channel is a pure LOS channel and the angle of the LOS ray if perfectly aligned with a constituent beam in the beam space, the channel matrix can be described by only one beam coefficient. However, if the angle of the LOS ray lies in between two beams in the beam space, two beam coefficients are required to express the channel, doubling the amount of overhead needed.

We have performed simulations evaluating the performance of reduced beam space explicit channel feedback. Using the same setup as earlier, i.e. a 4x4 port layout, the full beam space channel consist of 16 orthogonal beams. We have investigated the performance of only feeding back the channel coefficients for Nbeams=1,2,3,5,10,16 beams. A set of coefficients are fed back for each 6 PRB subband and 4 beam space rotation hypotheses per dimension are evaluated. The beam space rotation and beam selection is done once for the entire bandwidth, which consumes 4 feedback bits for signaling the chosen rotation hypothesis and Nbeams⋅log216 bits indicating the beam selection. The wideband overhead is thus 4+4Nbeams bits. For each subband, 2NbeamsNRXNqbits bits are required, where Nqbits=4,6,10 is the number of bits spent on quantizing the phase and amplitude of each coefficient. The total overhead is thus 4+4Nbeams+9⋅2NbeamsNRXNqbits, for 9 subbands Evaluation results are presented in Table 2 below.

Table 2: Simulation results comparing reduced beam space explicit feedback

| 50% RU |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell edge UTP gain [%] |  |  |  |  | Average UTP gain [%] |  |  |  |  |  |
| Rel. 13 DFT Codebook SU-MIMO |  |  | 0 |  | Rel. 13 DFT Codebook SU-MIMO |  |  |  | 0 |  |
| Rel. 13 DFT Codebook MU-MIMO |  |  | 28 |  | Rel. 13 DFT Codebook MU-MIMO |  |  |  | 9 |  |
| Reduced beam space explicit feedback MU-MIMO |  |  |  |  | Reduced beam space explicit feedback MU-MIMO |  |  |  |  |  |
| Number of Beams | Coefficient Quantization |  |  |  | Number of Beams | Coefficient Quantization |  |  |  |  |
|  | Unquantized | 4 bits | 6 bits | 10 bits |  | Unquantized | 4 bits | 6 bits |  | 10 bits |
| 1 | 36 | 19 | 33 | 40 | 1 | 13 | 3 | 11 |  | 12 |
| 2 | 75 | 40 | 73 | 73 | 2 | 26 | 13 | 25 |  | 27 |
| 3 | 102 | 45 | 77 | 99 | 3 | 35 | 15 | 29 |  | 35 |
| 5 | 110 | 39 | 95 | 111 | 5 | 43 | 12 | 36 |  | 43 |
| 10 | 103 | 1 | 79 | 122 | 10 | 45 | -4 | 33 |  | 48 |
| 16 (Full channel) | 114 | -19 | 73 | 106 | 16 (Full channel) | 47 | -17 | 29 |  | 46 |
| 70% RU |  |  |  |  |  |  |  |  |  |  |
| Cell edge UTP gain [%] |  |  |  |  | Average UTP gain [%] |  |  |  |  |  |
| Rel. 13 DFT Codebook SU-MIMO |  |  | 0 |  | Rel. 13 DFT Codebook SU-MIMO |  |  |  | 0 |  |
| Rel. 13 DFT Codebook MU-MIMO |  |  | 67 |  | Rel. 13 DFT Codebook MU-MIMO |  |  |  | 26 |  |
| Reduced beam space explicit feedback MU-MIMO |  |  |  |  | Reduced beam space explicit feedback MU-MIMO |  |  |  |  |  |
| Number of Beams | Coefficient Quantization |  |  |  | Number of Beams | Coefficient Quantization |  |  |  |  |
|  | Unquantized | 4 bits | 6 bits | 10 bits |  | Unquantized | 4 bits | 6 bits |  | 10 bits |
| 1 | 99 | 82 | 93 | 87 | 1 | 27 | 21 | 27 |  | 26 |
| 2 | 161 | 107 | 149 | 155 | 2 | 55 | 31 | 50 |  | 53 |
| 3 | 200 | 106 | 181 | 195 | 3 | 66 | 35 | 62 |  | 63 |
| 5 | 226 | 92 | 198 | 227 | 5 | 79 | 27 | 66 |  | 78 |
| 10 | 244 | 36 | 184 | 251 | 10 | 89 | 5 | 67 |  | 87 |
| 16 (Full channel) | 242 | *** | 154 | 207 | 16 (Full channel) | 87 | *** | 58 |  | 81 |

Observations:

- Explicit channel feedback of only the strongest beam has similar performance as DFT codebook

  - Both cases describe only the strongest DFT beam

- Performance increases rapidly when feeding back CSI for more beams, 3-5 beams give similar performance as full channel feedback.

- At least 6 bits per coefficient is needed for sufficient performance for 3-5 beams. The more beams are included in the report; the more bits are required to quantize the coefficients in order to have good performance.

- Quantizing the beam space coefficients with 6 bits gives only a small quantization loss.  With 3 beams, a 181% CE UTP gain and 62% average UTP gain over Rel. 13 DFT Codebook SU-MIMO at 70% RU is seen. The resulting feedback overhead is 4+3*4+2*3*2*6*9=664 bits, for a 10MHz BW and 2 UE RX antennas.

Based on these observations we make the following proposal.

Proposal:

- If explicit feedback is specified in Rel. 14, it should be based on frequency-selective, reduced beam space coefficient feedback.

# Implicit feedback based schemes

As shown in the previous section, explicit channel feedback can significantly increase the performance of MU-MIMO compared to DFT codebook due to fact that the spatial richness of the channel is captured in the CSI feedback. However, introducing explicit feedback would be a large deviation from the current implicit CSI feedback framework adopted in LTE, and would e.g. require new testing methodologies in RAN4. It would thus be desirable if the implicit feedback paradigm could be kept when introducing high resolution CSI feedback. Further, using an implicit feedback framework makes it possible to optimize the tradeoff between performance and overhead in an entirely different fashion than explicit feedback, by e.g. codebook design.

One option is to introduce a new kind of codebook that better captures spatially rich channels. In the previous section, we showed that reduced beam space coefficient feedback was an effective method of compressing explicit CSI. The same methodology can be adopted also for implicit CSI feedback when designing an enhanced codebook.

Consider a rotated 2D-DFT beam space transformation matrix BNV,NH qV,qH, as introduced in the previous section,

BNV,NH qV,qH=RNHqHDNH⨂RNVqVDNV00RNHqHDNH⨂RNVqVDNV=d10d20……dNVNH00d10d2……0dNVNH .

The columns of BNV,NH qV,qH thus correspond to mutually orthogonal rotated DFT beams transmitted on one polarization. A design principle of an enhanced codebook should be to construct the W1 matrix by selecting a subset IS of the columns of BNV,NH qV,qH, corresponding to the Nbeams strongest beams, as

W1=BIS=dIS(1)0dIS(2)0……dIS(Nbeams)00dIS(1)0dIS(2)……0dIS(Nbeams)

and linearly combine the Nbeams orthogonal beams per polarization in W2 by setting the co-phasing and relative power level of the beams as,

W2=p1ejα1p2ejα2⋮p2Nbeamsejα2Nbeams .

Note that each column of W2 could have this structure in case of multi-layer transmission. Note  also that p1 and ejα1 can be set to one as only the relative power level and phase between the beams needs to be captured.

In order to reduce the feedback overhead, however, one might consider merging in the relative power levels of the beams in the wideband W1 factor as w=BISp1ejα1p2ejα2⋮p2Nbeamsejα2Nbeams =dIS(1)0dIS(2)0……dIS(Nbeams)00dIS(1)0dIS(2)……0dIS(Nbeams)p100p2⋱⋱⋱00p2Nbeamsejα1ejα2⋮ejα2Nbeams =BISP=W1ejα1ejα2⋮ejα2Nbeams =W2=W1W2

We have evaluated the performance of MU-MIMO where the described multi-beam precoder CSI feedback has been used, using the same simulation scenario and assumptions as in the previous section. A multi-beam codebook with 4 rotation hypotheses per dimension and wideband relative beam power quantization using 2 bits is used.  We investigate how many bits are needed to quantize the co-phasing factors ejαi and how many orthogonal beams in the precoder are needed for sufficient MU-MIMO performance. The results are presented in Table 5. Similarly as for the explicit CSI case, including 3-5 beams seem to give sufficient MU-MIMO performance.  For phase quantization, it seems that at least 3 bits per beam is needed in order to have good enough performance. With 3 beams and 10MHz BW, the resulting feedback overhead is 4+3*4 for beam selection and selection of rotation hypothesis, 2*2=4 bits indicating the relative power level of the beams and 2*3*3*9=162 bits for the co-phasing factors, resulting in 182 bits in total for a rank 1 precoder, which is significantly lower than overhead for the corresponding explicit feedback scheme. The gains with the proposed implicit feedback scheme is however somewhat lower than for the proposed explicit feedback scheme, although this might be compensated by the lower feedback overhead.

Table 5: Evaluation results for multi-beam codebook implicit feedback.

| 50% RU |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell edge UTP gain [%] |  |  |  |  |  | Average UTP gain [%] |  |  |  |  |  |
| Rel. 13 DFT Codebook SU-MIMO |  |  |  | 0 |  | Rel. 13 DFT Codebook SU-MIMO |  |  |  | 0 |  |
| Rel. 13 DFT Codebook MU-MIMO |  |  |  | 28 |  | Rel. 13 DFT Codebook MU-MIMO |  |  |  | 9 |  |
| Multi-beam codebook MU-MIMO |  |  |  |  |  | Multi-beam codebook MU-MIMO |  |  |  |  |  |
| Number of Beams | Phase Quantization |  |  |  |  | Number of Beams | Phase Quantization |  |  |  |  |
|  | Unquantized per RB | PSK per subband |  |  |  |  | Unquantized per RB | PSK per subband |  |  |  |
|  |  | 2 bits | 3 bits |  | 10 bits |  |  | 2 bits | 3 bits |  | 10 bits |
| 1 | 37 | 34 | 37 |  | 41 | 1 | 11 | 9 | 12 |  | 11 |
| 2 | 68 | 47 | 48 |  | 57 | 2 | 22 | 12 | 16 |  | 18 |
| 3 | 80 | 48 | 68 |  | 65 | 3 | 26 | 15 | 20 |  | 22 |
| 5 | 95 | 50 | 66 |  | 79 | 5 | 32 | 17 | 24 |  | 26 |
| 10 | 92 | 48 | 59 |  | 72 | 10 | 33 | 16 | 22 |  | 25 |
| 70% RU |  |  |  |  |  |  |  |  |  |  |  |
| Cell edge UTP gain [%] |  |  |  |  |  | Average UTP gain [%] |  |  |  |  |  |
| Rel. 13 DFT Codebook SU-MIMO |  |  |  | 0 |  | Rel. 13 DFT Codebook SU-MIMO |  |  |  | 0 |  |
| Rel. 13 DFT Codebook MU-MIMO |  |  |  | 67 |  | Rel. 13 DFT Codebook MU-MIMO |  |  |  | 26 |  |
| Multi-beam codebook MU-MIMO |  |  |  |  |  | Multi-beam codebook MU-MIMO |  |  |  |  |  |
| Number of Beams | Phase Quantization |  |  |  |  | Number of Beams | Phase Quantization |  |  |  |  |
|  | Unquantized per RB | PSK per subband |  |  |  |  | Unquantized per RB | PSK per subband |  |  |  |
|  |  | 2 bits | 3 bits |  | 10 bits |  |  | 2 bits | 3 bits |  | 10 bits |
| 1 | 105 | 92 | 103 |  | 105 | 1 | 31 | 29 | 30 |  | 31 |
| 2 | 166 | 114 | 124 |  | 141 | 2 | 50 | 36 | 40 |  | 42 |
| 3 | 180 | 122 | 139 |  | 156 | 3 | 57 | 39 | 46 |  | 51 |
| 5 | 195 | 118 | 142 |  | 168 | 5 | 66 | 39 | 48 |  | 56 |
| 10 | 179 | 95 | 132 |  | 129 | 10 | 65 | 35 | 47 |  | 49 |

Observation:

- Substantial throughput gains also for the proposed enhanced implicit feedback rich CSI scheme. With a precoder consisting of 3 linearly combined beams and 8-PSK cophasing, a 124% CE UTP gain and a 46% average UTP gain over Rel.13 DFT Codebook SU-MIMO is seen, compared to 67% CE UTP gain and 26% average UTP gain for Rel.13 DFT Codebook MU-MIMO. The resulting overhead is 182 bits in total for a rank 1 precoder, which is significantly lower than overhead for the corresponding explicit feedback scheme.

We therefore make the following proposal:

Proposal:

- If implicit feedback enhancements for MU-MIMO is specified in Rel. 13, it should be based a high resolution codebook with precoders consisting of linear combinations of orthogonal 2D DFT beams.

# Discussion

As shown in this contribution, the gap in MU-MIMO performance between Rel.13 DFT Codebook CSI feedback and high resolution CSI feedback can be substantial for 32 TXRUs and can be expected to be even larger when the number of TXRUs increases even further.  To be able to fully utilize the potential of the increased number of TXRUs considered in Rel. 14, we think that it is necessary to specify CSI enhancements for MU-MIMO.  We have in this contribution proposed concepts for both implicit and explicit feedback schemes that can significantly increase MU-MIMO performance without incurring too large additional feedback overhead. We therefore propose that RAN1 should further study these schemes and  decide to specify either implicit or explicit CSI feedback enhancements, taking into account MU-MIMO performance, feedback overhead, testability and UE complexity issued of the respective schemes.

Proposal:

- Specify high resolution CSI feedback supporting MU-MIMO in Rel. 14 using either an implicit or explicit feedback framework.

# Conclusions

In this contribution we have discussed and presented evaluation results for high resolution CSI feedback targeting MU-MIMO. Based on this, we have made the following proposals.

Proposals:

- If explicit feedback is specified in Rel. 14, it should be based on frequency-selective, reduced beam space coefficient feedback.

- If implicit feedback enhancements for MU-MIMO is specified in Rel. 13, it should be based a high resolution codebook with precoders consisting of linear combinations of orthogonal 2D DFT beams.

- Specify high resolution CSI feedback supporting MU-MIMO in Rel. 14 using either an implicit or explicit feedback framework.

# References

RP-160623, “New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE”, Samsung, Nokia, CATT, 3GPP TSG RAN Meeting #71, Göteborg, Sweden, March 7 - 10, 2016

# Appendix

| Simulation Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMi 200m ISD |
| Antenna Configurations | 8x4 with 2x1 virt., UMi (130° tilt) |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (UMi) |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead accounted for. Channel estimation error modeled. |
| Codebook | 2D Grid of Beams based on DFT |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |
| Transmission Mode | TM10, with non-shifted CRS |