3GPP TSG RAN WG1 Meeting #85		R1-165264

Nanjing, PRC, 23 - 27 May 2016

Source: 	Ericsson

Title:	Contents of PSCCH for V2V over PC5

Agenda Item:	6.2.2.5

Document for:	Discussion and Decision

# Introduction

In RAN1 #84bis it was agreed that:

Agreement:

- Proposal 1:

  - Priority handling is supported over PC5 for eNB-scheduled and UE- autonomous V2V communication

- Proposal 2:

  - The priority information is taken into account in the resource (re)selection for UE-autonomous mode

    - Further details are FFS

- Proposal 3:

  - Down-select between two alternatives at RAN1#85 meeting

    - Alt.1 Priority information is signaled in SCI

    - Alt.2 Priority information is not signaled in SCI

Agreement:

- The following two cases are supported:

  - SA and the associated data are transmitted in the same TTI,

  - SA and the associated data are transmitted in different TTIs

    - The scheduling timing between SA and associated data is variable

      - In UE-autonomous resource selection mode, the timing is chosen by the transmitting UE from a configurable range

      - In eNB-scheduling mode, the timing is determined by eNB

      - SA includes information about the scheduling timing

Note: the association does not necessarily include the case of intention of using the resources for a different TB, if any (which is FFS)

The above agreements dictate at least part of the contents of the PSCCH and the DCI for support of V2X over PC5. In this paper we discuss the contents of the PSCCH. In a companion paper [1] we discuss related issues in the DCI in case of Mode-1 resource allocation

# Contents of PSCCH for V2X over PC5

To support V2X a lot of enhancements to the sidelink are required, especially enhancements to resource pool structures and resource allocation mechanisms. As a result, the contents of PSCCH need to be extensively revised. The following aspects need to be taken into account:

- Scheduling and booking: In our concept (details in [2]) each SA is used to schedule the (re)transmissions of a TB and book resources for transmissions of a future TB. Booking simply means that the UE declares its intention to use the frequency resources in the future (with a certain time offset from a current use) by the same UE.

- Priority: the concept of priority is useful for traffic differentiation and contention resolution, which aligns with QoS/bearer/traffic management aspects being discussed in RAN2 as well. Detailed use of priority information can be found in our companion contribution [5]. We also believe that SA is the best place to carry priority information, namely Alt. 1 of Proposal 3 in RAN1 #84bis agreement listed in Section 1 should be selected. As we discuss in [3], priority information contained in the SA can be used for resource selection.

- Another aspect that needs to be considered is the differentiation between centralized (mode 1) and autonomous resources (mode 2) in case of coexistence between them.

Proposal:

- RAN1 selects Alt.1 in the following:

  - Alt.1 Priority information is signaled in SCI

  - Alt.2 Priority information is not signaled in SCI.

Observation:

- The following aspects need to be considered in designing contents of SA:

  - Scheduling a TB and booking resources for a future TB.

  - Priority of traffic

  - Mode of resource allocation

Taking the above observations and the agreements in RAN1 #84bis into consideration, we propose the following format for the SCI used for V2X over PC5, captured in Table 1.

Table 1 : Contents of PSCCH for V2X over PC5

| Name of field | Meaning | Length in bits (indicative) |
| --- | --- | --- |
| Priority | Indicate priority of the associated data packets. This can be used for resource collision resolution, e.g., sensing for resource selection and reselection. More details are provided in our contributions [3][4][5]. | 2-3 |
| Resource allocation mode | Indicate whether the SA/data has been allocated by the eNB (mode-1) or autonomously by the UE (mode-2). It can be used for mitigating resource collisions, together with priority information. More details are provided in our contributions [3][4][5]. | 1 |
| Timing advance indication | Indicate timing adjustment for receiving the associated data packet. | It is FFS whether this field is needed in V2X. |
| Resource assignment | Indicate the time and frequency resources for the associated data (scheduling or unbooking). This includes retransmissions of the same TB. | - : number of (re)transmission that an SA can schedule (FFS) [2][6]. - : max time horizon for time scheduling of (re)tx of the TB scheduled by the SA [2]. Length is FFS. - : width of the data subband [7] in terms of resource blocks. Note that the above formula gives an upper bound on the number of bits needed. The actual number can be smaller subject to optimization (FFS the details). |
| Time offset resource booking | Indicates the time offset to the next TB potential transmission (for resource booking, i.e., in [2]) - For mode-1: this value is granted by the eNB. - For mode-2: this value is calculated by the UE, as specified in [2]. | *(3-4) |
| Modulation and coding scheme | Indicate format used for encoding the data that is being scheduled. | 5 |
| Resource booking and dropping | Used for: - Booking a new resource in the future for future TB transmission whose time index is indicated by the field “Time offset resource booking”, and - Dropping the coming booked resource, see [2] for details. | 2 |
| Retransmission | Indicate the RV of the scheduled packet, in case of retransmission | Length and need are FFS. |

We conclude this section by summarizing our observations and proposals:

Proposals:

- Contents of the PSCCH should be revised to support V2X. Details of the new contents are given in Table 1.

- Contents of the downlink control information to support V2X over PC5 should be revised accordingly.

# Conclusion

In this contribution we discussed contents of PSCCH for V2X over PC5.

Proposal:

- RAN1 selects Alt.1 in the following:

  - Alt.1 Priority information is signaled in SCI

  - Alt.2 Priority information is not signaled in SCI

Observation:

- The following aspects need to be considered in designing contents of SA:

  - Scheduling a TB and booking resources for a future TB.

  - Priority of traffic

  - Mode of resource allocation

Proposals:

- Contents of the PSCCH should be revised to support V2X. Details of the new contents are given in Table 1.

- Contents of the downlink control information to support V2X over PC5 should be revised accordingly.

# Reference

- R1-165246,	Contents of Downlink Control Information for V2V over PC5,	Ericsson

- R1-165276, Discussion on V2X PC5 Scheduling, Resource Pools and Resource Patterns, Ericsson

- R1-165266,	Sensing and autonomous resource selection, Ericsson

- R1-165263,	Resource reselection, Ericsson

- R1-165260,	Priority handling for V2V over PC5, Ericsson

- R1-165262,	Relation between SA and data for V2V over PC5, Ericsson

- R1-165273,	Subbands and measurements for sensing for V2V over PC5,	Ericsson

# Appendix:  SCI format 0 in Rel-12 sidelink

SCI, as defined in 3GPP TS 36.212 version 12.4.0 , has the following fields:

- Frequency hopping flag (FH): 1 bit

This field indicates whether frequency hopping is applied to transmissions of the associated PSSCH.

- Resource block assignment and hopping resource allocation  (RA):  bits (from 5 bits for 6 RBs to 13 bits for 100 RBs)

This field contains information about the number of allocated resource blocks for the associated PSSCH and their location (frequency domain information). If frequency hopping is applied, it provides further information about the hopping configuration

- Time resource pattern (TRP):  7 bits

This field indicates the subframes of the associated PSSCH, according to defined patterns (time domain information).

- Modulation and coding scheme (MCS): 5 bits

This field indicates the MCS used for the associated PSSCH.

- Timing advance indication  (TA):  11 bits

For sidelink transmission mode 1, certain transmitting UEs will use this field to indicate sidelink reception timing adjustment. For sidelink transmission mode 2 this field is set to zero.

- Group destination ID (GID) – 8 bits as defined by higher layers

Using this field a receiving UE can identify on the physical layer whether the data packet is intended for it. For each PSCCH resource configuration associated with sidelink transmission mode 1 and mode 2, a UE configured by higher layers to detect SCI format 0 on PSCCH shall attempt to decode the PSCCH according to the PSCCH resource configuration, and using the Group destination IDs indicated by higher layers. If the identifier does not match, they do not need to monitor sidelink channels until the next SC-Period.