3GPP TSG-RAN WG1#85 	R1-165322

Nanjing, China 23rd - 27th May 2016

Source: 	Ericsson

Title:	New TBS Designs Supporting UL 256QAM

Agenda Item:	6.2.8

Document for: 	Discussion and Decision

# Introduction

The Uplink Capacity Enhancements for LTE Work Item [1] specifies the introduction of 256QAM support for the PUSCH. The support of 256QAM shall be specified in a generic manner for both licensed and unlicensed spectrum. Focus of the work should be specific to the support of small cell deployments or deployments with eNB with many receive antennas. We provided our input in [2] and [3] toward the objective of

- Introduce new MCS table and signaling to support 256QAM in UL [RAN1]

The essential features of the design were agreed in RAN1 #84b [4]:

Agreements:

- The minimum TBS index for 16QAM is 11

- The minimum TBS index for 64QAM is 20

- The minimum TBS index for 256QAM is 30

- 3 entries are reserved for retransmissions

- The number of PUSCH MCS entries to support 256QAM is down-selected between

  - Alt 1: 6

  - Alt 2: 5

  - Alt 3: 4

Agreements:

- Reference number of REs available in PUSCH for UL 256QAM TBS design is 144 REs/PRB

- The maximum coding rate for PUSCH with 256QAM should not exceed 0.931

- FFS whether or not to introduce new TBS index(es) for PUSCH with 256QAM

  - Companies are encouraged to submit evaluations to RAN1#85

We discuss design principles for new UL 256QAM MCS entries in [5]. In this document, we provide further specific TBS designs supporting Rel-14 PUSCH 256QAM MCS table.

# TBS Design for Rel-14 UL 256 QAM

## TBS design principle

In Rel-12, the 36x110 TBS table for one spatial layer is populated with 186 unique transport block sizes (TBS). To support multiple spatial layers, 51 additional unique TBS are defined for one-layer to two-layer, one-layer to three-layer and one-layer to four-layer TBS translation. In total, there are 237 unique TBS defined in Rel-12. The design allows the TBS tables to be very efficiently stored using 8-bit indices to the unique TBS list instead of 19-bit integers representing the TBS directly.

The reduction of potentially 36x110=2970 TBS to only 186 TBS in the one layer table is achieved by allowing the actual code rate to deviate from the target code rate within a 4% band (the last MCS bounded by no more than 0.931 code rate). This is for example shown in Figure 1 for the Rel-8 64QAM MCS 17-28 and Rel-12 256QAM MCS 20-27.

Observation

By allowing the actual code rate to deviate from the target code rate within a 4% band, there are in total 237 unique TBS defined in Rel-12 enabling efficient TBS table storage.

Figure 1 Code rate inaccuracy in percentage for Rel-8 DL 64QAM and Rel-12 DL 256QAM.

## TBS design for single-layer UL 256QAM

Based on extensive analysis presented in [5], two new TBS rows with the following target code rates are identified for Rel-14 256QAM UL support:

| TBS Index | Modulation Order | r x 1024 | SE |
| --- | --- | --- | --- |
| 34 | 8 | 829.5417 | 6.4808 |
| 35 | 8 | 948.0000 | 7.4063 |

By allowing deviation from the target code rates within a band of 4% inaccuracy, the 2x110 TBS can be designed without introducing any new TBS for UL 256QAM one spatial layer support, as given in Table 1. The peak data rate in Table 1 is set by the largest TBS for NPRB = 100 according to Rel-8 LTE design principles. Actual code rates and code rate inaccuracy in percentages is shown in Figure 2. The highest actual code rate for TBS 35 is 0.9306. The TBS 35 code rate for 100 PRB is 0.920. It can be seen in the figure that the deviation is within a band of 4% similar to the designs for Rel-8 and Rel-12 DL.

Figure 2 Actual code rate and code rate inaccuracy in percentage for Rel-14 UL 256QAM.

Proposal 1

By allowing deviation from the target code rate within a band of 4%, the new TBS rows as provided in Table 1 can be designed without introducing any new transport block sizes for one spatial layer UL 256QAM support in Rel-14.

Table 1 Two new TBS rows for UL 256QAM support

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34 | 904 | 1864 | 2792 | 3752 | 4584 | 5544 | 6456 | 7480 | 8248 | 9144 |
| 35 | 1032 | 2088 | 3112 | 4264 | 5160 | 6200 | 7224 | 8504 | 9528 | 10296 |
|  | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| 34 | 10296 | 11064 | 12216 | 12960 | 14112 | 14688 | 15840 | 16416 | 17568 | 18336 |
| 35 | 11448 | 12576 | 13536 | 14688 | 15840 | 16992 | 17568 | 19080 | 19848 | 20616 |
|  | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| 34 | 19848 | 20616 | 21384 | 22152 | 22920 | 24496 | 25456 | 26416 | 27376 | 27376 |
| 35 | 22152 | 22920 | 24496 | 25456 | 26416 | 27376 | 28336 | 29296 | 30576 | 31704 |
|  | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
| 34 | 28336 | 29296 | 30576 | 31704 | 32856 | 32856 | 34008 | 35160 | 36696 | 36696 |
| 35 | 32856 | 34008 | 35160 | 35160 | 36696 | 37888 | 39232 | 39232 | 40576 | 42368 |
|  | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| 34 | 37888 | 39232 | 40576 | 40576 | 42368 | 42368 | 43816 | 43816 | 45352 | 46888 |
| 35 | 42368 | 43816 | 45352 | 46888 | 46888 | 48936 | 48936 | 51024 | 51024 | 52752 |
|  | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 |
| 34 | 46888 | 48936 | 48936 | 51024 | 51024 | 52752 | 52752 | 52752 | 55056 | 55056 |
| 35 | 52752 | 55056 | 55056 | 57336 | 57336 | 59256 | 59256 | 61664 | 61664 | 63776 |
|  | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 |
| 34 | 57336 | 57336 | 59256 | 59256 | 59256 | 61664 | 61664 | 63776 | 63776 | 63776 |
| 35 | 63776 | 63776 | 66592 | 66592 | 68808 | 68808 | 71112 | 71112 | 71112 | 73712 |
|  | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 |
| 34 | 66592 | 66592 | 68808 | 68808 | 68808 | 71112 | 71112 | 73712 | 73712 | 73712 |
| 35 | 75376 | 76208 | 76208 | 78704 | 78704 | 78704 | 81176 | 81176 | 81176 | 84760 |
|  | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 |
| 34 | 75376 | 76208 | 76208 | 78704 | 78704 | 78704 | 81176 | 81176 | 81176 | 84760 |
| 35 | 84760 | 84760 | 87936 | 87936 | 87936 | 90816 | 90816 | 93800 | 93800 | 93800 |
|  | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 |
| 34 | 84760 | 84760 | 87936 | 87936 | 87936 | 87936 | 90816 | 90816 | 90816 | 93800 |
| 35 | 93800 | 97896 | 97896 | 97896 | 97896 | 101840 | 101840 | 101840 | 105528 | 105528 |
|  | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 |
| 34 | 93800 | 93800 | 93800 | 97896 | 97896 | 97896 | 97896 | 101840 | 101840 | 101840 |
| 35 | 105528 | 105528 | 105528 | 105528 | 105528 | 105528 | 105528 | 105528 | 105528 | 105528 |

## TBS design for four-layer UL 256QAM

To support 256QAM transport blocks mapped to four-layer spatial multiplexing layers, a new table needs to be designed for one-layer to two-layer translation for TBS = 101840 and 105528. Allowing mapping inaccuracy of ±0.44%, the additional one-layer to two-layer mappings can be supported without introducing new TBS, as given in Table 2. The code rate for 211936 is 0.9236.

Table 2 TBS mapping table for one-layer to two-layer

| TBS_L1 | TBS_L2 |
| --- | --- |
| 101840 | 203704 |
| 105528 | 211936 |

Proposal 2:

One-layer to two-layer TBS mapping is proposed in Table 2. No new TBS needs to be introduced.

# Conclusion

Based on extensive analysis presented in this document, we make the following observation and proposals.

Observation

By allowing the actual code rate to deviate from the target code rate within a 4% band, there are in total 237 unique TBS defined in Rel-12 enabling efficient TBS table storage.

Proposal 1

By allowing deviation from the target code rate within a band of 4%, the new TBS rows as provided in Table 1 can be designed without introducing any new transport block sizes for one spatial layer UL 256QAM support in Rel-14.

Proposal 2:

One-layer to two-layer TBS mapping is proposed in Table 2. No new TBS needs to be introduced.

# References

- RP-160664 New Work Item on Uplink Capacity Enhancements for LTE enhanced LAA for LTE, 3GPP RAN WG #71;  Ericsson, CMCC

- R1-163345 MCS Table Design for UL 256QAM, Ericsson

- R1-163346 On the Performance of UL 256QAM, Ericsson

- 3GPP RAN1 #84b Chairman’s Notes

- R1-165321 Design Principles for New UL 256 QAM MCS Entries