3GPP TSG RAN WG1 Meeting #85	R1-165372

Nanjing, China, 23rd – 27th March 2016

Source:	Fraunhofer HHI

Title:	Notation of log-normal distributed random variables

Agenda Item:	7.2

Document for:	Discussion

# Introduction

At RAN#69 meeting, the study item (SI) on channel model for frequency spectrum above 6GHz was approved, cf. [1]. At 3GPP RAN1#84 meeting RAN1 started working on the SI. Discussion topics included requirements and scenarios, channel model methodology, as well as additional features. It was agreed that the targeted channel model should use the same methodology as the 3GPP-3D channel model described in TR 36.873, cf. [2].

This contribution should motivate for using a proper notation of mean and standard deviation of logarithmized parameters.

# Problem description

Currently it is quite common to denote the large-scale parameter mean and standard deviation simply by μX and σX, where X stands for the individual large-scale parameter. In addition a unit is provided like log10(s) or log10(deg) or dB. While giving the mean and standard deviation of a parameter in dB is quite unproblematic, the first two options might lead to misunderstandings, since it is not obvious whether the presented value is the logarithmized mean value lgμX or the mean logarithmized value μlgX.

Sometimes it happens that in addition to the mean values given in units of log10(s) or log10(deg) there are mean values presented in s or deg using the same symbols as mentioned above. Here we end up with the same problem – it is not obvious whether the mean value μX is given or the mean of the logarithmized value in linear scale 10μlgX.

The relationship of these two mean values is depicted in Figure 2-1 and Figure 2-2. While Figure 2-2 shows typical ranges for the delay spread mean and standard deviation values, Figure 2-1 shows typical ranges for the angular spread parameters.

Figure 2-1:Relationship between μX and μlgX given the standard deviation σlgX

Figure 2-2: Relationship between μX and μlgX given the standard deviation σlgX

# Background

Defining a normalized random variable X=Xxo, by

X~lgNμlgX, σlgX2≡lgX~NμlgX, σlgX2

with

μlgX=ElgX

being the mean of the logarithmized random variable X and

σlgX=VarlgX

being the standard deviation of the logarithmized random variable X.

The mean and standard deviation of random variable X are given by

μX=EX=10μlgX+σlgX2ln102

and

σX=VarX=μX10σlgX2ln10-1

Since

μX=EX=EXx0=EXx0=μXx0

it follows that

μX=EX=x010μlgX+σlgX2ln102

and with

σX=VarX=VarXx0

it follows that

σX=VarX=x0μX10σlgX2ln10-1=μX10σlgX2ln10-1

Given the mean and standard deviation of random variable X, the mean of its logarithm lgX is given by

μlgX=lgμX2σX2+μX2

while the standard deviation is given by

σlgX=lgσXμX2+1ln10

# Proposed solution

In order to prevent misunderstandings the following is proposed:

Proposal 1: Adopt the following notation for mean (μlgX=meanlog10X) and standard deviation (σlgX=stdlog10X) of logarithmized parameters.

Proposal 2: When presenting mean values of logarithmized parameters in linear scale, clarify whether 10μlgX  or μX is given.

##### Example 1: Delay Spread

After defining the delay spread in linear scale as ds, the normalized delay spread as DS=ds1s, and the normalized & logarithmized delay spread as lgDS=log10ds1s, the mean and standard deviation of the normalized &logarithmized delay spread can be denoted as μlgDS and σlgDS, while the mean and the standard deviation of the delay spread in linear scale can be denoted as μds and σds.

##### Example 2: Angular Spread of Departure

After defining the angular spread of departure in linear scale as asd, the normalized angular spread of departure as ASD=asd1deg, and the normalized & logarithmized angular spread of departure as lgASD=log10asd1deg, the mean and standard deviation of the normalized &logarithmized angular spread of departue can be denoted as μlgASD and σlgASD, while the mean and the standard deviation of the delay spread in linear scale can be denoted as μasd and σasd.

# Conclusion

This contribution reported on possible errorneous interpretation of large-scale parameters due to improper notation of mean and standard deviation.

Proposal 1: Adopt the following notation for mean (μlgX=meanlog10X) and standard deviation (σlgX=stdlog10X) of logarithmized parameters.

Proposal 2: When presenting mean values of logarithmized parameters in linear scale, clarify whether 10μlgX  or μX is given.

# References

- RP-151606, “New SID Proposal: Study on channel model for frequency spectrum above 6 GHz”, Samsung, Nokia Networks, 3GPP RAN Plenary#69.

- 3GPP TR 36.873, “Study on 3D channel model for LTE,” v12.2.0, June 2015.