3GPP TSG RAN WG1 Meeting #85bis		R1-165428

Nanjing, China, 23th - 27th May 2016

Source:	CATT

Title:	Performance evaluation of binary LDPC codes

Agenda Item:	7.1.5.1

Document for:	Discussion/Decision

Introduction

In the email discussion after RAN1#84, Channel coding candidates for 5G new RAT data transmission were discussed and the following were agreed [1]:

- Part 1: Channel coding candidates for 5G new RAT data transmission

- Candidates for 5G new RAT data transmission are identified as the following

  - LDPC code

  - Polar code

  - Convolutional code (LTE and/or enhanced convolutional coding)

  - Turbo code (LTE and/or enhanced turbo coding)

- Selection of 5G new RAT channel coding scheme(s) will consider

  - Performance

  - Implementation complexity

  - Latency (Decoding/Encoding)

  - Flexibility (e.g., variable code length, code rate, HARQ (as applicable for particular scenario(s)))

- Part 2: Coding Candidates and Initial Simulation Assumptions

- Coding Candidates

  - Identified channel coding schemes for each usage scenario

| eMBB | mMTC | URLLC |
| --- | --- | --- |
|  | Convolutional codes | Convolutional codes |
| LDPC | LDPC | LDPC |
| Polar | Polar | Polar |
| Turbo | Turbo | Turbo |

  - Common simulation assumptions are required to evaluate theoretical performance of proposed coding schemes

  - Selection of the coding scheme should also consider various other aspects

- Initial Simulation Assumptions

  - Focus mainly on the BLER performance of candidate coding schemes.

  - Evaluate performance of coding schemes with similar code rates and block sizes

  - Exact code constructions should be provided

  - Encoding/decoding complexity of the adopted algorithms should be described

- Part 3: Coding Candidates and Simulation Assumptions

- Simulation assumptions:eMBB

  - Evaluate the block error rate (BLER) performance versus SNR

| Channel* | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK, 64 QAM |  |  |  |
| Coding Scheme | Turbo | LDPC | Polar |  |
| Code rate | 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |  |  |  |
| Decoding algorithm** | Max-log-MAP | min-sum |  | List-X |
| Info. block length*** (bits w/o CRC) | 100, 400, 1000, 2000, 4000, 6000, 8000 Optional(12K, 16K, 32K, 64K) |  |  |  |

* Fading channels will be simulated in the next stage

** These algorithms are starting points for further study. Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated)

*** At least these info. block length and code rate shall be evaluated. Other info. block lengths and code rates are not precluded. Similar info. and encoded block lengths should be used for the evaluation. Total coded bits = info. Block length/code rate. Note: these info. block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis.

  - General guidelines

- Existing code constructions can be used for evaluation

- Whenever feasible, performance comparison should adopt coding constructions with matching computational complexities

- Simulation assumptions: URLLC and mMTC

  - Evaluate BLER performance versus SNR

| Channel* | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK, 16 QAM |  |  |  |
| Coding Scheme | Convolutional codes | LDPC | Polar | Turbo |
| Code rate | 1/12, 1/6, 1/3 |  |  |  |
| Decoding algorithm** | List-X Viterbi | min-sum | List-Y | Max-log-MAP |
| Info. block length*** (bits w/o CRC) | 20, 40, 200, 600, 1000 |  |  |  |

* Fading channels will be simulated in the next stage

** These algorithms are starting points for further study. Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated)

*** At least these info. block length and code rate shall be evaluated. Other info. block lengths and code rates are not precluded. Similar info. and encoded block lengths should be used for the evaluation. Total coded bits = info. Block length/code rate. Note: these info. block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis.

  - General guidelines

- Existing code constructions can be used for evaluation

- Whenever feasible, performance comparison should adopt coding constructions with matching computational complexities

- BLER simulations down to 10-4 is recommended (to observe the error floor) for URLLC

In this document, performance evaluation of binary LDPC codes for eMBB scenarios are  provided.

Evaluations and discussions

  - Evaluation methodologies and assumptions

The evaluations are performed based on the agreed simulation assumptions in [2], in which some parameters are left to be determined by each company.  The detailed simulation parameters are  as the following

- Simulated cases

  - CASE 1: AWGN Channel, QPSK Modulation , design code rate={1/2,2/3}, information block length ={4000, 6000},  LTE Turbo code with Max-log-MAP decoding algorithm

  - CASE 2: AWGN Channel, 64QAM Modulation , design code rate={1/2,2/3}, information block length ={4000, 6000},  LTE Turbo code with Max-log-MAP decoding algorithm

  - CASE 3: AWGN Channel, QPSK Modulation , design code rate={1/2,2/3,3/4}, information block length ={4000, 6000,8000},  SC-LDPC code with min-sum sliding window decoding algorithm

  - CASE 4: AWGN Channel, 64QAM Modulation , design code rate={1/2,2/3,3/4}, information block length ={4000, 6000,8000},  SC-LDPC code with min-sum sliding window decoding algorithm

  - CASE 5: AWGN Channel, QPSK Modulation , design code rate={5/6,8/9}, information block length ={324,576,1152,2304,4608,6912,9216},  LDPC code with min-sum decoding algorithm

  - CASE 6: AWGN Channel, QPSK Modulation , design code rate={1/2,2/3,7/9,5/6,8/9}, information block length ={288,270,252,216,162},  LDPC code with min-sum decoding algorithm

Other simulation parameters are listed in Table A-1 and Table A-2

  - Evaluation results

In this section we provide evaluation results with the following performance metrics

- Performance

- Implementation complexity

- Latency (Decoding/Encoding)

- Flexibility

It is noted that the first metrics are used to evaluate the performance of SC-LDPC codes which is a familyof LDPC codes. Other performance metrics, such as flexibility on variable code length and code rate, are given in the last metrics.

- Performance

### Figure 1: Performance of SC-LDPC codes

- Implementation complexity

- Encoding complexity

LDPC codes can be  encoded using the parity-check matrix directly by performing preprocessing prior to encoding (transforming it into upper triangular form and using back substitution). The encoding complexity can be simplified by using specific structures of the parity-check matrix, such as lower triangular structure.

- Decoding complexity

Decoding complexities of  binary LDPC codes and SC-LDPC codes using  sum-product algorithm and min-sum algorithm(MSA) are summarized in Table 1.  We denote the average iteration number of sliding window decoding algorithm as ISC,  the average iteration number of SPA and min-sum algorithm as IB, the average iteration number of Max-Log-Map algorithm as Iturbo. The average iteration number of the min-sum algorithms for SC-LDPC codes is shown in Figure 2.

Table 1: Decoding complexity for different codes

| Decoding algorithm | Multiplication | Addition | Comparison | Look up table |
| --- | --- | --- | --- | --- |
| sum-product algorithm for SC-LDPC |  | IB(Nw(2dv-1)+12Mw (dc-2)) | 3LISCMw(dc-2) | LISC Mw (dc-2) |
| sum-product algorithm for B-LDPC |  | IB(N(2dv-1)+ 12M(dc-2)) | 3IBM(dc-2) | 6IBM (dc-2) |
| min-sum algorithm for SC-LDPC |  | LISCNw(2dv-1) | 3LISCMw(dc-2) |  |
| min-sum algorithm for B-LDPC |  | IB N(2dv-1) | 3IBM (dc-2) |  |

Figure 2: Average iteration number of the min-sum algorithms for SC-LDPC codes

Table 2 shows that the min-sum algorithm would require Nw(2dv-1) additions and 3Mw(dc-2) comparisons with a one iteration of decoding window. Obviously, B-LDPC has decoding complexities of O(k).  For SC-LDPC code, the complexity rises linearly in K(K≈k).  For large packet size, the complexity is acceptable for eMBB.

- Latency

Since the BP decoder can start decoding only after the entire decoding sequence is received, we denote  the total decoding latency of  BP decoder as TBP.  TBP is given by TBP= TREE+ TDE, TREE is the time taken to receive the entire decoding sequence and TDE is the time need to decode the codeword.

Figure 3 shows a parity-check matrix H of SC-LDPC codes built from a base matrix B and a sliding window consisting of WPb rows and WPc columns, ms+1≤W≤L+ms.   The symbol ms is the syndrome former memory which can be viewed as the memory of the parity-check matrix[2-4].  At the first time instant, t=1, the decoder performs belief propagation to decode all of the first Pc symbols within the window, which is called the targeted symbols.  The decoding latency of the Pc targeted symbols with sliding window decoder is given by TWD=TRET+ TDE(W), where TRET is the time taken to receive all the symbols required to decode the Pc targeted symbols, and TDE is the time taken to decode the targeted symbols. The relation of  parameters TBP and TWD are  as  follows,

It is  noted that the decoding latency and decoding complexity of SC-LDPC codes should be similar with the given information length k of the binary LDPC block codes., We denote  the information length in decoding window as K, K is given by K=RWPc=k.

Figure 3: Sliding window decoding algorithms of SC-LDPC codes

- Flexibility

The new channel coding scheme should have a sufficient flexibility on variable code length and code rate.

We consider following aspects of the binary LDPC codes schemes with respect to the flexibility of NR system design.

- Flexibility on variable code length

Figure 4: Performance of binary LDPC codes with different code length under fixed code rate

- Flexibility on code rate

Figure 5: Performance of binary LDPC codes with different code rates under fixed code length

The QC-LDPC codes are considered in which parity-check matrices are arrays of circulant permutation  matrices (CPMs).  We construct the base matrix of binary LDPC codes with the same size with fixed code rate.  By changing the size of CPM, the LDPC codes can be constructed with different code length. With augmenting method , we can construct the LDPC codes with different code rates  under the fixed code structure. The performances are shown in Figures 4 and  5.We can also using extending , puncturing and shortening methods to reconstruct  a known code to fit the desired length of a specific application.  The binary LDPC codes have a sufficient flexibility on variable code length and code rate.

Looking at the above results, we have the following observations  for eMBB scenarios.

- The results show that (4,8) regular SC-LDPC codes and Turbo codes perform almost identically for rate Rd=1/2 and information latency length K closed to 4000 bits and 6000 bits. For (4,8) regular SC-LDPC codes, visable gain can be achieved by raising the code rate from 1/2 to 2/3.

- SC-LDPC code has the similar decoding latency comparing with that of the binary LDPC block code.   The complexity  is acceptable for large code block for eMBB deployment scenarios.

- Binary LDPC codes have sufficient flexibility in constructing variable code length and code rate.

Conclusions

In this contribution, we provide the performance evaluation results of binary LDPC codes for eMBB deployment scenario. According to the evaluation results, we observe the binary LDPC code has better performance with high code rate.   The encoding and decoding complexity is feasible for large code block  for eMBB deployment scenario

References

- “Chairman's Notes RAN1_84bis”,  3GPP TSG RAN WG1 #84bis, Busan, Korea, April 11-15, 2016.

- A. Jiménez-Felström and K. S. Zigangirov, Time-varying periodic convolutional codes with low-density parity-check matrix[J]. IEEE Trans. Inf. Theory, 1999,45(9): 2181–2191.

- A. Pusane, A. J. Feltstrom, A.Sridharan, et al., Implementation aspects of LDPC convolutional codes[J]. IEEE Trans. Commun., 2008,56(7):1060-1069

- A. R. Iyengar,M. Papaleo,P. H. Siegel, et al., Windowed Decoding of Protograph-Based LDPC Convolutional Codes Over Erasure Channels[J]. IEEE Transactions on Information Theory, 2012, 58(4) , 2303-2320.

- TR38.913,"StudyonScenariosand Requirements for Next Generation Access Technologies," v0.3.0.

- R1-162309, “Channel coding for the 5G new radio”, CATT

Appendix

  - Simulation assumption

Table A-1: Simulation assumptions of SC-LDPC codes for eMBB scenario

| Parameter | Assumption |
| --- | --- |
| Information length (denote k) | 4000bits, 6000 bits, 8000bits |
| Termination length(denote L) | L=60 |
| Design Code rate(denote Rd) | 1/2, 2/3, 3/4 |
| The number of variable nodes( denote dv) | 4 |
| The number of check nodes( denote dc) | Rd=1/2 dc =8 Rd=2/3 dc =12 Rd=3/4 dc =16 |
| Sliding window size(denote W) | 8 |
| Syndrome former memory(denote ms) | 2 |
| Columns of base matrix(denote c) | Rd=1/2:c=6 Rd=2/3: c=9 Rd=3/4: c=12 |
| Row of base matrix(denote b) | b=3 |
| Exact Code rate(denote R) | R= (Lc-(L+ms)(c-b))/ (Lc) Rd=1/2 L=60 c=6 b=3 ms=2:R=0.483 Rd=2/3 L=60 c=9 b=3 ms=2:R=0.656 Rd=3/4 L=60 c=6 b=3 ms=2:R=0.742 |
| CPM size(denote P) | Rd=1/2 k=4000:P=166 Rd=1/2 k=6000:P=250 Rd=1/2 k=8000:P=330 Rd=2/3 k=4000:P=82 Rd=2/3 k=6000:P=126 Rd=2/3 k=8000:P=166 Rd=3/4 k=4000:P=58 Rd=3/4 k=6000:P=82 Rd=3/4 k=8000:P=112 |
| Columns of Low-density parity-check matrix(denote N) | N= LcP Rd=1/2 L=60 c=6 P=166: N=59760 Rd=1/2 L=60 c=6 P=250: N=90000 Rd=1/2 L=60 c=6 P=330: N=118800 Rd=2/3 L=60 c=9 P=82: N=44280 Rd=2/3 L=60 c=9 P=126: N=68040 Rd=2/3 L=60 c=9 P=166: N=89640 Rd=3/4 L=60 c=12 P=58: N=41760 Rd=3/4 L=60 c=12 P=82: N=59040 Rd=3/4 L=60 c=12 P=112: N=80640 |
| Columns of Low-density parity-check matrix in window(denote Nw) | Nw=cPW Rd=1/2 W=8 c=6 P=166: N=7968 Rd=1/2 W=8 c=6 P=250: N=12000 Rd=1/2 W=8 c=6 P=330: N=15840 Rd=2/3 W=8 c=9 P=82: N=5904 Rd=2/3 W=8 c=9 P=126: N=9072 Rd=2/3 W=8 c=9 P=166: N=11952 Rd=3/4 W=8 c=12 P=58: N=5568 Rd=3/4 W=8 c=12 P=82: N=7872 Rd=3/4 W=8 c=12 P=112: N=10752 |
| Rows of Low-density parity-check matrix(denote M) | M= (L+ ms)bP Rd=1/2 L=60 b=3 P=166 ms=2: M =30876 Rd=1/2 L=60 b=3 P=250 ms=2: M =46500 Rd=1/2 L=60 b=3 P=330 ms=2: M =61380 Rd=2/3 L=60 b=3 P=82 ms=2: M =15252 Rd=2/3 L=60 b=3 P=126 ms=2: M =23436 Rd=2/3 L=60 b=3 P=166 ms=2: M =30876 Rd=3/4 L=60 b=3 P=58 ms=2: M =10788 Rd=3/4 L=60 b=3 P=82 ms=2: M =15252 Rd=3/4 L=60 b=3 P=112 ms=2: M =20832 |
| Rows of Low-density parity-check matrix in window (denote Mw) | M= bPW Rd=1/2 W=8 b=3 P=166: M =3984 Rd=1/2 W=8 b=3 P=250: M =6000 Rd=1/2 W=8 b=3 P=330: M =7920 Rd=2/3 W=8 b=3 P=82: M =1968 Rd=2/3 W=8 b=3 P=126: M =3024 Rd=2/3 W=8 b=3 P=166: M =3984 Rd=3/4 W=8 b=3 P=58: M =1392 Rd=3/4 W=8 b=3 P=82: M =1968 Rd=3/4 W=8 b=3 P=112: M =2688 |
| Decoding constraint length(denote vs) | vs = (ms+1) Pc |
| Max iteration number (denote Imax) | Imax=100 |
| Min iteration number (denote Imin) | Imin=10 |
| Information length in decoding window(denote K) | K= RWPc R=0.483W=8 P=166 c=6:K=3984 bits R=0.483W=8 P=256 c=6:K=6000 bits R=0.483W=8 P=330 c=6:K=7920 bits R=0.656W=8 P=82 c=9:K=3936 bits R=0. 656W=8 P=126 c=9:K=6048 bits R=0. 656W=8 P=166 c=9:K=7968 bits R=0.742W=8 P=58 c=12:K=4176 bits R=0. 742W=8 P=82 c=12:K=5904 bits R=0. 742W=8 P=112 c=12:K=8064 bits |

Table A-2 Simulation assumptions of Turbo codes for eMBB scenario

| Parameters | Assumptions |
| --- | --- |
| Information length (denote k) | 6016 bits |
| State number(denote S) | S=8 |
| Branch number(denote Bnum) | Bnum=2 |
| Decoding iteration number (denote Iturbo) | Iturbo=8 |
| Exact Code rate(denote R) | Rd=1/2 k=4032: R=0.4993 Rd=1/2 k=6016: R=0.4995 Rd=2/3 k=4032: R=0.6653 Rd=2/3 k=6016: R=0.6658 |