3GPP TSG-RAN WG1 #85	R1-165453

Nanjing, China, 23th – 27th May 2016

Agenda item:	7.1.5.1

Source: 	MediaTek

Title: 	Discussion on LDPC code design and performance evaluation

Document for:	Discussion/Decision

# Introduction

In RAN1 84bis meeting, the different channel coding schemes have been presented and discussed for NR. WF on Channel Coding Evaluation for 5G New Radio has been agreed in [1] as the guidance for the next step on study. In this document, we provide some analysis and propose Rate Compatible low-density parity check (LDPC) codes as candidate for next generation channel codec. The codebook is generated based on the enhanced LDPC code structure and the corresponding simulation results are based on the [1].

# Consideration on channel codec for NR

A good channel codec must meet following perspectives: flexible message length, compatible code rate with HARQ IR capability, competitive performance, low complexity encoder and low complexity decoder. The LDPC code for NR should  meet all above requirements.

Table1: characteristic comparison of generic protograph LDPC, 802.11n LDPC and NR LDPC codes

| characteristic | Generic Protograph LDPC | LDPC in 802.11n | LDPC Code for NR |
| --- | --- | --- | --- |
| Flexible message length | V | V | V |
| Compatible code rate with HARQ IR | ? | X | V |
| Competitive performance | ? | V | V |
| Low complexity encoder | ? | V | V |
| Low complexity decoder | V | V | V |

# A multi-edge, raptor-like LDPC code

To meet all above requirements, enhanced LDPC code is proposed and described for discussion. The protomatrix serves as a blueprint for constructing LDPC codes of different size whose performance can be predicted by analyzing the protomatrix. To fit specified info bit size, replace every 1 in the protomatrix with a ZxZ matrix which is the cyclic shift of the identity matrix of size ZxZ. But for high rate code CR = 8/9, it is hard to find a protomatrix with good SNR threshold. By introducing multi-edge for high code rate, the SNR threshold can be significantly improved. Also, we found that the proposed raptor-like LDPC codes perform remarkably good and have almost the same performance as 802.11n which is directly optimized on specific code rate. The multi-edge, rate-compatible and raptor-like [2] LDPC protomatrix structure is:

Where  is a multi-edge matrix with highest code rate  and  is a specified random 1s matrix. An example of a 33x49 protomatrix with N = 2, M1 = 18 and M2 = 31 is given below. Note that in the upper protomatrix illustration, the number in the matrix denotes the edge number and in the lower H matrix illustration with a lifting value of Z. -1 denotes an all zero matrix of size Z and a non-negative number denotes a right-circular-shifted size-Z identity matrix with the specified amount of shift (thus 0 denoting a size-Z identity matrix).

33x49 protomatrix

2x18 Multi-edge matrix

Apply shift coefficient for each element with lifting value Z = 63

Multi-edge matrix value

Figure 1: raptor-like LDPC code construction with specified shift coefficient

Proposal 1: Multi-edge, raptor-like LDPC codes should be a candidate of 5G SI of channel coding.

# Zero-Padding for flexible block size and code rate support

The LDPC code introduced in chapter 3 still supports limited information block sizes and code rates. To have flexible design, zero padding is introduced. With carefully codebook design, the performance with zero padding is still competitive. With zero padding, hardware complexity can be reduced, because some variable nodes can be removed in the BP decoder. The power consumption can also be lower. Below figures show an example to support code of CR=14/49 by zero-padding on code of CR=16/49 code. For our current codebook, we can support code rate down to CR = 1/12.

Figure 2: Illustration of zero padding to generate low rate codes

Proposal 2:  Provide LDPC code with wider range of code rate support and flexible block size design

# Codebook design

Owing to the flexible message length and code rate support, the proposed codebook can easily support LTE spec info bit size from 40 bits to 6144 bits. They have total 188 sets of info bits supporting code rate range from CR=1/12 to CR=8/9. Each size of info bit corresponds to specified value Z and padded bits P.

In the following table we show the codebook with design parameters Z and P based on eMBB simulation assumptions with info bits 100, 400, 1000, 2000, 4000, 6000 and 8000.  The protomatrix dimension is 33x49 with N = 2, M1 = 18 and M2 = 31.

Table2: codebook for eMBB simulation assumption

| K (info bits) | Z (lifting) | #padded bits | #padded blocks |
| --- | --- | --- | --- |
| 100 | 7 | 12 | 1.71 |
| 400 | 25 | 0 | 0.00 |
| 1000 | 63 | 8 | 0.13 |
| 2000 | 125 | 0 | 0.00 |
| 4000 | 250 | 0 | 0.00 |
| 6000 | 375 | 0 | 0.00 |
| 8000 | 500 | 0 | 0.00 |
| 12000 | 750 | 0 | 0.00 |

Proposal 3:  Provide a well designed and flexible LDPC codebook

# Simulation Results

In this section we compare the performance of the proposed LDPC codes and LTE Turbo codes. The LDPC codes are decoded by sum-product algorithm and the Turbo codes are decoded by the max-log-MAP algorithm with a scaling by 0.8 of the extrinsic LLRs for each information bit. We compare LDPC codes with 30 decoding iterations to Turbo codes with 8/10 decoding iterations. Based on the agreement of eMBB simulation assumptions[1]. We select QPSK modulation, info block length = 1k/6k bits and with various code rate.

For LDPC K(info bits) = 1000 bits, Turbo code info bit length is selected as 1024 bits (with 24 CRC bits) with 8 iterations, the simulation results are as following.

Figure 3: BLER performance for eMBB info bits = 1k

For LDPC K(info bits) = 6000 bits, Turbo code info bit length is selected as 6016 bits (with 24 CRC bits) with 10 iterations, the simulation results are as following.

Figure 4: BLER performance for eMBB info bits = 6k

It can be observed that good performance granularity can be achieved with flexible rate compatible design. LDPC code outperforms LTE Turbo code in above simulation scenarios at BLER of 1%. For all code rates the difference is about 0.1~0.3 dB.

# Conclusions

The following summarizes the proposals in this contribution.

Proposal 1: Multi-edge, raptor-like LDPC codes should be a candidate of 5G SI of channel coding.

Proposal 2: Provide LDPC code with wider range of code rate support and flexible block size design.

Proposal 3: Provide a well designed and flexible LDPC codebook.

# References

- R1-163757, WF on Channel Coding Evaluation for 5G New Radio

- T.-Y. Chen, K. Vakilinia, D. Divsalar, and R. D. Wesel, “Protograph-Based Raptor-Like LDPC Codes”, IEEE Trans. Com., May 2015.