3GPP TSG RAN WG1 Meeting #85	R1-165686

# Nanjing, China, 23rd -27th May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0251 | rev |  | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Scrambling Sequence for paging MPDCCH and PDSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell, Intel, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-05-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Currently TS36.211 does not explicity separate the Nacc (number of subframes that the same scrambling code is used) for either the paging MPDCCH or paging PDSCH. Therefore according to current specifications the value of Nacc used depends on the CEMode. Given that neither the network nor the UEs have an accurate view of the UE CEmode when the UE is in Idle mode, especially in use cases such as when MPDCCH is used to provide Direct Indication information to all idle BL/CE UEs, or when there’s multiplexing of paging records spanning multiple UEs, and from a robustness point of view, it is proposed that Nacc is fixed to 4 for paging messages (PDSCH & MPDCCH) in TS36.211. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text in 36.211 detailing the scrambling code for: - Downlink physical channels (section 6.3.1) - MTC physical downlink control channel (section 6.8B.5) is updated to explicitly mention paging. Note, that since paging only uses TM1/2, there is no need to update similar text in section 6.10.3 covering UE-specific reference signals associated with PDSCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment in UE decoding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.8B.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 6.3	General structure for downlink physical channels

This clause describes a general structure, applicable to more than one physical channel.

The baseband signal representing a downlink physical channel is defined in terms of the following steps:

-	scrambling of coded bits in each of the codewords to be transmitted on a physical channel

-	modulation of scrambled bits to generate complex-valued modulation symbols

-	mapping of the complex-valued modulation symbols onto one or several transmission layers

-	precoding of the complex-valued modulation symbols on each layer for transmission on the antenna ports

-	mapping of complex-valued modulation symbols for each antenna port to resource elements

-	generation of complex-valued time-domain OFDM signal for each antenna port

Figure 6.3-1: Overview of physical channel processing

### 6.3.1	Scrambling

For each codeword , the block of bits , where  is the number of bits in codeword  transmitted on the physical channel in one subframe, shall be scrambled prior to modulation, resulting in a block of scrambled bits according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised at the start of each subframe, where the initialisation value of  depends on the transport channel type according to

where  corresponds to the RNTI associated with the PDSCH transmission as described in clause 7.1 3GPP TS 36.213 [4].

For BL/CE UEs, the same scrambling sequence is applied per subframe to PDSCH for a given block of  subframes. For the  block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for PDSCH. The PDSCH transmission spans  consecutive subframes including invalid subframes where the PDSCH transmission is postponed..

For BL/CE UEs,

- if the PDSCH is carrying SIB1-BR

- else if the PDSCH is carrying SI message (except for SIB1-BR) or if the PDSCH transmission is associated with P-RNTI:

- for frame structure type 1 and  for frame structure type 2

- otherwise

- for UEs assuming CEModeA (according to the definition in section 12 of [4]) or configured with CEModeA

- for frame structure type 1 and  for frame structure type 2 for UEs assuming CEModeB (according to the definition in section 12 of [4]) or configured with CEModeB

Up to two codewords can be transmitted in one subframe, i.e., . In the case of single codeword transmission,  is equal to zero.

### 6.3.2	Modulation

For each codeword , the block of scrambled bits shall be modulated as described in clause 7.1 using one of the modulation schemes in Table 6.3.2-1, resulting in a block of complex-valued modulation symbols .

-----  Omitted -------

### 6.8B.2	Scrambling

Scrambling shall be perfomed according to Clause 6.8A.2 with EPDCCH repladed by MPDCCH except that the same scrambling sequence is applied per subframe to MPDCCH for a given block of  subframes and  is the MPDCCH set number. For an MPDCCH associated with a 2+4 PRB set as defined in [4],  is used to generate the scambling sequence for mapping to REs in 6 PRBs.

For the block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for the MPDCCH. The MPDCCH transmission spans  consecutive subframes, including invalid subframes where the MPDCCH transmission is postponed.

For BL/CE UEs,

- if the MPDCCH transmission is associated with P-RNTI:

- for frame structure type 1 and  for frame structure type 2

- otherwise

- for UEs assuming CEModeA (according to the definition in section 12 of [4]) or configured with CEModeA:

- for frame structure type 1 and  for frame structure type 2 for UEs assuming CEModeB (according to the definition in section 12 of [4]) or configured with CEModeB.

### 6.8B.3	Modulation

Modulation shall ber perfomed according to 6.8A.3 with EPDCCH replaced by MPDCCH.