3GPP TSG-RAN WG1 Meeting #85	R1-166043

Nanjing, China, May 23-27, 2016

Title:	LS on UL DM-RS for NPUSCH format 1

Release:	Rel-13

Work Item:	NB_IoT-Core

Source:	RAN WG1

To:	RAN WG4

Cc:

Contact Persons:

#### Name:	Matthew Webb

#### E-mail Address:	matthew.webb@huawei.com

Attachments:

1. Overall Description:

RAN1 wishes to respectfully inform RAN4 of an agreement to adopt the legacy LTE group hopping, as defined in 
Sec. 5.5.1.3, TS 36.211, for NB-IoT UL DMRS for NPUSCH format 1, as follows:

  - The group hopping pattern is revised from legacy LTE with 30 replaced by M in the following equations

  - In the above, M=30, 14, 12, and 16 for, respectively, 12-tones, 6-tone, 3-tone and single-tone cases.

  - The group-hopping pattern is reset:

    - Every subframe for multi-tone transmission

    - Every RU for single-tone transmission

  - The group-hopping for single-tone transmission only applies to the Hadmard sequence index

  - nIDPUSCH is not defined, and nIDRS =NIDNcell.

RAN1 has also agreed to introduce related RRC parameters for NB-IoT:

  - A new cell-specific RRC parameter, Group-hopping-enabled, to enable and disable the group hopping,

  - A new UE-specific RRC parameter, Disable-group-hopping to disable the function despite it is enabled by the cell-specific parameter unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure,

  - A new RRC parameter for the offset, ss (the same range of the one in LTE)

2. Actions:

RAN1 respectfully asks RAN4 to take this information into account in their work on NB-IoT.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #86	August 22-26, 2016		       Gothenburg, SE

TSG RAN WG1 Meeting #86bis	October 10-14, 2016		       Lisbon, PT