3GPP TSG RAN WG1 Meeting #86	R1-166172

Gothenburg, Sweden, August 22-26, 2016

Agenda Item:	7.2.11.1.3

Source:	Huawei, HiSilicon

Title:	Evaluation of NB-IoT downlink positioning

Document for:	Discussion and decision

# Introduction

In RAN#72, WID for enhancements of NB-IoT was approved, and the following was agreed on positioning functionality [1]:

Support of UTDOA or OTDOA:

- Study accuracy, UE complexity, UE power consumption for both UTDOA and OTDOA using NB-IoT and provide recommendation to RAN#73 on which one solution to adopt [RAN1]

  - 3GPP network operators are invited to provide inputs to RAN1#86 on their positioning requirements. Companies are encouraged to include both methods in their evaluations.

- Based on the study make a choice (either uplink positioning or OTDOA) during RAN#73

Accordingly, in this contribution we evaluate the accuracy of OTDOA positioning for NB-IoT.

# Link-level timing error performance

In this section, estimated timing error performances are evaluated by link-level simulation based on NSSS [2], under MCL levels of 144 dB, 154 dB and 164 dB for both standalone and in-band/guard-band operation modes. Simulation assumptions can be found in Table A.1 in the appendix, where a fading channel model of EPA 1Hz is considered. Considering the most challenging case of in-band operation, a larger measurement time duration i.e. 2400 ms is used so that the timing accuracy of 65ns can be achieved with a probability more than 67% for 164dB MCL. CDF curves of timing error for standalone and in-band/guard-band are shown in Figure 1 and Figure 2 respectively. Furthermore, the percentiles corresponding to the timing error threshold of 65ns are summarized in Table 1.

Figure 1 CDFs of downlink timing error performance for standalone mode

Figure 2 CDFs of downlink timing error performance for in-band/guard band mode

Table 1	CDF of 65 ns timing error

| Deployment mode | 144 dB MCL | 154 dB MCL | 164 dB MCL |
| --- | --- | --- | --- |
| In-band/guard band | 100% | 98.4% | 72.8% |
| Standalone | 100% | 100% | 98.3% |

# System-level positioning accuracy performance

In this section, positioning accuracy performance of OTDOA is evaluated for NB-IoT. For current evaluation, some of specific assumptions are listed in Table A.2 in the appendix while others can refer to another contribution where a framework for the study is discussed [3].

A general link-to-system mapping methodology is used for positioning accuracy evaluation where signal quality of different cells are observed from system-level simulation and timing error at each signal quality level per cell is measured by applying a UE receiver algorithm to link-level simulation.

Performance results of horizontal accuracy are shown in Figure 3. It can be seen that the percentiles corresponding to the accuracy threshold of 50 meters are 89% for stand-alone operation mode and 74% for in-band operation mode, further performances are summarized in Table 3.

Figure 3	CDFs of positioning error for OTDOA

Table 3	Positioning accuracy performance for OTDOA

|  | 40% | 50% | 70% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| In-band/guard band accuracy (meters) | 20.9 | 26.73 | 44.22 | 60.28 | 95.54 |
| Standalone accuracy (meters) | 13.68 | 16.19 | 25.82 | 34.3 | 53.21 |

# Conclusion

In this contribution, the positioning accuracy of OTDOA for both standalone operation mode and in-band/guard band operation modes are evaluated respectively. UE performs physical layer timing measurement based on NSSS of different cells. We have the following observations:

Observation 1: 50 meters horizontal accuracy can be achieved by OTDOA for 89% of UEs in stand-alone operation.

Observation 2: 50 meters horizontal accuracy can be achieved by OTDOA for 74% of UEs in in-band operation.

Observation 3: At the 67% level, OTDOA achieves 44 meters horizontal error for in-band/guard-band operation and 25 meters horizontal error for standalone operation.

# References

- RP-161324, “New work item proposal: Enhancements of NB-IoT,” RAN#72.

- [2]	R1-166173, “Design of NB-IoT downlink positioning,” Huawei, HiSilicon, RAN1#86, Gothenburg, Sweden

- [3]	R1-166171, “NB-IoT positioning scenarios and requirements,” Huawei, HiSilicon, RAN1#86, Gothenburg, Sweden

# Appendix

Table A.1	Link-level simulation assumptions for OTDOA

| Carrier frequency | 900MHz |
| --- | --- |
| BS Tx antenna | 1 for standalone 2 for in-band/guard-band |
| UE Rx antenna | 1 |
| Fading channel model | EPA 1Hz |
| Residual frequency error | Randomly chosen from the set {-50, 50} Hz |
| Reference signal | NSSS |
| Measurement time duration | 2400 ms for standalone and in-band/guard-band |

Table A.2	System-level assumptions for OTDOA

| Network synchronization | Synchronized without timing error |
| --- | --- |
| Mobility | Static |
| Fading channel mode | EPA 1Hz |
| Reference signal | NSSS |