3GPP TSG-RAN WG1 #86	R1-166272

22nd – 26th Aug 2016

Gothenburg, Sweden

Agenda item:	7.2.4.2.1

Source: 	Qualcomm Incorporated

Title: 	Non-precoded CSI-RS Codebooks for eFD-MIMO

Document for:	Discussion/Decision

# Introduction

In 3GPP RAN1#85, the following conclusions were made for non-precoded CSI-RS codebook for eFD-MIMO [1]:

Conclusion:

- Class A codebook enhancement:

  - Starting from Rel-13 Class A codebook design structure by enabling different N1 and N2 combinations

- Reduce the number of combinations of codebook parameters (N1, N2, O1, and/or O2)

  - To be concluded in RAN1#86

  - Examples:

    - Down select from 19 (N1,N2) combinations; and/or

    - Reduce the number of (O1,O2) combinations

    - Other possibilities are not excluded

In this contribution, we further discuss codebook design for the newly supported number of non-precoded CSI-RS ports for eFD-MIMO and provided evaluation results for different candidate options.

# Possible port layouts for codebook design

According to the WID, the newly supported number of non-precoded CSI-RS ports is restricted to the set of {20, 24, 28, 32}. Table 1 shows all possible port layouts and associated (N1,N2) combinations for the newly supported number of non-precoded CSI-RS ports under X-Pol antenna array. An example on the possible port layouts for 32 CSI-RS ports is also shown in Figure 1 below.

Table 1 Possible 1D/2D layout for {20, 24, 28, 32} non-precoded CSI-RS ports

| Number of CSI-RS ports | 1D | 2D |
| --- | --- | --- |
| 20 | (10,1) | (2,5), (5,2) |
| 24 | (12,1) | (2,6), (6,2), (3,4), (4,3) |
| 28 | (14,1) | (2,7), (7,2) |
| 32 | (16,1) | (2,8), (8,2), (4,4) |

| (N1,N2)=(16,1) |  |
| --- | --- |
| (N1,N2)=(8,2) | (N1,N2)=(2,8) |
| (N1,N2)=(4,4) |  |

Figure 1: Possible 1D/2D layout for non-precoded CSI-RS 32-ports

If all the above (N1, N2) combinations are supported for new codebook design, it could mean that 15x4x2=120 codebooks will be designed in Rel-14, where we assume 8 different codebook sets based on two set of (O1, O2) and four codebook “Config” are associated with each (N1, N2) combination. Compared to Rel-13 for which a total of 48 codebooks, e.g., 6x4x2, were designed, the number of codebook in Rel-14 is increased more than 2 times. Although the specification efforts can be minimized by reusing most Rel-13 codebook design methodology, RAN4 performance testing and UE implementation efforts can still be very high. It is not clear whether all these port layouts will be used in a real network deployment, such as 7 vertical ports. Therefore, we prefer to down-select a subset from all possible (N1, N2) combinations for Rel-14 codebook design, such as only (N1, N2) = (16, 1) supported for 1D port layout.

Proposal 1: For 1D port layout consider only (N1, N2) = (16, 1) for codebook design.

# Codebook design for newly supported number of CSI-RS ports

A scalable codebook design framework was introduced in Rel-13, for which a codebook is configured by a set of RRC parameters to support a variety of port layouts. Therefore, a straightforward solution for Rel-14 codebook design for {20, 24, 28, 32} ports is to extend the Rel-13 codebook by introducing additional (N1, N2) combinations. Some investigations may be needed, e.g., determining the set of (O1, O2) for new (N1, N2) combinations, but in principle the existing parameter values can be reused.

Although the extension of Rel-13 codebook to additional (N1, N2) combinations may work for the newly supported number of non-precoded CSI-RS ports, it may not be sufficient for achieving the optimal performance. A big difference between Rel-13 and Rel-14 is that Rel-14 codebook design needs to support a larger number of antenna ports and a larger antenna array, such as 32 ports with 16 columns x-pol antenna array. It is known that the beam width is reduced with the increase of array form factor, and the beam width of 32-ports may be only a half of that of 16-ports. If the same codebook “Config” is reused it could mean the effective DoA range covered by a set of beams in W1 codebook is significantly reduced. Using a small oversampling factor such as 2 may increase effective range but at the price of a loss of beam granularity. It is concluded in the study item that significant performance loss is observed for an oversampling factor of 2. Therefore, a new codebook may be needed to improve the W1 beam coverage for the newly supported number of CSI-RS ports to avoid performance degradation for relatively large ASD/ESD.

Several codebook enhancements options were proposed in [2] – [4], and summarized below.

- Alternative 1: 4x4 grid of beams with 16 beam selection

The first alternative is to use a large beam group (L1, L2) for W1 codebook. One example is shown in Figure 2. A 4x4 grid which consists of total 16 beams can be selected for W1 covering 2 times angular spread than legacy Rel.13 Class A codebook Config2. However, W2 feedback overhead is also increased from 4 to 6 bits.

Figure 2: Example of 4x4 grid of beams with 16 beam selection

- Alternative 2: 2x8 grid of beams with 8 beam selection

Figure 3 shows another example for large beam group design with (L1, L2) = (2, 8). It is extended from the legacy Rel.13 codebook Config3 by introducing more beams in a beam group. 5 bits feedback for W2 is required to select one of 8 beams and polarization co-phasing.

Figure 3: Example of 2x8 grid of beams with 8 beam selection

- Alternative 3: 2x8 grid of beams with 4 beam selection

In another example, a larger beam group can be supported without increasing the W2 codebook size. As shown in Figure 4, the four beams in the group are non-uniformly distributed in the 2D grid of (2, 8). The four beams are divided into two groups each containing two adjacent beams, and the two groups are separated by a gap of two beams. The benefit of this option is to maintain same W2 feedback overhead but provide large beam coverage compared to legacy Rel-13 Config3 codebook.

Figure 4: Examples of 2x8 grid of beams with non-uniform 4 beam selection

- Alternative 4: UE adaptive inter-beam spacing with 4 beam selection

Another example to support large beam group without increasing W2 codebook size is to use adaptive inter-beam spacing. For example, as shown in Figure 3, two options are considered to select four beams from a grid of beams with (L1, L2) = (2,8). The two options are differentiated by the inter-beam spacing, e.g., (p1, p2) = (1,1) or (2,1). The first option provides more beam granularity when channel angular spread is small, and the second provides sufficient angular coverage thus better supports large angular spread channel. The selection of two options can be UE specific based on channel angular spread, and the feedback of the selection of (p1, p2) can be included also in W1 feedback.

Figure 4: UE adaptive inter-beam spacing (p1, p2) with 4 beam selection

# Performance Evaluation

In this section, we will provide evaluation results of different codebook design options for the newly supported number of non-precoded CSI-RS ports. The baseline is an extension of Rel-13 codebook Config3. Four codebook enhancement options discussed in section 3 are considered in the evaluation:

In the evaluation, two antenna array (8, 4, 2) and (8, 8, 2) are considered. Both are virtualized to 32-ports. For the former, 2 vertical antenna elements are virtualized to one port using DFT vector, and for the latter four elements are virtualized to one port. The used codebook parameters are summarized in Table 2.  FTP traffic is assumed with a packet arrival rate =4.2 corresponding to 50% resource utilization for 32-ports baseline. Other simulation assumption are based on [5].

Table 2: Codebook parameters used in the evaluation

| (O1, O2) | (S1, S2) | (p1, p2) |
| --- | --- | --- |
| (4, 4) | (2,2) | (1,1) or (2,2)* |

Note: (p1, p2) = (2, 2) is only applied to Alt. 4.

Table 3 shows the system performance for (4, 4, 2) port layout with 32-ports under 3D UMa and 3D UMi. It can be seen that all the four codebook enhancement schemes could provide non-negligible performance gain over the baseline, especially on cell edge throughput performance of 3D UMa. Alt. 1 with 6-bits W2 has the best performance among four alternatives, but the performance gap between Alt. 1 and other three alternatives are marginal.

Table 3: Non-full buffer simulation results for 32-ports with (4, 4, 2) port layout

| 32TxRU, (4,4,2) |  | Baseline (Config3) | Alt. 1 with 6bits W2 | Alt. 2 with 5bits W2 | Alt. 3 with 4bits W2 | Alt. 4 with two (p1, p2) |
| --- | --- | --- | --- | --- | --- | --- |
| 3D UMa ISD 200m | 5%-tile (Mbps) | 8.13 | 8.90 (9.47%) | 8.6768 (6.73%) | 8.85 (8.85%) | 8.602 (5.81%) |
|  | 50%-tile (Mbps) | 21.739 | 22.68 (4.31%) | 21.978 (1.1%) | 22.6 (3.96%) | 22.599 (3.96%) |
|  | Mean (Mbps) | 25.852 | 26.89 (4.02%) | 26.355 (1.95%) | 26.84 (3.81%) | 26.77 (3.55%) |
| 3D UMi ISD 200m | 5%-tile (Mbps) | 8.457 | 8.69 (2.75%) | 8.639 (2.16%) | 8.51 (0.63%) | 8.602 (1.72%) |
|  | 50%-tile (Mbps) | 23.39 | 24.27 (3.77%) | 24.242 (3.64%) | 23.39 (0%) | 24.096 (3.02%) |
|  | Mean (Mbps) | 27.90 | 28.41 (1.82%) | 28.382 (1.73%) | 28.128 (0.82%) | 28.522 (2.23%) |

Table 4 shows the system performance for (2, 8, 2) port layout with 128 antenna elements, 32-ports under 3D UMa and 3D UMi. As more antenna ports are now on the horizontal dimension, Alt. 2~4 which support large angular spread on horizontal have comparable or even better performance than Alt. 1.

Table 4: Non-full buffer simulation results for 32-ports with (2, 8, 2) port layout

| 32TxRU, (2,8,2) |  | Baseline (Config3) | Alt. 1 with 6bits W2 | Alt. 2 with 5bits W2 | Alt. 3 with 4bits W2 | Alt. 4 with two (p1, p2) |
| --- | --- | --- | --- | --- | --- | --- |
| 3D UMa ISD 200m | 5%-tile (Mbps) | 10.00 | 10.63 (6.30%) | 10.73 (7.32%) | 10.65 (6.54%) | 10.72 (7.24%) |
|  | 50%-tile (Mbps) | 25.64 | 26.21 (2.20%) | 25.96 (1.23%) | 26.18 (2.12%) | 26.14 (1.96%) |
|  | Mean (Mbps) | 30.21 | 30.69 (1.58%) | 30.58 (1.20%) | 30.99 (2.56%) | 30.66 (1.47%) |
| 3D UMi ISD 200m | 5%-tile (Mbps) | 10.17 | 10.68 (4.93%) | 10.65 (4.67%) | 10.58 (3.91%) | 10.72 (5.36%) |
|  | 50%-tile (Mbps) | 27.58 | 28.35 (2.78%) | 28.55 (3.50%) | 28.19 (2.21%) | 28.17 (2.11%) |
|  | Mean (Mbps) | 32.1 | 32.68 (1.51%) | 32.77 (1.79%) | 32.97 (2.41%) | 32.42 (0.70%) |

Since the performance of the four alternatives are close and Alt. 3 with 4bits W2 has the benefit of relatively low overhead for both W1 and W2, we prefer to support Alt. 3 for codebook enhancement.

Proposal 2: Consider Alt. 3 as new one codebook config or replacing Config3 as codebook enhancement for the newly supported number of non-precoded CSI-RS ports.

Another issue is to determine the (O1, O2) combinations for enhanced codebook design. In Rel-13, two (O1, O2) combinations are supported for each port layout. This can be extended to new Rel-14 codebook as shown in Table 5 below.

Table 5: Possible (O1, O2) combinations for different port layout

| (N1,N2) | (O1,O2) |
| --- | --- |
| N1> N2 | (8,4), (4,4) |
| N1< N2 | (8,8), (8,4) |
| N1=N2 | (8,8), (4,4) |

To conclude whether all the combinations are needed for Rel-14 codebook, we evaluate the performance of different (O1, O2) combinations for 32 TXRU with (4, 4, 2) and ((2, 8, 2) port layouts. Results are summarized in Table 6. It can be seen that a marginal gain of less than 4% percent can be observed for port layout (4, 4, 2) when (O1, O2) is changed from (4,4) to (8,8). However, negative gain is observed for port layout (2, 8, 2) when using large oversampling factor in the horizontal dimension. Since there is strong benefit to use large oversampling factors we propose to downselect or support only one (O1, O2) combination for each port layout for Rel-14 codebook.

Table 6: Non-full buffer simulation results for 32-ports with different (O1, O2)

| 32 TXRU |  | (4,4,2) (O1,O2) = (4,4) | (4,4,2) (O1,O2) = (8,8) | (2,8,2) (O1,O2) = (4,4) | (2,8,2) (O1,O2) = (8,4) |
| --- | --- | --- | --- | --- | --- |
| 3D UMa ISD 200m | 5%-tile (Mbps) | 8.13 | 8.41 (3.42%) | 10.00 | 9.92 (-0.81%) |
|  | 50%-tile (Mbps) | 21.739 | 22.24 (2.31%) | 25.64 | 25.12 (-2.01%) |
|  | Mean (Mbps) | 25.852 | 26.53 (2.63%) | 30.21 | 29.86 (-1.15%) |
| 3D UMi ISD 200m | 5%-tile (Mbps) | 8.457 | 8.69 (2.75%) | 10.17 | 9.52 (-3.51%) |
|  | 50%-tile (Mbps) | 23.39 | 24.27 (3.77%) | 27.58 | 27.20 (0.68%) |
|  | Mean (Mbps) | 27.90 | 28.41 (1.82%) | 32.1 | 31.58 (0.49%) |

Proposal 3: Support only one (O1, O2) combination for each port layout for Rel-14 Class A codebook design.

# Conclusion

In this contribution, we discuss codebook enhancements for Rel-14 eFD-MIMO. We have the following proposals.

Proposal 1: For 1D port layout consider only (N1, N2) = (16, 1) for codebook design.

Proposal 2: Consider Alt. 3 as new one codebook config or replacing Config3 as codebook enhancement for the newly supported number of non-precoded CSI-RS ports.

Proposal 3: Support only one (O1, O2) combination for each port layout for Rel-14 Class A codebook design.

# References

RP-160623, “New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE”, Samsung

R1-164525, “Discussion on codebook design for eFD-MIMO”, LG Electronics

R1-164312, “Discussion on rank 1-2 codebook design for eFD-MIMO”, ZTE

R1-164430, “Non-precoded CSI-RS codebooks for eFD-MIMO”, Qualcomm Inc.

TR 36.897, “Study on elevation and beamforming /  Full-Dimension MIMO for LTE”