3GPP TSG RAN WG1 Meeting #86	                                                                                     R1-166322

Gothenburg, Sweden 22nd - 26th August 2016

# Source: 	ZTE Corporation, ZTE Microelectronics

Title:	Discussion on the design of linear combination codebook

Agenda Item:	7.2.4.2.3

Document for:	Discussion and Decision

- Introduction

In RAN1#85, the following agreement is achieved on advanced CSI reporting [2].

- Specify enhancement on CSI reporting to improve eNB precoding. The specified enhancement is to be selected from the following categories:

  - Enhancements to Rel-13 feedback codebooks (FFS which numbers of antenna ports from the set {8, 12, 16, 20, 24, 28, 32}) that increase CSI resolution through improved beam selection / construction in W1 and/or improved beam/port selection / combining / weighting mechanisms in W2

  - Parameters representing channel coefficients, or some reduced space representation thereof including beam combining / weighting with coefficient quantisation or channel quantisation or channel covariance matrix quantisation

    - Uplink physical channel enhancements to carry the representation of channel coefficients can be included if selected

In [3]-[5], companies raised some linear combination (LC) codebook schemes and provided initial simulation results. In this contribution, we further discuss LC codebook design on the aspect of beam selections and feedback approaches, and we conduct simulations to confirm our analysis and conclusions.

- Discussion on linear combination codebook

The linear combination of  beams can be represented as

(2-1)

Where

,

Where  and  are the number of ports of the first and second dimension respectively,  and  are the oversampling factors, and  are relative power level and phase respectively, and . Then we can use the resulting vector  to generate desired linear combination precoding matrix.

- Beam Selections

In order to obtain vector , we should firstly determine which vectors are chosen to be linearly combined. A direct simple way is to select those vectors indicated by. It is not necessary to introduce new signal to feed back the beam selection information through this method. However, the vectors in  convey too much redundant information which is not helpful to recover the real channel information. In fact, when the best codeword  is determined by legacy codebook based on codebook configuration, the remain channel information should lie in the orthogonal space of  , which means that we should construct the linear codebook by orthogonal 2D DFT beams. Consider set , it is obvious that the vectors in set  are mutually orthogonal due to the property of 2D DFT beams. Then we can choose some vectors from  to generate linear combination precoding.

Obviously, it is impossible to use all orthogonal 2D DFT beams to get the target precoding because of extreme large feedback overhead. Therefore down-selection from these beams should be done in order to reduce overhead. Theoretically, we should choose the best  beams including   in the set  since these  beams reflect better channel information compared with other beams. However, this approach may also cause feedback overhead problem, which will be analyzed in next section.

When UE selects the best codeword , it means that the main direction of channel information can be determined, and the other  best beams would be around the direction of  . Based on this consideration, we can pre-define  beam groups, each beam group contains beams (may not be the best)  with pre-define configuration patterns, and then UE chooses one group according to channel measurement to generate linear combination precoding matrix and feedback the selected group index to eNB. In addition, since horizontal and vertical angle spreads vary in different scenarios, the configurations of beam groups may be different.  Figure 1 illustrates this scheme with,,  and  , ,  respectively.

Figure 1. A simple illustration of proposed scheme

For the issue on beam selection, we can summerize three schemes based on above discussion, which is listed as follows.

- Scheme 1: Choose the beams indicated by  by legacy codebook based on codebook configuration.

- Scheme 2: Choose the best  orthogonal beams from the set .

- Scheme 3 (Proposed scheme): Pre-define  beam groups, each beam group contains  orthogonal beams with pre-define patterns, and then UE choose one group to generate linear combination precoder.

- Feedback Overhead

The PMI based feedback of linear combination codebook has two components: the feedback indicating the selected beams, and the feedback indicating the weight coefficients for linear combination and polarization co-phasing.

For the feedback overhead of weight coefficients, assume that m bits and n bits are used to quantify each amplitude coefficient and each phase coefficient respectively. Based on equation (2-1), the total feedback overhead of coefficient is  bits.

For the sake of simplicity, we assume that codebook-config = 1 for Scheme 1. For the feedback overhead of beam selections, scheme 1 requires  bits to feedback beam selection information. For scheme 2, there are two alternatives to indicate the selected beams. One is to feed back the indices of the selected beams in the legacy codebook, the other is to feed back the beam indices of the set . The former one requires  bits, whereas the latter one only requires  bits. Note that, however, the set  should be constructed by pre-defined manner in the latter alternative. For the proposed scheme, the feedback overhead is  bits. The total feedback overhead of three schemes is summarized in Table 1.

Table 1. Feedback overhead of different schemes

|  | Feedback overhead of beam selections | Feedback overhead of coefficients | Co-phasing |
| --- | --- | --- | --- |
| Scheme 1 |  |  | 2 |
| Scheme 2 | Alt1. |  | 2 |
|  | Alt2. |  |  |
| Proposed |  |  | 2 |

- Simulation Results

2.3.1. Beam selection evaluation

To verify the rational of our proposed scheme, we conduct simulations to analyze the distribution of the best orthogonal beams after UE determines the best beam with configuration  and  in 3D-Umi /3D-Uma scenario. The orthogonal beam indexing of each group is showed as figure 2.

Figure 2. Orthogonal beam indexing of (N1, N2) = (4, 4) and (N1, N2) = (8, 2) respectively

Figure 3 shows the selected probability of each beam when we set . It is observed that the distribution of the best beams is relatively fixed in different scenarios. Specifically, for the case , the indices of the best beams are (0, 4, 12, 1, 3) and (0, 4, 12, 1, 8) in Umi and Uma scenario respectively. As for the case , the indices (0, 2, 14, 1, 4) is the best  for both Umi and Uma scenario.

Figure 3(a) Probability of being selected in (N1, N2) = (4, 4) for Umi (left) and Uma (right)

Figure 3(b) Probability of being selected in (N1, N2) = (8, 2) for Umi (left) and Uma (right)

Figure 4 shows the pattern of the selected  beams. It is seen in Figure 4(a) that for (N1, N2) = (4, 4), beams (1, 4, 12) are selected in both Umi and Uma. However, beam 3 is selected in Umi, whereas beam 8 is selected in Uma. The difference comes from the fact that Umi has larger vertical angular spread. Moreover, it is seen in Figure 4(b) that in Umi and Uma, the same beams are selected for (N1, N2) = (8, 2). To summarize the selected beam pattern, each beam group contains the orthogonal beams around the selected beam in the set .

Figure 4(a) Best K beam pattern in (N1, N2) = (4, 4) for Umi (left) and Uma (right)

Figure 4(b) Best K beam pattern in (N1, N2) = (8, 2) for Umi (left) and Uma (right)

2.3.2. System level simulations without quantization

System level simulations are also conducted to compare the performance of three schemes. As for scheme 3, we set and, and the orthogonal beams in each group is pre-defined as Table 2 based on the study above. As for scheme 1, codebook-config = 3 is used. Other system simulation parameters are attached in Appendix.

Table 2. Orthogonal beam index in each group of different scenarios

| (N1, N2) | Scenario | Beam index in each group |
| --- | --- | --- |
| (4, 4) | Umi | {0, 4, 12, 1}, {0, 4, 12, 3}, {0, 4, 1, 3}, {0, 12, 1, 3} |
|  | Uma | {0, 4, 12, 1}, {0, 4, 12, 8}, {0, 4, 8, 1}, {0, 12, 1, 8} |
| (8, 2) | Umi | {0, 2, 14, 1}, {0, 2, 14, 4}, {0, 2, 1, 4}, {0, 14, 1, 4} |
|  | Uma | {0, 2, 14, 1}, {0, 2, 14, 4}, {0, 2, 1, 4}, {0, 14, 1, 4} |

Table 3 shows the simulation results without quantization.

Table 3. Evaluation results for different schemes in different scenarios

| (N1, N2, O1, O2) = (4, 4, 4, 4) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scenario | Scheme | RU | Mean | 5% | 50% |
| 3D-UMi | Legacy cfg1 | 0.65 | 17.52 | 7.64 | 18.37 |
|  | Scheme 1 | 0.64 | 17.64(+0.7%) | 7.87(+3.0%) | 18.26(-0.6%) |
|  | Scheme 2 | 0.60 | 19.06(+8.8%) | 9.31(+21.9%) | 19.93(+8.5%) |
|  | Proposed | 0.62 | 18.67(+6.7%) | 8.97(+17.4%) | 19.37(+5.4%) |
| 3D-UMa | Legacy cfg1 | 0.63 | 18.00 | 10.02 | 18.22 |
|  | Scheme 1 | 0.63 | 18.12(+0.7%) | 9.98(-0.4%) | 18.41(+1.0%) |
|  | Scheme 2 | 0.59 | 19.48(+8.2%) | 11.63(+16.1%) | 19.89(+9.2%) |
|  | Proposed | 0.59 | 19.33(+7.4%) | 11.58(+15.6%) | 19.72(+8.2%) |
| (N1, N2, O1, O2) = (8, 2, 4, 4) |  |  |  |  |  |
| Scenario | Scheme | RU | Mean | 5% | 50% |
| 3D-UMi | Legacy cfg1 | 0.68 | 16.78 | 6.44 | 17.32 |
|  | Scheme 1 | 0.67 | 16.90(+0.7%) | 6.56(+1.9%) | 17.26(-0.3%) |
|  | Scheme 2 | 0.62 | 18.82(+12.2%) | 9.23(+43.3%) | 19.55(+12.9%) |
|  | Proposed | 0.63 | 18.38(+9.5%) | 9.09(+41.1%) | 19.21(+10.9%) |
| 3D-UMa | Legacy cfg1 | 0.64 | 17.54 | 8.94 | 17.71 |
|  | Scheme 1 | 0.64 | 17.36(-1.0%) | 9.19(+2.8%) | 17.43(-1.6%) |
|  | Scheme 2 | 0.58 | 19.52(+11.3%) | 11.82(+32.2%) | 19.93(+12.5%) |
|  | Proposed | 0.59 | 19.36(+10.4%) | 11.38(+27.3%) | 19.67(+11.1%) |

It can be observed from Table 3 that:

(1). Compared to legacy scheme, scheme 2 and proposed scheme achieve 8.8% and 6.7% mean performance gain, 21.9% and 17.4% cell-edge performance gain for (N1, N2) = (4, 4) respectively, remarkable gain is also seen in other scenarios. However, scheme 1 is better than legacy scheme only in some cases.

(2). Compared to scheme 2, proposed scheme has slight performance loss with much less feedback overhead.

2.3.3. System level simulations with quantization

In this sub-section, we evaluate the potential schemes with quantized coefficients. Note that m bits and n bits are used to quantify each amplitude coefficient and each phase coefficient respectively. We simulate the cases that (m, n) equals (3, 5), (2, 5) and (2, 3). The simulation results are shown in Figure 5.

Figure 5(a) Mean performance (left) and 5% performance (right) gain comparison of different schemes with different quantization in (N1, N2) = (4, 4) 3D-Umi scenario

Figure 5(b) Mean performance (left) and 5% performance (right) gain comparison of different schemes with different quantization in (N1, N2) = (4, 4) 3D-Uma scenario

It can be observed from Figure 5(a) and Figure 5(b) that:

(1). Scheme 2 and proposed scheme still can achieve remarkable performance gains with quantization, especially for the cell edge performance. Scheme 1 suffers severe performance loss as the quantization accuracy decreases, at least 10% cell edge performance loss can be observed when the quantization level is (2, 3).

(2). As for scheme 2 and proposed scheme, it is seen that 2 bits and 3 bits uniform quantization for amplitude and phase coefficient is enough to have good performance gain.

Based on the analysis and evaluation results, we have the following proposal.

Proposal: Adopt the proposed scheme with the beam selection pattern in Fig. 4 for linear combination codebook.

- Conclusions

In this contribution, we analyze LC codebook design on the aspect of beam selections and feedback overhead and numerous evaluations are presented. Based on the analysis, proposed scheme is a good tradeoff between performance and feedback overhead. We have the following proposal.

Proposal: Adopt the proposed scheme with the beam selection pattern in Fig. 4 for linear combination codebook.

- References

[1] R1-151085, “New WID Proposal: Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”, Samsung

[2] R1-163454, “WF on Rel.14 Advanced CSI”, Nokia, Alcatel-Lucent Shanghai Bell, Samsung, CATT, Huawei, HiSilicon, Qualcomm

[3] R1-164314, “Discussion on CSI reporting of linear combination codebook”, ZTE Corporation, ZTE Microelectronics.

[4] R1-164780, “Linear combination (LC) codebook based CSI reporting and simulation results”, Samsung.

[5] R1-165100, “High resolution CSI feedback”, Ericsson.

- Appendix

| System level simulation parameters |  |
| --- | --- |
| Scenarios | 3D-UMi 200m ISD and 3D-Uma 500m ISD |
| Antenna Configurations | 2x1 virtulization with 130° tilt |
| Antenna Spacing | (dV,dH)=( 0.8λ, 0.5λ) |
| Number of UE antenna | 2Rx cross-polarized antenna |
| Traffic model | FTP 1 with packet size 0.5M byte |
| OLLA | Target at 10% BLER |
| CSI-RS | Period is 5 ms and overhead is accounted. |
| Codebook | Extension of Rel-13 Class A codebook |
| HARQ | Max 4 retransmissions |
| Transmission rank | 1 |
| SU/MU pre-coding | ZF |
| Scheduling | Proportional fair, up to 4 UEs, up to 4 layers |
| CQI/PMI reporting interval and frequency granularity | 5ms for CSI, 6RB |
| Feedback scheme | Rel-12 enhanced CSI feedback, PUSCH mode 3-2, Ideal channel covariance /PMI feedback |
| Delay for scheduling and AMC | 6ms |
| Receiver | MMSE-IRC. With non-ideal interference covariance matrix estimation by using complex Wishart distribution with 12 degrees of freedom (Model in TR36.829 with DMRS based sample covariance matrix) |
| HARQ Scheme | Chase Combining |
| Maximum number of retransmissions | 4 |
| Traffic model | FTP1 model with 0.5Mbyte |
| Feedback Assumption | Non-ideal modeling of channel estimation error modeling is used, based on DMRS for data demodulation, based on IMR for interference measurement |
| Handover margin | 3dB |