3GPP TSG-RAN WG1 #86				     															  R1-166395

August 22nd – 26th, 2016

Gothenburg, Sweden

Agenda item:	8.1.7

Source: 	Qualcomm Incorporated

Title: 	URLLC system capacity and URLLC/eMBB multiplexing efficiency analysis

Document for:	Discussion/Decision

# Introduction

In NR, it is crucial to support efficient multiplexing of eMBB and URLLC services in the same carrier to maximize system capacity for both services and make optimal use of time-frequency resources. In this contribution, we showed via theoretical queueing analysis and system-level simulations that dynamic multiplexing of eMBB and URLLC in both frequency and time domains is highly desirable to achieve this goal. In contrast, static/semi-static frequency-division multiplexing results in low cell capacity for URLLC services and low overall system utilization.

# eMBB/URLLC multiplexing in the same carrier

It’s envisioned that URLLC services will exploit shorter subframes as compared to the one used by eMBB to meet the stringent latency requirement. There are various proposals for multiplexing subframes. Due to the bursty and delay-sensitive nature of URLLC traffic, it may not fully utilize all the time-frequency resource. It is highly desirable to multiplex eMBB and URLLC services dynamically in both frequency and time domains in the same carrier. As an example, Figure 1 shows that eMBB transmissions (the yellow part) occupy one long subframe. When URLLC traffic arrives randomly within the long subframe, its HARQ transmissions are carried in short subframes (the blue and red parts) and are both frequency-division-multiplexed (FDM) and time-division-multiplexed (TDM) with the eMBB traffic.

Figure 1: An example of dynamic eMBB (yellow) and URLLC (blue/red) multiplexing

Dynamic eMBB-URLLC multiplexing both improves system efficiency and significantly increases the cell capacity for URLLC. One alternative proposal is to partition system bandwidth statically or semi-statically, and serve URLLC and eMBB traffic in an FDM fashion. Next, we present theoretical queueing analysis and system-level simulation results to show that static frequency division multiplexing leads to low system utilization and low cell capacity for URLLC traffic. Hence, dynamic multiplexing is desired.

## Theoretical Queueing Analysis

Assume some bandwidth is reserved for URLLC. Next we use a simplified queueing model to gain qualitative insight on how the system capacity of URLLC scales with the reserved bandwidth. System-level simulation results will be presented next. Consider the URLLC traffic model to be Poisson. Consider the URLLC KPIs that comprise of hard latency and reliability requirements. That is, one packet of X (e.g., 32 bytes) bytes must be delivered with L (e.g., 1ms) ms between radio protocol layer 2/3 ingress/egress points and the long-term average loss of packets (due to HARQ failure and missed deadline) must be less than R (e.g., 1e-5) [1]. The amount of reserved bandwidth decides how many concurrent URLLC transmissions can be FDM’ed in one short subframe. From these assumptions, let’s consider an M/D/m/m queueing model: ‘M’ means the arrival process is Poisson, ‘D’ means that the over-the-air delay is a deterministic value (e.g., assuming all URLLC transmissions can be decoded successfully in one transmission and there is no need for HARQ retransmissions nor ACK/NACK feedback), the first ‘m’ denotes the maximum number of concurrent transmissions, and the second ‘m’ indicates that if a packet has non-zero queueing delay, then it is of no use and dropped from the system due to missing a stringent hard deadline.

In the M/D/m/m queueing model, the loss of system reliability is the Erlang-B formula [2, p179]:

where λ is the Poisson arrival rate in the unit of packets per short subframe, and (1/μ) is the deterministic service time in the unit of short subframe; let μ=1 here. The corresponding system utilization, defined as the time-average proportion of allocated resources, is

Figure 2 shows that increasing the Poisson arrival rate yields more severe queueing effect, leading to higher packet and reliability loss. In other words, as the reliability requirement is tightened, we must reduce the traffic load (hence lower system utilization) in order to continue meeting the new requirement. Figure 3 shows the maximally supported arrival rate (i.e., system capacity) and the association system utilization at which the 1e-5 reliability is satisfied. We observe that, as the number of allowed FDM’ed transmissions (i.e., reserved bandwidth for URLLC) decreases, the system capacity decreases super-linearly fast with low system utilization. As an example, if the number of FDM’ed transmissions is reduced from 16 to 8 to 4, the maximally supported arrival rate changes roughly from 4.2 to 1.0 to 0.1 packets per short subframe, respectively.

Figure 2. Loss of system reliability and the system utilization as a function of Poisson arrival rate in an M/D/m/m queueing model with m=10. The hard latency requirement is implied in the queueing model.

Figure 3. Maximally supportable Poisson arrival rate and the associated system utilization subject to 1e-5 reliability requirement in an M/D/m/m queueing model, where ‘m’ is the number of allowed FDM’ed transmissions and scales with the reserved bandwidth for URLLC.

## System-Level Simulations

### Assumptions

The system-level simulation assumptions are summarized in Table I.

Table I: System-Level Simulation Assumptions

The simulation is performed on the FDD downlink in the rural scenario. There is one URLLC serving cell with 20 eMBB neighboring cells in a wrapped-around model. Control channel (PDCCH, PUCCH) overhead is not modelled. The eNB periodically sends CSIRS for channel estimation and the UEs report CSI periodically to the eNB for the MIMO operation. The scheduling policy is delay-based and focuses on providing equal grade of service to the UEs. The serving cell is subject to full-buffer inter-cell interference from all neighboring cells. We use ‘TTI’ and ‘subframe’ interchangeably in this contribution. It is worth noting that our notion of a subframe is the smallest schedulable unit of time for a given service, where control can occur once within any subframe.

### KPIs

Each UE is subject to hard latency and reliability requirements. The latency is defined as the time between a URLLC packet entering the MAC buffer at the transmitter and being successfully decoded at the receiver [4]. That is, the latency comprises of scheduling, queueing, transmission, eNB/UE processing, HARQ RTT, and decoding. The latency deadline is a hard requirement, i.e., if a packet cannot be decoded within the latency bound, it is of no use and dropped from the network, resulting in loss of reliability. The reliability requirement for a UE is defined as the long-term proportion of packet loss. As an example, the 1e-5 reliability requirement means that at most one out of 100,000 packets is allowed to fail. There are two sources of reliability loss: (i) packets that expect to miss the deadline upon the next reception are dropped at the transmitter side; (ii) HARQ failure at the receiver side.

We assume all UEs have the same Poisson arrival rate following the FTP model 3. The system capacity for URLLC corresponds to the largest arrival rate at which the hard latency and reliability requirements are satisfied for all UEs, including the cell-edge ones. As an example, if the maximum Poisson arrival rate at which all UEs meet the QoS is 1000 packets/sec/UE, where there are 22 UEs in the URLLC serving cell and 256-bit packet payload is used (see Table I), then the system capacity is

1000 packets/sec/UE * 256 bits/packet * 22 UEs/cell = 5.63 Mbps/cell.

In our system-level simulations, we assume that each UE is subject to the 1e-5 reliability and {500, 750, 1000}us hard latency requirements. A wide range of Poisson arrival rates are swept to find the maximally supported arrival rate for the UEs.

### Simulation Results

Figure 4 shows the URLLC system capacity under different reserved bandwidth and hard latency requirements. The system capacity scales superlinearly with the bandwidth as predicted by the queueing analysis in Figure 3. The capacity gain results from better trunking efficiency. This result suggests that it is more beneficial to allocate wide bandwidth to URLLC services, yielding better spectral efficiency.

Figure 4: The URLLC system capacity under different reserved bandwidth and hard latency requirements. The reliability requirement is 1e-5.

Observation 1: URLLC capacity increases superlinearly as the bandwidth increase. It is more beneficial to allocate wide bandwidth to URLLC services, yielding better spectral efficiency

Figure 5 shows the system utilization of the reserved bandwidth for URLLC when the UEs are fully loaded with Poisson traffic at the capacity-achieving points shown in Figure 4. We observe that the system utilization is generally low, and increasing the traffic load further will create more queueing effect so that the 1e-5 reliability can no longer be met. As an example, the utilization is 32.4% when the reserved bandwidth is 10MHz and the latency is 750us, and 67.6% of the reserved bandwidth are wasted under static FDM. Under dynamic eMBB and URLLC multiplexing, the eMBB capacity could jump from 0 to 28.26Mbps following the calculation

4.18 bps/Hz * 10MHz * 67.6% = 28.26Mbps,

where 4.18bps/Hz is the eMBB DL full-buffer spectral efficiency [3].

These results suggest that reserving bandwidth to URLLC is inefficient, and dynamic multiplexing of eMBB and URLLC services over the entire system bandwidth is desired to fully exploit system resources and maximize the URLLC capacity. URLLC system utilization becomes even lower when reserved BW becomes small and latency bound tightens (for example, to achieve 500us latency, 1e-5 reliability with BW = 10MHz, the maximum system utilization of URLLC is merely 2.3%).

Observation 2: Bandwidth reservation solely for URLLC service is inefficient due to the stringent delay and reliability requirement, as well as (bursty) random arrival of URLLC packets.

Observation 3: Dynamic multiplexing of eMBB and URLLC services is highly desirable for efficient use of system resources and maximize the URLLC capacity, following the principle of statistical multiplexing (trunking efficiency).

Figure 5: System utilization of reserved bandwidth at the maximum capacity in Figure 4.

# Conclusions

In this contribution, we provide analysis on the URLLC capacity from both queuing theory perspective and the simulation perspective. As expected, due to stringent delay and reliability requirement for URLLC, even at the URLLC capacity, there are still ample slack capacity that can used for other services. Following the system design principle that statistic multiplexing is highly desirable to exploit trunking efficiency, it is very import to allow URLLC service to be multiplexed with other services, for example eMBB.

Below are our observations and proposal.

Observation 1: URLLC capacity increases superlinearly as the bandwidth increase. It is more beneficial to allocate wide bandwidth to URLLC services, yielding better spectral efficiency

Observation 2: It is very inefficient to reserve bandwidth solely for URLLC service due to the stringent delay and reliability requirement, as well as random arrival of URLLC packets.

Observation 3: Dynamic multiplexing of eMBB and URLLC services is highly desirable for efficient use of system resources and maximize the URLLC capacity, following the principle of statistic multiplexing (trunking efficiency).

Proposal 1: NR should consider dynamic multiplexing of eMBB and URLLC services in both FDM and TDM fashion over the system bandwidth.

# 4			References

[1]. 3GPP Technical Report 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

[2]. D. Bertsekas and R. Gallager, “Data Networks,” 2nd ed., Prentice Hall, 1992.

[3]. R1-166390, “Updated Sub6 DL Full-buffer KPI evaluation for eMBB,” Qualcomm Incorporated.

[4]. R1-166398, “URLLC system-level simulation assumptions,” Qualcomm Incorporated.