3GPP TSG RAN WG1 Meeting #86                                            R1-166468

Gothenburg, Sweden 22nd - 26th August 2016

Source:	CATT

Title:	Remaining issues on evaluation assumption and methodology of MA

Agenda Item:	8.1.2.2

Document for: 	Discussion and Decision

Introduction

During RAN1#84bis and following RAN1 reflector email discussion, some of link-level and system-level evaluation assumptions for MA were discussed and concluded (referring to Annex). This contribution is an update version of R1-164244 [1]. In this contribution, we discuss the remaining issues on link-level and system-level simulation (LLS/SLS) assumptions for MA. The following six remaining issues are discussed: controversial proposals from email discussion, modelling of interference, comparison base of OMA and NOMA, definition of SNR in OMA and NOMA, performance metrics for SLS evaluation, and evaluation methodology.

Discussions

Remaining issues from email discussion

LLS evaluation assumption:

Most of evaluation assumptions were discussed and concluded by online and email discussion [84b-11] (details can be referred to Annex). Some parameters for LLS are still controversial and therefore could not be agreed.

Total allocated bandwidth for transmission and target spectral efficiency: These two simulation parameters should be considered together. This would reveal the resource usage efficiency and reflect the advantage of each NOMA scheme over OMA. Note that it will provide a fair comparison but not impose limitation on MCS, coding rate and etc.

Proposal 1:

| Parameters | Values or assumptions |
| --- | --- |
| Total allocated bandwidth for transmission (UL) | 4RB (0.72MHz), 12RB (2.16MHz) |
| Total allocated bandwidth for transmission (DL) | 4RB (0.72MHz), 12RB (2.16MHz) |

Proposal 2:

| Parameters | Values or assumptions |
| --- | --- |
| Target spectral efficiency Definition: number of transmission bits per UE / total number of resource elements shared for data transmission | 0.25, 0.375, 0.5 (bps/Hz/ UE) For 4RB case (without CRC): 120 bits, 192 bits, 264 bits For 12RB case (without CRC): 408 bits, 624 bits, 840 bits |

Equal/Unequal transmit SNR: Equal transmit SNR reflects consistent large-scale parameters and the effect of power control. The UEs are evaluated under the same environment while instantaneous received SINR are different among UEs. For unequal transmit SNR, the distribution of unequal transmit SNR is needed, and a large number of samples to reflect various received SINR distribution are needed.

Proposal 3:

| Parameters | Values or assumptions |
| --- | --- |
| SNR distribution of multiple UEs | Equal transmit SNR (short-term variation remains) |

Modeling of interference

The interference modelling should be addressed in the evaluation methodology. In the single cell link level simulation, the interference from other NOMA UEs is modelled as AWGN.  The Gaussian Approximation (GA) by modelling multi-UE interference as Gaussian noise would have inaccurate and optimistic performance results. The GA for NOMA interference modelling would model the interference exactly the same as that of Orthogonal Multiple Access (OMA). Then NOMA and OMA cannot be distinguished for the Link to System mapping (L2S). This will lead to the over optimistic performance from system level simulation. Moreover, the selection of receiver algorithms would not be tested with the inaccurate modelling of interference. If the interference is modelled as Gaussian noise, the signal processing gain of Successive Interference Cancellation (SIC) receiver and Reduced complexity Maximum Likelihood (R-ML) receiver cannot reflect in the final system performance results. For accurate interference modelling, the interference from paired UEs should be modelled explicitly instead of GA in evaluation of NOMA.

Proposal 4: The interference from paired UEs should be explicitly instead of GA modelled in evaluation of NOMA schemes.

Comparison Baseline of OMA and NOMA

To have fair comparison of OMA and NOMA, the comparison baseline and the definition of SNR should be set up first.

The comparison baseline of OMA and NOMA should be as follows. The number of physical resource block (PRB), the number of UEs, the average total transmit power, the average transmit power per UE, and the transport block size (TBS) are kept the same. For example, when the number of UEs is 4, the total bandwidth is 12 PRB and the TBS is 408 bits, the target spectrum efficiency is 0.25 bps/Hz per UE. For NOMA, each UE will occupy the whole bandwidth. For OMA, each UE will occupy 3 PRBs or 36 subcarriers.

Proposal 5: To have fair comparison of OMA and NOMA, the number of PRB, the number of UEs, the average transmit power per UE, and the TBS should be kept the same.

Definition of SNR in OMA and NOMA

The definition of SNR for OMA is the same as that in LTE, where SNR is the ratio of transmit power of target UE over thermal noise power per RE.

SNR for NOMA can be defined per RE or per pattern (layer) or defined as total SNR of all UEs or SNR per UE. To simplify simulation, the long-term average SNR of UEs of OMA and NOMA is assumed to be the same, and the average total transmit power and transmit power per UE of OMA and NOMA are kept the same. The SNR is defined as average transmit power per UE over noise power.

Example: Taking PDMA with overloading factor (OF) 300% as an example, the number of UEs is 12, the bandwidth is 12 PRB and PDMA pattern matrix  is defined as

.

a) OMA

Let the signal power and noise power per PRB be Ps and Pn, SNR per UE is defined as Ps/ Pn. As the total number of UEs is 12 and each UE occupies 1PRB, each UE's signal power is Ps*1 = 1Ps, and total signal power is 1Ps*12 = 12Ps.

b) PDMA

Since the average total transmit power and average transmit power per UE of OMA and PDMA are the same, total system power and signal power per UE is 12Ps, and 1Ps,, respectively. Since the column weight of PDMA pattern matrix  is 3/3/2/2/2/2/2/2/1/1/1/1, the signal power represented by each element‘1’ of the first two columns is Ps*4/12/3 = Ps/9, that of the third column to eighth column is Ps,*4/12/2 = Ps/6, and that of the last two columns is Ps*4/12/1 = Ps/3.

Proposal 6: The definition of SNR for OMA is the same as that in LTE, where SNR equals ratio of transmit power of target UE over thermal noise power per RE. The SNR for NOMA is defined as average transmit power per UE over noise power, where the long-term average SNR of UEs of OMA and PDMA is assumed to be the same, and the average total transmit power and transmit power per UE of OMA and PDMA are kept the same.

Performance Metrics for mMTC SLS Evaluation

According to 38.913 [2], the target for connection density should be 1 000 000 device/km2 in urban environment. 3GPP needs to achieve this target by designing highly connection efficient RAT, in which connection efficiency is measured as number of supported devices per TRP per unit frequency resource (device/TRP/MHz).

Let δ denote connection efficiency, ω the channel bandwidth and ρ the TRP density (TRP/km2). Then connection density D could be derived from connection efficiency according to equation (1).

(1)

- Connection efficiency = system packet arrival rate (packet/s) / mMTC traffic arrival rate (packet/s/device) / number of TRP / simulation bandwidth

  - mMTC traffic is small packet with sparse packet arrival. Details are discussed in [3]. It is assumed that the packet size is equivalent to the transport block size in the simulation.

  - System packet arrival rate should meet the requirement of packet dropping rate <= 1%.

  - The packet dropping criteria is given as follows.

    - Drop a packet if the packet reaches the maximum number of transmissions N and cannot be decoded correctly where N is maximum number of HARQ transmissions in the simulation, or

    - Drop a packet if its transfer is not completed within a maximum transfer time T_drop, e.g. [10ms].

    - For the case when the system is overloaded, the packet dropping processing can be reasonably simplified, details are FFS.

Proposal 7：Packet dropping rate is included as one performance metric in SLS for mMTC and system packet arrival rate should meet the requirement of packet dropping rate <= 1%.

Evaluation Methodology

The details of the evaluation methodology are given as follows.

Step 1: Set a certain system device number S, and then get system packet arrival rate Npacket based on mMTC traffic arrival rate. Set the system number of TRP as M and simulation bandwidth as W.

Step 2: Generate the traffic packet according to the small packet model at packet arrival rate Npacket and randomly select devices.

Step 3: Run simulation and obtain the packet dropping rate.

Step 4: Change the system device number, repeat step 1-3 to find the system device number Scapacity satisfying the packet dropping rate requirement.

Step 5: Calculate connection efficiency by Scapacity/M/W.

Proposal 8: To include evaluation methodology step 1-5 for mMTC scenario.

Conclusion

In this contribution, we discuss the remaining issues on link/system-level evaluation assumptions for MA. The following five parts are discussed: controversial proposals from email discussion, modelling of interference, traffic models, performance metrics for SLS evaluation, and evaluation methodology.

The proposals are as follows:

Proposal 1:

| Parameters | Values or assumptions |
| --- | --- |
| Total allocated bandwidth for transmission (UL) | 4RB (0.72MHz), 12RB (2.16MHz) |
| Total allocated bandwidth for transmission (DL) | 4RB (0.72MHz), 12RB (2.16MHz) |

Proposal 2:

| Parameters | Values or assumptions |
| --- | --- |
| Suggested target spectral efficiency Definition: required transmission bits per UE / total number of resource elements shared for data transmission | For 4RB case (without CRC): 120 bits, 192 bits, 264 bits For 12RB case (without CRC): 408 bits, 624 bits, 840 bit |

Proposal 3:

| Parameters | Values or assumptions |
| --- | --- |
| Suggested SNR distribution of multiple UEs | Equal transmit SNR (short-term variation remains) Unequal transmit SNR (the SNR distribution is FFS, e.g., uniformly distributed within a range of 3dB, and proponents should report their assumption) |

Proposal 4: The exact interference from paired UEs is modelled in the evaluation of NOMA scheme.

Proposal 5: To have fair comparison of OMA and NOMA and that of different NOMA schemes, the number of PRB, the total number of connected UEs, the average total transmit power, the average transmit power per UE, and the TBS should be kept the same.

Proposal 6: The definition of SNR for OMA is the same as that in LTE, where SNR equals ratio of transmit power of target UE over thermal noise power per RE. The SNR for NOMA is defined as average transmit power per UE over noise power, where the long-term average SNR of UEs of OMA and PDMA is assumed to be the same, and the average total transmit power and transmit power per UE of OMA and NOMA are kept the same.

Proposal 7：Packet dropping rate is included as one performance metric in SLS for mMTC and system packet arrival rate should meet the requirement of packet dropping rate 1%

Proposal 8: To include evaluation methodology step 1-5 for mMTC scenario.

References

- R1-164244, “Remaining issues on link/system-level evaluation assumptions for MA”, CATT, RAN1 #85.

- 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, v0.3.0.

- R1-166467, “Discussion on traffic model of mMTC”, CATT, RAN1 #86.

Annex

# Table 1 Evaluation parameters – LLS for UL

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM /SC-FDMA Other waveform is not precluded |
| Channel coding | LTE Turbo as start point, other coding schemes are not precluded. |
| Numerology | Same as Release 13 |
| System Bandwidth | 10 MHz |
| Total allocated bandwidth for transmission | Companies need to report this value. |
| Overhead | 2 DMRS symbols, no SRS, i.e., 144 available RE per RB for data transmission, or equivalent overhead |
| Target spectral efficiency | Proponents report per UE spectral efficiency and the number of UEs multiplexed if multi-UEs LLS is assumed |
| BS antenna configuration | 2/4 Rx as baseline 8Rx optional |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| SNR distribution of Multiple UEs | Proponents report if single-UE or multi-UE LLS is used, and what SNR distribution is assumed. |
| Suggested SNR distribution of multiple UEs | Equal transmit SNR (short-term variation remains) Unequal transmit SNR (the SNR distribution is FFS, e.g., uniformly distributed within a range of 3dB, and proponents should report their assumption) |
| Propagation channel & UE velocity NOTE2 | TDL for in TR38.900 as mandatory EPA, EVA, ETU as optional 3km/h, 30km/h, 120km/h |
| Max number of HARQ transmission | 1, 4 |
| Given BLER level (to calculate sum throughput) | 0.1 for 1 transmission as starting point, other numbers not precluded, e.g., 0.01 for 1 transmission |
| Overloading factor (Optional, definition refers to R1-163881) | Some example values: 100%, 150%, 200%, 300% |

Note1: Non-ideal effects (e.g., channel estimation, frequency offset) evaluation FFS.

Note2: Companies could choose the propagation model for bringing evaluations at RAN1#85, but companies are expected to provide evaluations at least for the channels listed in the table by RAN1#86.

# Table 2 Evaluation parameters – LLS for DL

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM Other waveform is not precluded |
| Channel coding | LTE Turbo as starting point, other coding schemes are not precluded. |
| Numerology | Same as Release 13 |
| System Bandwidth | 10 MHz |
| Total allocated bandwidth for transmission | Companies need to report this value. |
| Overhead | 2 PDCCH symbols, 2 CRS ports for TM2 , i.e., 132 REs per RB for data transmission, or equivalent overhead |
| Target spectral efficiency | Proponents report per UE spectral efficiency and the number of UEs multiplexed if multi-UEs LLS is assumed |
| BS antenna configuration | 2/4 Tx as baseline 8Tx optional |
| UE antenna configuration | 2 Rx |
| Transmission mode | TM2 as starting point (refer to TS36.213) |
| SNR distribution of Multiple UEs | Fixed gap {0, 5, 10, 15, 20} dB between UEs |
| Number of UEs | 2 UEs as start point |
| SNR of the reference UE | 0dB (The SNR of the other UE would be 0, 5, 10, 15, or 20dB) |
| Power allocation between UEs | Dynamic |
| Propagation channel & UE velocity NOTE2 | CDL in TR38.900 as mandatory EPA, EVA, ETU as optional 3km/h, 30km/h, 120km/h |
| Max number of HARQ transmission | 1, 4 |
| Given BLER level (to calculate sum rate region) | 0.1 for 1 transmission as starting point |

Note1: Non-ideal effects (e.g., channel estimation, frequency offset) evaluation FFS.

Note2: Companies could choose the propagation model for bringing evaluations at RAN1#85, but companies are expected to provide evaluations at least for the channels listed in the table by RAN1#86.