3GPP TSG RAN WG1 Meeting #86		R1-166476

Gothenburg, Sweden 22nd - 26th August 2016

Source:	CATT

Title:	Evaluation of non-binary LDPC codes for NR

Agenda Item:	8.1.4.1 Channel coding

Document for:	Discussion and Decision

Introduction

In RAN #71[1], the study item description on "Next Generation New Radio Access Technology" has been approved. URLLC (Ultra-Reliable and Low Latency Communications) is one of the scenarios for the new radio access technology (NR). The channel coding scheme for URLLC scenario is required to meet  high reliability and  low latency requirements.

In RAN1#84b [2], the following simulation assumptions are agreed for the URLLC scenario:

| Channel* | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK, 16 QAM |  |  |  |
| Coding Scheme | Convolutional codes | LDPC | Polar | Turbo |
| Code rate | 1/12, 1/6, 1/3 |  |  |  |
| Decoding algorithm** | List-X Viterbi | min-sum | List-Y | Max-log-MAP |
| Info. block length*** (bits w/o CRC) | 20, 40, 200, 600, 1000 |  |  |  |

In RAN1#85 [3], performance evaluation of non-binary LDPC codes was provided.

In this contribution, the basic concepts of non-binary LDPC codes are introduced. Performance comparisons are provided following the simulation assumptions agreed in RAN1#84b [2]. Performance comparisons in fading channels are also presented for more general comparison.

Non-binary LDPC codes

Non-binary LDPC codes are a class of linear block codes which is based on high order finite fields GF(q). Non-binary LDPC codes with short or moderate code length can achieve better performance in the  waterfall and error-floor regions of BLER vs SINR curve. Due to the  constraints of high order finite fields, non-binary LDPC codes have the capability of correcting burst errors and are suitable for encoding in combination with high order modulations.

Consider the finite field GF(q), where q is a power of a prime. Let  be a primitive element of GF(q). The q powers of , form all the q elements of GF(q). A non-binary LDPC code C[n,k] over GF(q) is defined by the null space of an (n-k)×n  parity check matrix H over  GF(q).  The following is an example of the  4×6  parity check matrix for a non-binary LDPC code C [6,2] over GF(16):

Performance in AWGN channels

The evaluations are mainly performed based on the agreed simulation assumptions in [2]. Following notations are used for convenience:

k：Information length (bits)

R：Code rate

  - k=20, R =1/3

Simulation Parameters for Figure 3.1:

| Code Type | k | R | Decoder | Channel | Modulation | CRC | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-binary LDPC GF(16) | 20 | 1/3 | FFT-QSPA | AWGN | QPSK | No |  |
| Polar | 20 | 1/3 | List 32 | AWGN | QPSK | 16 bits | Similar with the performance in [4] |

Figure 3.1  k=20, R =1/3, QPSK, AWGN

  - k=40, R =1/3

Simulation Parameters for Figure 3.2:

| Code Type | k | R | Decoder | Channel | Modulation | CRC | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-binary LDPC GF(64) | 42 | 1/3 | FFT-QSPA | AWGN | QPSK | No |  |
| Non-binary LDPC GF(16) | 40 | 1/3 | FFT-QSPA | AWGN | QPSK | No |  |
| Turbo | 40 | 1/3 | max-log-map | AWGN | QPSK | No |  |
| Polar | 40 | 1/3 | List 32 | AWGN | QPSK | 16 bits | Similar to the performance in [4] |

Figure 3.2  k≈40, R =1/3, QPSK, AWGN

  - k=200, R =1/3

Simulation Parameters for Figure 3.3 and Figure 3.4:

| Code Type | k | R | Decoder | Channel | Modulation | CRC | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-binary LDPC GF(64) | 192 | 1/3 | FFT-QSPA | AWGN | QPSK | No |  |
| Non-binary LDPC GF(16) | 192 | 1/3 | FFT-QSPA | AWGN | QPSK/16QAM | No |  |
| Turbo | 200 | 1/3 | Scaled max-log-map | AWGN | QPSK/16QAM | 16 bits |  |
| Polar | 200 | 1/3 | List 32 | AWGN | QPSK/16QAM | 16 bits | Similar to the performance in [4] |

Figure 3.3  k≈200, R =1/3, QPSK, AWGN

Figure 3.4  k≈200, R =1/3, 16QAM, AWGN

  - k=600, R =1/3

Simulation Parameters for Figure 3.5 and Figure 3.6:

| Code Type | k | R | Decoder | Channel | Modulation | CRC | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-binary LDPC GF(64) | 576 | 1/3 | FFT-QSPA | AWGN | QPSK | No |  |
| Non-binary LDPC GF(16) | 576 | 1/3 | FFT-QSPA | AWGN | QPSK/16QAM | No |  |
| Turbo | 608 | 1/3 | Scaled max-log-map | AWGN | QPSK/16QAM | 16 bits |  |
| Polar | 600 | 1/3 | List 32 | AWGN | QPSK/16QAM | 16 bits | Similar to the performance in [4] |

Figure 3.5  k≈600, R =1/3, QPSK, AWGN

Figure 3.6  k≈600, R =1/3, 16QAM, AWGN

Observation 1: non-binary LDPC code has attractive performance under the simulation assumptions agreed in RAN1#84b [2] in AWGN channels.

Performance in fading channels

The evaluations are mainly performed based on the agreed simulation assumptions in [2]. Following notations are used for convenience:

k：Information length (bits)

R：Code rate

Fd：Doppler shift

Ts：symbol period

The normalized Doppler shift= Fd* Ts

  - k=40, R =1/3

Simulation Parameters for Figure 4.1 and Figure 4.2:

| Code Type | k | R | Decoder | Channel | Modulation | CRC | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-binary LDPC GF(16) | 40 | 1/3 | FFT-QSPA | Independent Rayleigh fading channel | QPSK/ 16QAM | No | No interleaver |
| Non-binary LDPC GF(16) | 40 | 1/3 | FFT-QSPA | Correlated Rayleigh fading channel | QPSK/ 16QAM | No | No interleaver; Fd*Ts=0.1 |
| Turbo | 40 | 1/3 | max-log-map | Independent Rayleigh fading channel | QPSK/ 16QAM | No | No interleaver |
| Turbo | 40 | 1/3 | max-log-map | Correlated Rayleigh fading channel | QPSK/ 16QAM | No | No interleaver; Fd*Ts=0.1 |
| Turbo | 40 | 1/3 | max-log-map | Correlated Rayleigh fading channel | QPSK/ 16QAM | No | bit interleaver; Fd*Ts=0.1 |

Figure 4.1  k=40, R =1/3, QPSK, Fading

Figure 4.2  k=40, R =1/3, 16QAM, Fading

  - k=200, R =1/3

Simulation Parameters for Figure 4.3 and Figure 4.4:

| Code Type | k | R | Decoder | Channel | Modulation | CRC | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-binary LDPC GF(16) | 192 | 1/3 | FFT-QSPA | Independent Rayleigh fading channel | QPSK/ 16QAM | No | No interleaver |
| Non-binary LDPC GF(16) | 192 | 1/3 | FFT-QSPA | Correlated Rayleigh fading channel | QPSK/ 16QAM | No | No interleaver; Fd*Ts=0.1 |
| Turbo | 200 | 1/3 | max-log-map | Independent Rayleigh fading channel | QPSK/ 16QAM | No | No interleaver |
| Turbo | 200 | 1/3 | max-log-map | Correlated Rayleigh fading channel l | QPSK/ 16QAM | No | No interleaver; Fd*Ts=0.1 |
| Turbo | 200 | 1/3 | max-log-map | Correlated Rayleigh fading channel | QPSK/ 16QAM | No | bit interleaver; Fd*Ts=0.1 |

Figure 4.3  k≈200, R =1/3, QPSK, Fading

Figure 4.4  k≈200, R =1/3, 16QAM, Fading

  - k=600, R =1/3

Simulation Parameters for Figure 4.5 and Figure 4.6:

| Code Type | k | R | Decoder | Channel | Modulation | CRC | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-binary LDPC GF(16) | 576 | 1/3 | FFT-QSPA | Independent Rayleigh fading channel | QPSK/16QAM | No | No interleaver |
| Non-binary LDPC GF(16) | 576 | 1/3 | FFT-QSPA | Correlated Rayleigh fading channel | QPSK/16QAM | No | No interleaver; Fd*Ts=0.1 |
| Turbo | 608 | 1/3 | max-log-map | Independent Rayleigh fading channel | QPSK/16QAM | No | No interleaver |
| Turbo | 608 | 1/3 | max-log-map | Correlated Rayleigh fading channel l | QPSK/16QAM | No | No interleaver; Fd*Ts=0.1 |
| Turbo | 608 | 1/3 | max-log-map | Correlated Rayleigh fading channel | QPSK/16QAM | No | bit interleaver; Fd*Ts=0.1 |

Figure 4.5  k≈600, R =1/3, QPSK, Fading

Figure 4.6  k≈600, R =1/3, 16QAM, Fading

Observation 2: non-binary LDPC codes can achieve  better performance compared with other codes in fading channels.

Implementation Complexity

Decoding complexities of  non-binary LDPC , Polar, Turbo codes are summarized in Table 1.

Table 1: Decoding complexity of different codes

| Decoding algorithm | Multiplication | Addition | Look-up table |
| --- | --- | --- | --- |
| FFT-QSPA algorithms for non-binary LDPC |  |  |  |
| List+CRC algorithms for Polar |  |  |  |
| Log-MAP for Turbo |  |  |  |
| Max-Log-MAP for Turbo |  |  | - |

Note: v is the memory length of component code, L is the List size of Polar code

Conclusions

In this contribution, we provide the performance evaluation results of non-binary LDPC codes for NR. According to these evaluation results, we observe the non-binary LDPC code has attractive performance under the simulation assumptions agreed in RAN1#84b [2] in AWGN channel. More importantly, non-binary LDPC codes can achieve much better performance compared to other codes in fading channels. The excellent performance of non-binary LDPC codes makes they are particularly suitable for URLLC scenario.

Observation 1: non-binary LDPC code has attractive performance under the simulation assumptions agreed in RAN1#84b [2] in AWGN channels.

Observation 2: non-binary LDPC codes can achieve better performance compared with other codes in fading channels.

Proposal 1: Non-binary LDPC codes have good performance of correcting the robust error for the applications in URLLC scenario.

References

RP-160671, “New SID Proposal: Study on New Radio Access Technology”, NTT DOCOMO, March 2016.

Chairman’s notes, 3GPP TSG RAN WG1 #84bis meeting.

R1-164250, “Performance evaluation of non-binary LDPC codes”, CATT, May 2016.

R1-164378 ,“Performance of channel coding schemes for mMTC and uRLLC scenarios”, Huawei, HiSilicon, May 2016.