3GPP TSG RAN WG1 Meeting #86																	R1-166523

Gothenburg, Sweden, 22 - 26 Aug 2016

Source: 	Intel Corporation

Title:	Discussion on Class A codebook extension

Agenda item:	    7.2.4.2.1

Document for:	Discussion and Decision

# Introduction

In the RAN#70 meeting, a work item on Enhancements on Full-Dimension (FD) MIMO for LTE was approved [1]. The detailed objectives are as follows.

- Specify enhancements on reference signal in the following areas

  - Non-precoded CSI-RS, extending the existing numbers {1, 2, 4, 8, 12, 16} of CSI-RS antenna ports for support of {20, 24, 28, 32} CSI-RS ports with mechanism for reducing the overhead for CSI-RS transmission

- Extend specification support for CSI reporting in the following areas

  - Codebook(s) associated with the newly supported number of non-precoded CSI-RS ports for a subset of possible port layouts, both 1D and 2D

For the design of codebook(s) associated with the newly supported number of non-precoded CSI-RS ports for a subset of possible port layouts, many companies proposed Rel. 13 Class A codebook as a basis of codebook extension. However, there is a potential problem of the Rel. 13 Class A codebook especially for a large number of ports due to narrower beam width. As discussed in [2]-[3], as number of antenna port increases, the DFT beams gets narrower. However in some scenarios narrower beam width is not always desirable. In this contribution we propose enhancement to Class A codebook to support configurable beam width for PMI code-words.

# Beam Broadening

In this section, we introduce solution to widen the angular coverage without overhead increase.  The need for antennas having wide beam patterns, as opposed to narrow beam ones, arises in the scenarios with higher rank transmission (e.g. 3-4 layers), where the potential loss in the gain due to beam widening may not be critical. At the same time usage of the widen beam may offer the increased probability of the higher transmission rank selection. The use of widen beam is also preferable in high mobility scenario for more robust selection of the precoding at the eNB which will be less sensitive to fast change of the UE position.

The proposed beam broadening function applied to the set of DFT-based PMI vectors W can be illustrated as follows:

,

where Wb is PMI corresponding to broaden beam, N1 or N2 is number of CSI-RS antenna ports in horizontal and vertical domain, respectively, {p,c} parameters of beam broadening function that can be configured by the eNB depending on the required beam width. Note that larger c provides broader beam for a given p while smaller p provides broader beam for a given c. The basic principle of beam broadening function is supplying additional phase to reduce the possibility of constructive addition of all the contributions of the array elements. The examples of PMI patterns for different values of {p,c} is illustrated in Figure 1 and Figure 2.

Figure 1 Example of beam broadening for different c values

Figure 2 Example of beam broadening for different p values

# Performance evaluation

To demonstrate the performance benefits of proposed beam broadening function the system-level evaluations were carried out using non-full buffer traffic model with a packet size of 0.5Mbytes. For evaluation 4 receive antennas at the UE with maximum 4 transmission layers was assumed. It was assumed that half of the original DFT-based beams in rank-3/4 PMIs were replaced with beam broadened PMI so that the CSI reporting overhead is not increased. The results are presented in Figure 3 for RU of 4% and 8%, which corresponds to the scenario when higher rank selection is possible.

Figure 3 Packet throughput results with Rel-13 and Rel-14 codebook with beam broadening function applied to some rank3/4 PMIs

It can be seen from the presented results that the proposed beam broadening enhancement can improve the performance of FD-MIMO technology and, therefore, should be considered as Rel-14 codebook enhancement. Summarizing discussion above the following proposal can be made:

Proposal:

- Class A FD-MIMO should support beam broadening function in Rel-14 to improve the performance in high mobility scenarios and scenarios with higher transmission ranks (e.g. 3/4)

# Conclusions

In this contribution we have proposed enhancement to Class A codebook to support configurable beam width for PMI codewords. Based on the discussion and evaluation results the following proposal was made:

- Class A FD-MIMO should support beam broadening function in Rel-14 to improve the performance in high mobility scenarios and scenarios with higher transmission ranks (e.g. 3/4)

# References

- RP-160623, New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE, Samsung, 3GPP TSG RAN Meeting #71, Göteborg, Sweden, March 7 - 10, 2016

- R1-162689, View on class A codebook extension, Samsung, 3GPP TSG RAN Meeting #84bis, Busan, Korea, April 11 - 15, 2016

- R1-163041, Qualcomm Incorporated, Non-precoded CSI-RS Codebooks for eFD-MIMO, 3GPP TSG RAN Meeting #84bis, Busan, Korea, April 11 - 15, 2016

Appendix

| Parameter | Value |
| --- | --- |
| Scenario | 3D-UMi, ISD = 200 m (homogeneous) Geographical distance based wrapping |
| eNB antenna configuration | URA X-pol, slants -45/+45 degree, 16 TXRUs, 1x8x2 port layout 8x1 subarray per TXRU 0.5-wavelength horizontal spacing 0.8-wavelength vertical spacing |
| UE antenna configuration | 4 Rx X-pol, slant 0/90 degrees |
| Traffic model | FTP model 1, S=0.5 Mbyte packet size |
| Cell association | CRS antenna port 0, mapped to the two vertical TXRUs Handover margin = 3dB |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Channel estimation | Ideal |
| Interference covariance estimation | Ideal |
| CSI feedback | Mode 3-2 with 5 ms periodicity |
| CRS configuration | Colliding across all modelled cells |
| Transmission mode | TM9 |
| Scheduling | Proportional Fair |
| OLLA | 10% BLER target |
| Elevation beamforming | One vertical beam per TXRU electrically down-tilted to 100 degrees |
| UE receiver | MMSE-IRC |
| Max HARQ transmissions | 4 |