3GPP TSG RAN WG1 Meeting #86		R1-166718

Gothenburg, Sweden 22nd - 26th August 2016

Agenda Item:		7.2.2.4.1

Source:			Samsung

Title:			Enhancement for UL SPS

Document for:		Discussion and Decision

1. Introduction

In RAN1#84 meeting, SL resource allocation in SPS manner was discussed and enhancing SPS techniques supporting multiple SPS configuration and dynamic trigger/release operation were agreed in RAN1#84bis as follows [1]:

Agreement:

- For SPS of V2V traffic for mode-1 SPS on PC5:

  - The eNB may configure multiple SPS configurations for a given UE

    - At least SPS-configuration-specific MCS (if MCS is part of the SPS-configuration) and SPS-configuration-specific periodicity can be configured

      - FFS if/which other SPS parameters can differ across the SPS-configurations

    - The eNB can dynamically trigger/release the different SPS-configurations by use of (E)PDCCH

      - Details of the trigger/release are FFS

  - Working assumption: The UE can indicate to the eNB that it does not intend to transmit data before a transmission associated to an SPS configuration

    - FFS any details of the signaling protocol

    - FFS whether eNB acknowledgment of the UE indication is needed

In RAN2#93bis meeting, RAN 2 also discussed the dynamic variation for CAM messages in terms of size, periodicity and timing [2]. In order to support UL/SL SPS with variations of periodicity and size, the multiple SPS configurations running in parallel and the dynamic SPS configuration are needed for flexible resource allocations.

In this contribution, the support for multiple SPS configurations and the associated signaling in PDCCH for mode-1 SL SPS is discussed

2. Discussion

There is a requirement that the E-UTRA(N) shall be capable of transferring periodic broadcast messages between two UEs supporting V2X Services with variable message payloads of 50-300 bytes, not including security-related message component (See CPR-019 in [3]). For many V2V data services, the message sizes are small and the inter-arrival time of transmission is fairly constant (for example, the CAM messages are periodic with frequency of 1-10 Hz). The control signaling overhead (PSCCH) can be significant in order to support a large number of vehicles. So, it is important to allocate the resources at once and let the vehicle use these resources instead of re-allocating the resources periodically. To support this efficiently, semi-persistent scheduling support is desirable for V2X communication.

Although the V2V traffic is approximately periodic for relatively long intervals or in proximity of certain events, it is necessary to take into account the possible deviations when designing semi-persistent transmissions in the LTE V2X framework.

Figure 1 shows an example transmission of a V2V message from a UE. The messages typically contain a basic message of a fixed size regarding vehicle status while the additional messages are transmitted occasionally from upper layers which can contain information such as security [4].

Figure 1: V2V message variation in periodicity and RB size

Multiple SL SPS configurations can be used to handle the periodicity variation for the basic message transmission. In this case, only switching between multiple SL SPS configurations is needed and only 1 SL SPS configuration needs to be active at a given time. The additional messages may either be configured using SPS with a different periodicity, if possible, or may be dynamically scheduled by the eNB. In this case, multiple SPS configurations for SL are possible actively running at a given time. In addition, the reporting of information to eNB from UE can also be supported via SPS, which can be simultaneously active. See companion documents [5,6] for more details.

Proposal 1

- When the eNB configures multiple SPS for SL transmissions, the UE may at least switch between the SPS configurations. In addition, it is supported to have a simultaneously active SPS on the UL for UE reporting to eNB;

If multiple SPS configurations are supported, it is necessary to limit the number of configurations to a reasonable number to reduce implementation complexity. For example, 4 SPS configurations for SL transmissions may be sufficient. Furthermore, it may be necessary to identify the SPS configurations to enable activation/release of different configurations. This can be further discussed as part of RAN 2 discussions for SPS. See companion document [6] for more details.

Proposal 2

- RAN1 should request input from RAN 2 on (a) the maximum number of allowed SL SPS configurations for a UE, (b) the identification procedure for each SPS configuration, in order for RAN1 to decide the DCI formats for activation/release of these configurations;

Simultaneous activation and release can be supported for fast SPS switching between configurations, where release of one configuration activates another configuration.  Table 1 shows an example of using few bits of the DCI format fields for simultaneous activation and release of a SPS configuration. If the same configuration is made active and release simultaneously, it can be used to signify that the configuration is re-activated.

TABLE 1

| Activation | Release | Configuration |
| --- | --- | --- |
| …000 | …000 | Re-activate configuration 0 |
| …000 | …001 | Activate configuration 0, Release configuration 1 |
| …000 | …010 | Activate configuration 0, Release configuration 2 |
| … | … |  |
| …001 | …000 | Activate configuration 1, release configuration 0 |
| …001 | …001 | Re-activate configuration 1 |
| …001 | …010 | Activate configuration 1 & release configuration 2 |
| …001 | …011 | Activate configuration 1 & release configuration 3 |
| … | … | … |

The fields of the DCI format 5 used for sidelink are reconfigured to support SPS activation and release for V2V, as shown in Table 2, assuming the maximum number of SPS configurations for the UE is limited to 4 (2 bits).

TABLE 2

| Parameter | Bits | Usage |
| --- | --- | --- |
| Resource for PSCCH | 6 | N/A or set to ‘0’ |
| TPC command | 1 | N/A or set to ‘0’ |
| Resource block assignment and hopping resource allocation | 5-13 | All bits are set to ‘0’ to indicate SPS activation/release |
| Frequency hopping flag | 1 | N/A or set to ‘0’ |
| Time Resource Pattern (T-RPT) | 7 | Bits 6-4 is N/A or set to ‘0’ Bits 3-2 is used for release Bits 1-0 is used for activation |

Proposal 3

- Simultaneous activation and release of SPS configurations should be supported to reduce PDCCH signalling overhead when switching between different SPS configurations.

3. Conclusion

The following proposals are made to support multiple SPS configurations on SL:

Proposal 1

- When the eNB configures multiple SPS for SL transmissions, the UE may at least switch between the SPS configurations. In addition, it is supported to have a simultaneously active SPS on the UL for UE reporting to eNB;

Proposal 2

- RAN1 should request input from RAN 2 on (a) the maximum number of allowed SL SPS configurations for a UE, (b) the identification procedure for each SPS configuration, in order for RAN1 to decide the DCI formats for activation/release of these configurations;

Proposal 3

- Simultaneous activation and release of SPS configurations should be supported to reduce PDCCH signalling overhead when switching between different SPS configurations.

4. References

- Chairman’s notes, 3GPP TSG RAN WG1 Meeting #84bis

- R2-163056, “Report from LTE Break-Out Session”, 3GPP TSG RAN WG2 Meeting #93bis

- 3GPP TS 22.885 v14.0.0, “Study on LTE support for V2X services”

- ETSI TS 302 637-2, “Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Part 2: Specification of Cooperative Awareness Basic Service”

- R1-164763, “UE reporting procedure for SL SPS transmissions”, Samsung

- R2-163451, “UE reporting and dynamic SL SPS transmission”, Samsung