3GPP TSG RAN WG1 Meeting #86			R1-166728

# Gothenburg, Sweden 22nd - 26th August 2016

Agenda item:	7.2.4.1.2

Source: 	Samsung

Title: 	Additional details on aperiodic CSI-RS

Document for:	Discussion and Decision

# Introduction

The following agreement on aperiodic CSI-RS was made in RAN1#85 [1]:

Agreement: - Scheme 1: Aperiodic NZP CSI-RS resource is supported in Rel-14 for Class B eMIMO-Type, where - Only PUSCH based A-CSI reporting is supported. - A new aperiodic CSI-RS resource allocation/configuration is defined - Without Subframe_config - Aperiodic CSI-RS transmission instance is indicated by UL-related DCI transmitted on a control channel with a CSI request field: - Aperiodic CSI-RS transmission is in the same DL subframe as the associated UL-related DCI - FFS whether or not the UE may not assume more than one aperiodic NZP CSI-RS resource. If more than one resource is allowed, FFS the signaling details - FFS the number of bits for the CSI request field - The control channel can be PDCCH or EPDCCH. FFS whether or not the case of EPDCCH can be precluded or with some restrictions/relaxations - FFS whether or not periodic CSI-RS can be additionally configured - Scheme 2: For Class B eMIMO-Type, the following CSI-RS resource configuration mechanism is supported - A UE receives an activation/release trigger containing a choice from multiple higher-layer-configured NZP CSI-RS resources for a given CSI process - Each NZP CSI-RS resource can be either periodic or, aperiodic CSI-RS resource (if scheme 1 is supported) - Details of the activation/release trigger (including the set of NZP CSI-RS resource configuration parameters) are FFS - For an activation trigger received in subframe n, the transmission of the associated NZP CSI-RS resource will start no earlier than subframe n+X where X>0 - For a release trigger received in subframe n, the transmission of the associated NZP CSI-RS resource will stop after subframe n+Y where Y>0 - FFS the impact of multi-shot configuration, if supported - FFS whether or not there are any significant issues regarding the potential misalignment about activation and/or release between eNB and UE, and if so, how to address them - FFS whether or not to further enhance rate matching for PDSCH, particularly whether or not aperiodic NZP CSI-RS will not have impact on PDSCH RE mapping - FFS the QCL details - Down-selection between scheme 1 and scheme 2 (if any) to be discussed in RAN1#86

This contribution focuses on how to resolve the highlighted FFS points in order to reach the 90-95% completion target in [2].

# Aperiodic CSI-RS: joint operation of scheme 1 and 2

UE-specific beamformed (BF) CSI-RS (realized through CLASS B eMIMO-Type with K=1) is beneficial in reducing the number of CSI-RS ports a UE needs to measure and at the same time increasing CSI-RS penetration (due to UE-specific beamforming gain). However, the total amount of CSI-RS overhead per cell increases with the number of serviced UEs. This increase in overhead is primarily caused by the semi-static and periodic CSI-RS resource allocation per UE. It is expected that such total/cumulative CSI-RS overhead exceeds that of non-precoded (NP) CSI-RS when the number of UEs is sufficiently large.

To circumvent this drawback, two aspects of CSI-RS resource allocation can be enhanced:

- CSI-RS resource pool: This is addressed by increasing the amount of CSI-RS resources (hence the number of possible CSI-RS resource configurations) through enhancing the definition of CSI-RS resource.

- CSI-RS resource reuse: This is related to the number of UEs which can share a given CSI-RS resource (or resource configuration) – or more generally, the number of UEs which can share a CSI-RS resource pool. This can be addressed by a more dynamic (or less semi-static) CSI-RS resource allocation.

Figure 1 Aperiodic CSI-RS operation from eNB perspective: joint operation of scheme 1 and 2

To jointly address the above two problems, we propose the following procedure which jointly utilizes the agreed scheme 1 and scheme 2 as follows:

- Per the agreement, a new aperiodic CSI-RS (Ap-CSI-RS) resource configuration is defined without Subframe_config which is applicable only for CLASS B eMIMO-Type. This new Ap-CSI-RS resource is defined with the following ‘resource parameters’:

  - PC

  - Number of ports NPORT∈{1,2,4,8} (antennaPortsCount in [3])

  - T-F (time-frequency) pattern (5-bit resourceConfig in [3], CSI Reference Signal Configuration in [4])

  - Port index subset Port(0),Port(1),….,Port(NPORT-1) ⊂15,16,…,22.

- “Two-step” CSI-RS resource configuration: The above four parameters

  - Step 1 – Semi-static (via RRC signalling) resource configuration:

    - A new CSI-RS resource configuration IE for Ap-CSI-RS (distinct/separate from Rel.13 periodic CSI-RS resource configuration) is defined.

    - This new configuration allocates M1 resources (M1 to be decided) where each resource is parametrized in terms of the four resource parameters. To illustrate, if M1 is decided to be 8 (analogous to the maximum number of CSI-RS resources for CLASS B), Pc=3, NPORT={4}, T-F = {0,2,9,10,16,17,24,25}, Port index={{15,16,17,18}} is an example of a Step 1 resource configuration for a UE.

  - Step 2 – Semi-persistent (via activation/deactivation) resource configuration:

    - Out of the M1 resources configured in Step 1, M2<M1 resources are further down selected for the UE in Step 2 using Scheme 2. For instance, if M1 is decided to be 8, M2 can be decided to be 2. In this case, a 5-bit (or in general log2M1M2-bit) payload is needed to select 2 out of 8 combinations.

    - This resource down-selection can be signalled either using UL/DL grants (analogous to SPS) or MAC CE (RAN1 to decide in RAN1#86). Consequently, the values of X and Y are determined by the existing procedure of the selected signalling mechanism.

- Given the latest Ap-CSI-RS resource configuration (Step 1 + Step 2), the UE is scheduled to receive an Ap-CSI-RS via an UL grant which carries a CSI request field.

  - Since the scheduled Ap-CSI-RS occurs in the same subframe as the UL grant (as agreed), no additional bit is needed to indicate the location of Ap-CSI-RS.

  - However, the UL-related DCI needs to include an additional field which indicates CSI-RS resource selection (1 out of M2) – thus the need for a log2M2–bit field. This field is present (in DCI format 0 or 4) only when the UE is configured with aperiodic CSI-RS.

Since both schemes 1 and 2 are used for aperiodic CSI-RS, there is no need for down-selection. The two schemes are in fact parts of the aperiodic CSI-RS feature for Rel.14 eFD-MIMO.

Proposal: Specify the support for the above two-step CSI-RS resource configuration (semi-static followed by semi-persistent) and Ap-CSI-RS scheduling mechanism:

- Agree on the values of M1 and M2

- Agree on the signalling mechanism for Step 2 (log2M1M2-bit signalling for resource down-selection): UL/DL grant vs. MAC CE (to be decided in RAN1#86)

  - The values of X and Y are in accordance to the existing procedure for the selected mechanism

- When a UE is configured with Ap-CSI-RS, UL-related DCI (format 0 and 4) carries an additional log2M2-bit DCI field to select 1 out of M2 resources)

# Conclusions

In this contribution, we present our proposal to resolve the FFS points identified in RAN1#85 for aperiodic CSI-RS (Ap-CSI-RS). Our proposal can be summarized as follows:

- Specify the support for the above two-step CSI-RS resource configuration (semi-static followed by semi-persistent) and Ap-CSI-RS scheduling mechanism:

  - Agree on the values of M1 and M2

  - Agree on the signalling mechanism for Step 2 (log2M1M2-bit signalling for resource down-selection): vs. MAC CE (to be decided in RAN1#86)

    - The values of X and Y are in accordance to the existing procedure for the selected mechanism

  - When a UE is configured with Ap-CSI-RS, UL-related DCI carries an additional log2M2-bit DCI field to select 1 out of M2 resources)

# References

- 3GPP, RAN1#85, Chairman’s Notes

- R1-166724, Rapporteur Work Plan on eFD-MIMO, Samsung, Nokia, CATT

- 3GPP, TS36.331v13.1.0, “E-UTRA Radio Resource Controller (RRC) Protocol Specification”

- 3GPP, TS36.211v13.1.0, “E-UTRA Physical Channels and Modulation”