3GPP TSG RAN WG1 Meeting #86	R1-166970

Gothenburg, Sweden 22nd - 26th August 2016

Source:	Panasonic

Title: 	Discussion on DCI design in V2V

Agenda Item:		7.2.2.4.1

Document for:		Discussion and Decision

Introduction

Mode 1 SPS on PC5 was agreed in previous RAN1 meeting but the details on DCI design is still unclear. This contribution is update of R1-164906 and continues the discussion on DCI design.

Discussion

In existing LTE system, periodicity and process number of SPS are configured by RRC [2] while resource allocation and SPS activation/release are indicated by DCI which is scrambled by SPS C-RNTI [3]. SPS and dynamic scheduling transmissions share the same DCI format but some special fields are used for SPS activation/release as shown in Appendix part.

For V2V/V2X, there would two typical public safety traffics like CAM (Cooperative Awareness Message) and DENM (Decentralized Environmental Notification Message) [4][5]. First one is kind of SPS traffic and second one is rather event triggered traffic. In addition SPS and dynamic scheduling are supported for cross-scheduling based on the agreements in last meeting. We don’t see strong argument that the two scheduling methods can not be supported even for single carrier. Therefore, SPS and dynamic scheduling transmissions should be supported for single carrier and multiple carries. In addition, to minimize the blind decoding times, it is preferable that SPS and dynamic scheduling share the same DCI format whose size is aligned with DCI format 0/1A, same as existing LTE. DCI format 5 could be the starting point for the design.

So we propose

Proposal 1: SPS and dynamic scheduling should be supported for both single carrier and multiple carries in V2V

Proposal 2: SPS and dynamic scheduling share the same DCI format whose size is aligned with DCI format 0/1A. DCI format 5 is the starting point.

In order to align SPS and dynamic scheduling DCI size, there could be two directions,

- Some special fields in DCI format are reused for SPS activation/release. SPS and dynamic scheduling transmissions use different RNTI to scramble DCI.  It is similar as current LTE system.

- A new field is defined in DCI for differentiating dynamic scheduling, SPS activation and release. There is no differentiation on RNTI scrambling between SPS and dynamic scheduling.

For the 1st direction, the issue is which fields in DCI format 5 are reused for activation/release. Basically based on our understanding there could be two options like following

Option 1: all fields in DCI format 5 are used for SPS activation/release. Original fields are indicated by RRC or fixed in spec.

Table 1 SPS activation/release in option 1

| Fields for DCI format 5 | SPS activation | SPS release |
| --- | --- | --- |
| Resource for PSCCH | All bits are set to “0” | All bits are set to “1” |
| TPC command for PSCCH and PSSCH | All bits are set to “0” | All bits are set to “1” |
| Frequency hopping flag | All bits are set to “0” | All bits are set to “1” |
| Resource block assignment and hopping resource allocation | All bits are set to “0” | All bits are set to “1” |
| Time resource pattern | All bits are set to “0” | All bits are set to “1” |

Option 2: Some fields or partial field in DCI format 5 are used for SPS activation/release. RRC indicates some information originally indicated in DCI.

Table 2 SPS activation/release in option 2

| Fields for DCI format 5 | SPS activation | SPS release |
| --- | --- | --- |
| Resource for PSCCH |  |  |
| TPC command for PSCCH and PSSCH |  |  |
| Frequency hopping flag |  |  |
| Resource block assignment and hopping resource allocation |  |  |
| Time resource pattern | MSB 6 bits are set to "000000" | MSB 6 bits are set to "111111" |

For the 2nd direction, the design is like

Option 3: a new field is added for differentiating dynamic scheduling and SPS activation/release

Table 3 SPS activation/release in option 3

| Fields for DCI format 5 | SPS activation | SPS release | dynamic scheduling |
| --- | --- | --- | --- |
| Resource for PSCCH |  |  |  |
| TPC command for PSCCH and PSSCH |  |  |  |
| Frequency hopping flag |  |  |  |
| Resource block assignment and hopping resource allocation |  |  |  |
| Time resource pattern |  |  |  |
| Dynamic scheduling and SPS activation/release | All bits are set to “0” | All bits are set to “1” | Some special value like “…0011…” |

The problem for option 3 is the increased field may cause size of DCI format 5 exceeds that of DCI format 0/1A so blind decoding times are increased. To avoid such situation, a new field whose size is same as SPS activation/release could be added to DCI format 0/1A to keep aligned size between DCI format 5 and DCI format 0/1A. Or another approach is all legacy fields in DCI format 0/1A are automatically enabled when SPS V2V is configured, for example

-“CFI” field (3bits) is configured regardless of CA or non-CA

-“UL index” field (2bits) is configured regardless of FDD or TDD

-“Downlink Assignment Index” (2 bits) is configured regardless of FDD or TDD

The disadvantage of such approach is it will increase the size of DCI format 0/1A and impact its link budget.

In an extreme case, V2V SPS could totally be configured by RRC which could also indicate when to apply SPS. The merit of such approach is it reduces complexity of L1 signaling design but it also loses dynamics in physical layer.

Proposal 3: RAN1 should discuss different approaches on how to support both SPS and dynamic scheduling transmissions for V2V.

If duration of SPS is indicated in DCI format (i.e., how long SPS will last in terms of subframes), potentially there is no need to design SPS release related signalling as UE knows when activated SPS will be released. But on the other hand, to design SPS release signalling is useful for eNB to stop current SPS transmission quickly. But such release may not be explicit. Both manners could be supported. ENB could activate new SPS configuration and release old SPS configuration simultaneously. When UE receives activation signalling, it will automatically release previous SPS configuration and carry out new SPS configuration. When UE receives release signalling, it will release current SPS configuration.

So we propose

Proposal 4: SPS release signalling should be designed even if duration of SPS is indicated in DCI format.

Proposal 5: When UE receives new activation signalling during SPS is on-going, on-going SPS process is released and new SPS activation could be done simultaneously.

Another follow-up issue is what the impact to SA is, for example whether receiver UE needs to know current sidelink transmission is SPS or not and how if so. Based on our understanding, there would be frequent joining or leaving of vehicles in a group due to mobility so SA should be transmitted frequently to avoid newly joined vehicles lose the data. In this sense, no distinct on SPS and non-SPS may also be acceptable. But on the other hand, to add a counter in SA to indicate duration of SPS may be helpful for receiver to avoid collisions especially if eNB indicates SPS duration in DCI format.

Proposal 6: RAN1 should discuss SA impact to support SPS transmission.

Conclusion

In this contribution we mainly discussed signalling design to support SPS activation/release in V2V and we analysed/listed several options. We propose,

Proposal 1: SPS and dynamic scheduling should be supported for both single carrier and multiple carries in V2V

Proposal 2: SPS and dynamic scheduling share the same DCI format whose size is aligned with DCI format 0/1A. DCI format 5 is the starting point.

Proposal 3: RAN1 should discuss different approaches on how to support both SPS and dynamic scheduling transmissions for V2V.

Proposal 4: SPS release signalling should be designed even if duration of SPS is indicated in DCI format.

Proposal 5: When UE receives new activation signalling during SPS is on-going, on-going SPS process is released and new SPS activation could be done simultaneously.

Proposal 6: RAN1 should discuss SA impact to support SPS transmission.

- Reference

[1] R1-164906, “Signaling design to support SPS activation/release in V2V”, Panasonic

[2] 3GPP TS 36.331 V12.5.0

[3] 3GPP TS 36.213 V12.5.0

[4] EEE Std. 1609.2-2012 (draft D17): "Wireless Access in Vehicular Environments - Security Services for Applications and Management Messages"

[5] SI EN 302 637-2 V1.3.2 Intelligent Transport Systems (ITS)

- Appendix

Table 9.2-1: Special fields for Semi-Persistent Scheduling Activation PDCCH/EPDCCH Validation [2]

|  | DCI format 0 | DCI format 1/1A | DCI format 2/2A/2B/2C/2D |
| --- | --- | --- | --- |
| TPC command for scheduled PUSCH | set to '00' | N/A | N/A |
| Cyclic shift DM RS | set to '000' | N/A | N/A |
| Modulation and coding scheme and redundancy version | MSB is set to '0' | N/A | N/A |
| HARQ process number | N/A | FDD: set to '000' TDD: set to '0000' | FDD: set to '000' TDD: set to '0000' |
| Modulation and coding scheme | N/A | MSB is set to '0' | For the enabled transport block: MSB is set to '0' |
| Redundancy version | N/A | set to '00' | For the enabled transport block: set to '00' |

Table 9.2-1A: Special fields for Semi-Persistent Scheduling Release PDCCH/EPDCCH Validation [2]

|  | DCI format 0 | DCI format 1A |
| --- | --- | --- |
| TPC command for scheduled PUSCH | set to '00' | N/A |
| Cyclic shift DM RS | set to '000' | N/A |
| Modulation and coding scheme and redundancy version | set to '11111' | N/A |
| Resource block assignment and hopping resource allocation | Set to all '1's | N/A |
| HARQ process number | N/A | FDD: set to '000' TDD: set to '0000' |
| Modulation and coding scheme | N/A | set to '11111' |
| Redundancy version | N/A | set to '00' |
| Resource block assignment | N/A | Set to all '1's |