3GPP TSG RAN WG1 Meeting #86	R1-167131

Göteborg, Sweden, August 22-26, 2016

Agenda Item:	7.2.5.1

Source:	Huawei, HiSilicon

Title:	MUST UE multiplexing and resource allocation

Document for:	Discussion and decision

# Introduction

In RAN#71, WID RP-160680 with title “Downlink Multiuser Superposition Transmission for LTE” [1] was approved. In RAN#85, it was agreed in R1-165799 that

- No new TM for MUST

- MUST Case 1 and Case 2 using up to 2Tx is supported in the following TMs

  - TM 2/3/4

  - FFS TM 8/9/10

- A UE is signalled by RRC if it is to be configured for potential MUST operation

- FFS MUST Case 3 using up to 8Tx is supported in the following TMs

  - TM 4/8/9/10

  - Companies are encouraged to perform more evaluations especially using the agreed FTP model

- At least one new DCI is to be monitored by a UE once configured into MUST operation

  - FFS on details

- FFS MUST-near UE may assume MUST interference presence/absence is consistent among all of its scheduled PRBs for CRS-based TM and DMRS-based TM

Moreover, in RAN#85 it was agreed in R1-165764 that

- For Case 1 and 2 described in MUST WID, Far UE’s modulation order is limited to QPSK when it is co-scheduled with near UE in a given subframe.

In this contribution, we address UE multiplexing and resource allocation related to MUST with paired UEs.

# Multiplexing and resource allocation with wideband scheduling

With wideband scheduling, all sub-bands (SB) in any given subframe are assigned to the same pair of UEs – one MUST-far UE and one MUST-near UE.An example of multiplexing of a near UE codeword and far UE codeword with wideband scheduling is shown in Figure 1, where “F” (resp. “N”) represent a codeword bit or composite constellation bit of MUST-far UE (resp. MUST-near UE) and each column represents the bit label of a composite constellation symbol transmitted in a PDSCH RE (REs not carrying PDSCH information are omitted for clarity of presentation).

As the MUST-far UE uses QPSK modulation, the two most significant bits (MSB) of the composite constellation’s label are assigned to the MUST-far UE codeword in all PDSCH REs and in all sub-bands (SB). The remaining bits of the composite constellation’s label are all assigned to the MUST-near UE codeword.

Figure 1. Example of UE multiplexing in MUST with wideband scheduling.

Proposal 1: For MUST with wideband scheduling, the two most significant bits of the composite constellation label are assigned to the MUST-far UE. The label bits not assigned to MUST-far UE are assigned to the MUST-near UE.

We further observe that, as each paired UE uses the same MCS in all sub-bands, MCS and TBS can be signaled to UEs as in conventional LTE.

# Multiplexing and resource allocation with sub-band scheduling

With MUST sub-band scheduling, each sub-band can be potentially assigned to a different pair of UEs or assigned to a single UE. Figure 2 shows an example of UE multiplexing with MUST sub-band scheduling with two MUST-far UES (F1, F2) and two MUST-near UEs (N1, N2).

UE F1 is scheduled as MUST-far UE in sub-bands SB0, SB3 and as a single UE in SB2. UE F1 is paired with MUST-near UE N1 in SB0 and with MUST-near UE N2 in SB3.

UE F2 is scheduled as far UE in sub-bands SB1, SB4. UE F2 is paired with MUST-near UE N2 in SB1 and with MUST-near UE N1 in SB4.

As MUST-far UEs use QPSK modulation, in any given sub-band the two most significant bits (MSB) of the composite constellation’s symbol labels are assigned to the codeword of the UE which has been scheduled as MUST-far UE in the given sub-band. The remaining bits of the composite constellation’s symbol labels are all assigned to the codeword of the MUST-near UE scheduled in that sub-band.

Reusing conventional LTE signaling formats for MCS and TBS is possible when any MUST-near UE uses the same number of composite constellation label bits in all its PDSCH REs, as shown in the example of Figure 2. As a result, MCS and TBS can be signaled to UEs using the same bit fields already defined in conventional LTE DCI formats.

Figure 2. Example of UE multiplexing in MUST with sub-band scheduling.

Proposal 2: For MUST with sub-band scheduling, in each sub-band with more than one UE co-scheduled UE, the two most significant bits of the composite constellation label are assigned to the UE which is scheduled as MUST-far UE in that sub-band. The label bits not assigned to the MUST-far UE are assigned to the MUST-near UE.

Proposal 3: The composite constellation symbols transmitted in the PDSCH REs allocated to a MUST-near UE in a given sub-frame carry the same number of MUST-near UE codeword bits.

Proposal 4: MCS and resource allocation are signalled to MUST co-scheduled UEs using the same bit fields already defined in conventional LTE (Rel. 12) DCI formats.

# Conclusions

In this contribution, we discussed resource allocation and multiplexing of paired UEs in MUST transmission. We discussed the wideband scheduling case and sub-band scheduling case. The following proposals are given:

Proposal 1: For MUST with wideband scheduling, the two most significant bits of the composite constellation label are assigned to the MUST-far UE. The label bits not assigned to MUST-far UE are assigned to the MUST-near UE.

Proposal 2: For MUST with sub-band scheduling, in each sub-band with more than one UE co-scheduled UE, the two most significant bits of the composite constellation label are assigned to the UE which is scheduled as MUST-far UE in that sub-band. The label bits not assigned to the MUST-far UE are assigned to the MUST-near UE.

Proposal 3: The composite constellation symbols transmitted in the PDSCH REs allocated to a MUST-near UE in a given sub-frame carry the same number of MUST-near UE codeword bits.

Proposal 4: MCS and resource allocation are signalled to MUST co-scheduled UEs using the same bit fields already defined in conventional LTE (Rel. 12) DCI formats.

# References

RP-160680, “New work item proposal: Downlink Multiuser Superposition Transmission for LTE”, Mediatek Inc., CMCC, Gothenburg, Sweden, March 7-10, 2016.