3GPP TSG RAN WG1 Meeting #86	R1-167217

Gothenburg, Sweden, Aug.  22 – 26, 2016

Agenda Item:	8.1.6

Source:	Huawei, HiSilicon

Title:	A common synchronization signal for a NR carrier supporting different numerologies

Document for:	Discussion/Decision

# Introduction

It was agreed to support a NR carrier using multiple numerologies in RAN1 #85 [1]:

- Forward compatibility of NR shall ensure smooth introduction of future services and features with no impact on the access of earlier services and UEs

  - Multiplexing different numerologies within a same NR carrier bandwidth (from the network perspective) is supported

    - FDM and/or TDM multiplexing can be considered

For a NR carrier using multiple numerologies, it was further agreed to study means to share a synchronization signal in RAN1 #85:

- For a NR carrier (from network perspective) using multiple numerologies, at least the following is for further study

  - multiple frequency/time portions using different numerologies share a synchronization signal

    - Note: The synchronization signal refers to the signal itself and the time-frequency resource used to transmit the synchronization signal

A working assumption on scaled subcarrier spacing was made in RAN1 #85:

- RAN1 concludes on alternative 1 (15 kHz) as the baseline design assumption for the NR numerology

- RAN1 concludes on scale factors N =2n for subcarrier spacing as the baseline design assumption for the NR numerology

In this contribution, we discuss means to share a synchronization signal for a NR carrier using multiple numerologies, especially concerning scaled subcarrier spacing.

# Issues and design objectives

New Radio (NR) access technology aims to meet a broad range of use cases including enhanced mobile broadband, massive Machine Type Communication (MTC), critical MTC, and additional requirements defined during the RAN requirements study [2]. To address various scenarios and requirements, NR may support multiple numerologies, e.g., via scalable numerology and/or mixed numerology. More specifically, a carrier with scalable numerology may operate with different subcarrier spacing in a Time Division Multiplexing (TDM) manner, i.e., at some time instances the subcarrier spacing takes one value while at some other time instances the subcarrier spacing takes some other values. A carrier with mixed numerology may be operated with multiple subcarrier spacing values in a Frequency Division Multiplexing (FDM) manner, i.e., the carrier is divided into several subbands in the frequency domain, where each subband uses a specific subcarrier spacing value. To avoid Inter-Carrier-Interference (ICI), subband specific filters may be added for a mixed carrier.

In order to simplify system design and reduce overhead, it is expected to design common synchronization signal for a NR carrier with multiple numerologies.

# Synchronization signals

In order to achieve common synchronization signal for a NR carrier using multiple numerologies, the two alternatives as discussed in section 3.1 and section 3.2 can be considered.

Common Synchronization Signal with an X-time repetitive waveform

### Transceiver

The intention is to generate a same time domain signal using different subcarrier spacing, which avoids ICI and simplifies implementation.

Transmitter

- Mapping to Resource Elements

The mapping is subcarrier spacing specific, as shown in Figure 2, where two subcarrier spacing with Scale Factor (SF) {1,2} are supported. For SF1, PSS sequence is mapped to every other RE, i.e., X=2 as in Equation 1, while for SF2, PSS sequence is mapped in a localized way, i.e. X=1.

(1)

- OFDM Symbol Generation

The OFDM symbol generation is unchanged from LTE, i.e. the OFDM symbol is generated using the selected subcarrier spacing according to SF.

The time domain signal sSF1 is a X time repetition of the time domain signal sSFX, as proved in Appendix A.

sSF1n+qNX=sSFXn,              n=0,1,…,NX-1;  q=0,1,…,X-1	           (2)

The time repetition property can be further shown in Figure 3, considering the basic subcarrier spacing 15 kHz with SF1 and SFX. The mapping is SF specific as given in Equation 1 and the FFT size is 2048.

- Time Repetition

For SF greater than 1, i.e. SFX, the synchronization signal is further repeated in the time domain to form an X time repetition, as shown in Figure 2 (X=2 as an example). Therefore a common time signal can be achieved for both SF1 and SFX. It should be noted that the time repetition is done immediately after the first OFDM symbol, i.e. without inserting Cyclic Prefix (CP) in between. The repetitive property can be further shown in Figure 3, where time domain samples for sSF1 and sSF2 are provided.

Figure 2. Mapping and repetition for subcarrier spacing SF1 and SF2 at the transmitter

Figure 3. Time domain samples (Not all samples are shown for better readability)

Receiver

Thanks to the common synchronization signal property, the LTE SYNC receiver structure can be reused, i.e. a common low pass filter and common baseband processor (e.g. matched filter for PSS signal processing) may be used, as shown in Figure 4.

Figure 4. Receiver structure for common synchronization signal

### Benefits

- Reduces implementation complexity due to enabling single IFFT

LTE synchronization signal is generated by mapping a sequence in the frequency domain, and then OFDM modulated, i.e. processed by a single IFFT even when there are other signals/channels transmitted in the same OFDM symbol. This is especially convenient when the network may operate with different bandwidths using one single subcarrier spacing, as the PSS signal generation can share the same OFDM symbol generation with other FDM multiplexed signals/channels.

However, it would be not feasible to use a single IFFT if different sub-carrier spacing values are used for synchronization signal and other signals/channels transmitted in the same OFDM symbol. Generating the common synchronization signal with an X-time repetitive waveform can still use single IFFT, thus it can reduce the implementation complexity.

However, further investigation can be carried out to evaluate the impact on forward compatibility, NR frame structure design, MIB detection complexity and trade-off between performance and overhead.

Common synchronization signal with default subcarrier spacing

### Transceiver

The intention is to generate common synchronization signal with default subcarrier spacing. The default subcarrier spacing belongs to one of the candidate subcarrier spacing values for the carrier, where the candidate subcarrier spacing values for a carrier may be dependent on the carrier frequency of the carrier. The bigger subcarrier spacing could be used as the default subcarrier spacing considering it may provide better time resolution and may be more robust to high speed scenario, e.g. 30 kHz could be used when the candidate subcarrier spacing values includes 15 kHz and 30 kHz. On the other hand, using the narrower subcarrier spacing as the default will provide better coverage. However, which subcarrier spacing to be used can be further studied and evaluated.

Transmitter

Similar scheme (e.g. sequence generation and OFDM symbol generation) in LTE can be reused by replacing 15 KHz with the default subcarrier spacing. One example of synchronization signal using default subcarrier spacing of 30 kHz is provided in Figure 5.

Figure 5. Mapping of synchronization signal using a default subcarrier spacing of 30 kHz

Receiver

Since common synchronization signal can also be achieved, the LTE SYNC receiver structure can also be reused as generating common synchronization signal with an X-time repetitive waveform discussed in section 3.1.

### Benefits

- Reduces blind decoding of essential system information (e.g. MIB)

Same default sub-carrier spacing can be used for both synchronization signal and essential system information transmission, which can avoid the blind decoding of essential system information transmission for subcarrier spacing. The subcarrier spacing used for other signals/channels (e.g. shared channels) can be indicated in the essential system information or RRC signaling.

- Reduces implementation complexity due to reducing the blind detection of MIB

Reducing the blind detection of essential system information transmission can reduce the implementation complexity at the UE side.

- Good forward compatibility to additional numerology

The synchronization signal is basically not related to the numerology, which provides good forward compatibility to additional numerology.

# Conclusions

In this contribution, we discuss means to share a common synchronization signal for a NR carrier using multiple numerologies, especially concerning scaled subcarrier spacing.

It is further realized that there might be multiple UE types for NR with capabilities of a limited number of subcarrier spacing values and/or frequency bands, i.e. not as many as the network. It would be therefore desired for the network to transmit a synchronization signal detectable by any type of UEs even with limited subcarrier spacing/frequency band capability. This may have an implication that the synchronization signal transmission having a linkage to the UE capability.

Proposal:	Study the following means for achieving common synchronization signal for a NR carrier using multiple numerologies:

- Common synchronization signal with an X-time repetitive waveform and different subcarrier spacing values. X equals to the largest Scale Factor.

- Common synchronization signal with default subcarrier spacing. FFS the value of the default subcarrier spacing, e.g. frequency band dependent.

# References

- RAN1 #85 Chairman Notes

- 3GPP TR 38.913 v0.3.0, Study on scenarios and requirements for next generation access technology

- 3GPP TS36.211

- J. Proakis, Digital Communications. McGraw-Hill, 2001.

# Appendix A. Time-repetitive waveform

The OFDM waveform without a cyclic prefix can be defined by:

for 0≤t<N×Ts, where ,  is 2048, Ts=1/fs   and fs=N× according to the LTE standard. Hence it is possible to define:

H[k]=ak,  &-NRBDLNscRB/2≤k≤NRBDLNscRB2∩k≠0  &&0,  &elsewhere

and

st=k=-N/2N/2H[k]∙ej2πk∆ft.

The sampled version by setting t=n/fs is:

sn=1Nk=-N/2N/2Hkej2πnkN

for n=0,1,…,N-1 where Hk is a Fourier coefficient at frequency .

LTE PSS sequence is given by:

where u is the Zadoff-Chu root sequence index.

- Subcarrier Spacing with SF1

- For SF1, PSS sequence is mapped to 1 RE in every X REs, as in Equation 1. Therefore the sampled signal is:

- sSF1n=1Nk=-N/2N/2Hkej2πnkN=1Nm=3161d(m)ej2πnX(m-30)N+m=030d(m)ej2πnX(m-31)N

It can be further shown that:

sSF1n+qNX=1Nk=-N2N2Hkej2πn+qNXkN=1Nm=3161dmej2πn+qNXXm-30N+m=030dmej2πn+qNXXm-31N=1Nm=3161dmej2πnXm-30N+m=030dmej2πnXm-31N=sSF1n,

n=0,1,…,N/X-1;  q=0,1,…,X-1

This proves that sSF1 is time domain X repetition waveform.

- Subcarrier Spacing with SFX

For SFX, PSS sequence is mapped in a localized way, as in Equation 1. Therefore the sampled signal is:

- sSFXn=1NXk=-N2XN2XHkej2πnkNX=1NXm=3161dmej2πnm-30NX+m=030dmej2πnm-31NX=1NXm=3161dmej2πnXm-30N+m=030dmej2πXm-31N=sSF1n,

- n=0,1,…,N/X-1

This, together with the above conclusion that sSF1 is time domain X repetition waveform, proves that the time domain signal  sSF1 is X repetition of the time domain signal sSFX (without considering normalization factor).