3GPP TSG RAN WG1 Meeting #86 	R1-167363

Gothenburg, Sweden 22nd - 26th August 2016

Source:	NTT DOCOMO

Title:	Discussion on power ratios for MUST scheme

Agenda Item:	7.2.5.1

Document for: 	Discussion

Introduction

At the RAN1 #85 meeting, the following agreements about transmission power ratios for Downlink Multiuser Superposition Transmission (MUST)[1] were reached.

- For further down-selection on the set of transmission power ratios, companies are encouraged to provide the scheduling PDF of power ratios and the corresponding performance for different sets of power ratios

  - For MUST Case 1 and Case 2, multiple power ratios are supported at least for some combinations of MUST-near UE and MUST-far UE modulation orders

    - For case 3, FFS

- For Case 1 and 2, and for each combination of modulation order,

  - The number of power ratios generating non-uniform composite constellation should be chosen from 0 (for some combinations, if any), 1, 2 or 3.

    - The details are FFS.

  - Power ratios generating non-uniform composite constellation should be selected from the range [0.7, 0.95].

    - The values of power ratio is FFS.

    - 0.7 should be excluded in case of 64QAM (for near UE) + QPSK (for far UE).

In this contribution, we discuss and present the views on optimization of the transmission power ratio for the MUST scheme. The number and values for transmission power ratios are discussed in respect of system performance and standardization impact. The scheduling PDF of power ratios and the corresponding performance for different sets of power ratios are provided.

Discussion

Transmission power ratios for MUST

In the study item of MUST, MUST categories with Gray-mapping composite constellation have been discussed [2]. For MUST-superposed UEs, multiple transmission power ratios can be adopted on top of the Gray-mapping composite constellation. For a combination of modulation orders, one of the power ratios is specially determined in order to achieve legacy composite constellations with constant Euclidian distances among different constellation points. The transmission power ratios of legacy constellations are targeted to achieve the constant Euclidian distances of composite constellation points rather than to match the channel conditions of the UEs. The values of the transmission power ratios are determined according to the composite modulation order of the superposed UEs. In addition to the transmission power ratios of legacy constellations, adaptive transmission power ratios generating non-uniform composite constellation can also be adopted for MUST superposed UEs. With adaptive transmission ratios, the Euclidian distance between different component constellations can’t be kept the same in the composite constellation. The adaptive transmission power ratios are targeted to better match channel conditions of the superposed UEs so as to maximize the scheduling metric, e.g. proportional fairness metric, of the system.

Candidates of transmission power ratios

In MUST schemes, assistance information is required for UE receivers to cancel the inter-superposition-layer interference. The transmission power ratios can be signalled or blindly detected at the receiver side. Therefore, both system performance and the signalling overhead or blind detection complexity for transmission power ratios should be taken into account when determining the number of transmission power ratios.

For MUST superposed UEs, the values of transmission power ratios of legacy constellations are determined according to the composite modulation orders of the superposed UEs. Note that different power ratios need to be adopted for different combinations of ranks of the superposed UEs. For example, for the combination of a rank 2 MUST-near UE and a rank 1 MUST-far UE, the data of two UEs are multiplexed on one of the layers of rank 2 MUST-near UE, while there is no inter-user interference in the other layer.

For the transmission power ratios of non-uniform composite constellation, power ratios which are selected from the ones mostly chosen can be considered as candidates. We consider different number of power ratios generating non-uniform composite constellation, ranging from 1 to 3. The transmission power ratios candidates for both uniform and non-uniform composite constellations are presented in Table I, where transmission power ratios (α1, α2) for near and far UEs, α1+α2=1.

Table I: Transmission power ratios of non-uniform composite constellation and uniform composite constellation

| Number of power ratios for non-uniform constellation | Power ratios for non-uniform composite constellation | Power ratios for uniform composite constellation |
| --- | --- | --- |
| 1 | α1= {0.14} | •For (MOD_N, MOD_F) = (QPSK, QPSK), (1/3, 2/3) rank 2-1, (1/5, 4/5) rank 1-1 and rank 2-2 •For (MOD_N, MOD_F) = (16QAM, QPSK), (5/13, 8/13) rank 2-1, (5/21, 16/21) rank 1-1 and rank 2-2 •For (MOD_N, MOD_F) = (64QAM, QPSK), (21/53, 32/53) rank 2-1, (21/85, 64/85) rank 1-1 and rank 2-2 |
| 2 | α1= {0.14, 0.23} |  |
| 3 | α1= {0.14, 0.17, 0.23} |  |

System-performance of different candidates of transmission power ratios

The system-level simulations are conducted with MUST scenario 1 and evaluation assumptions in [2]. The detailed simulation parameters are shown in Table A-I in the Annex A. We assume 2 x 2 antenna configuration with SU-MIMO of TM4. When MUST is applied, up to two superposed data layers from two co-scheduled UEs per spatial layer are supported and the maximum number of spatial layers in a cell is two. For MUST-Far UE, the modulation order is limited only to QPSK. Transmission power alignments for different subbands and the same precoder for paired UEs are assumed. The scheduling SINRs of superposed users are approximated from LTE OFDMA implicit feedback and OLLA is further applied to compensate the CQI imperfections in the evaluations. The R-ML receiver is adopted for cell-center UE to deal with the inter-user interference. In the evaluations, FTP traffic model 1 with resource utilization (RU) of around 60% and 80% are assumed. File dropping is modeled according to [3]. For performance metrics of FTP traffic, 5/50/95%, mean user perceived throughput (UPT) and mean UPT below 5% CDF are evaluated. In addition, ratio of served cell throughput over offered cell throughput is also provided.

Table II: Performance of MUST with 1/2/3 power ratios of non-uniform constellation + legacy power ratios of uniform constellation for wideband scheduling for FTP traffic (RU ~80%)

| Throughput (Mbps) | Baseline | 3 adaptive power ratio + legacy power ratio |  | 2 adaptive power ratio + legacy power ratio |  | 1 adaptive power ratio + legacy power ratio |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MUST | Gain | MUST | Gain | MUST | Gain |
| Mean UPT | 8.8520 | 10.2905 | 16.25% | 10.1869 | 15.08% | 10.1083 | 14.19% |
| 95%ile UPT | 32.0000 | 34.7827 | 8.70% | 34.7826 | 8.70% | 34.7826 | 8.70% |
| 50%ile UPT | 4.9383 | 6.0606 | 22.73% | 6.0150 | 21.80% | 5.9259 | 20.00% |
| 5%ile UPT | 0.7851 | 0.9836 | 25.28% | 0.9685 | 23.36% | 0.9556 | 21.72% |
| Mean UPT below 5% | 0.4266 | 0.6430 | 50.73% | 0.6157 | 44.33% | 0.6065 | 42.17% |
| RU (%) | 78.95% | 75.62% | ---- | 75.87% | ---- | 76.04% | ---- |
| Served/Offered | 97.98% | 98.87% | ---- | 98.80% | ---- | 98.79% | ---- |
|  / packet size | 11.0 / 100 Kbytes |  |  |  |  |  |  |

Table III: Performance of MUST with 1/2/3 power ratios of non-uniform constellation + legacy power ratios of uniform constellation for subband scheduling for FTP traffic (RU ~80%)

| Throughput (Mbps) | Baseline | 3 adaptive power ratio + legacy power ratio |  | 2 adaptive power ratio + legacy power ratio |  | 1 adaptive power ratio + legacy power ratio |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MUST | Gain | MUST | Gain | MUST | Gain |
| Mean UPT | 8.7677 | 9.3872 | 7.07% | 9.3353 | 6.47% | 9.2789 | 5.83% |
| 95%ile UPT | 29.6297 | 30.7693 | 3.85% | 30.7693 | 3.85% | 30.7693 | 3.85% |
| 50%ile UPT | 5.2980 | 5.7972 | 9.42% | 5.7555 | 8.64% | 5.7143 | 7.86% |
| 5%ile UPT | 0.9976 | 1.1035 | 10.62% | 1.0930 | 9.56% | 1.0841 | 8.67% |
| Mean UPT below 5% | 0.6619 | 0.7592 | 14.70% | 0.7489 | 13.14% | 0.7424 | 12.16% |
| RU (%) | 82.63% | 81.48% | ---- | 81.58% | ---- | 81.73% | ---- |
| Served/Offered | 98.94% | 99.21% | ---- | 99.18% | ---- | 99.18% | ---- |
|  / packet size | 13.5.0 / 100 Kbytes |  |  |  |  |  |  |

Table I and II present the performance of MUST with multiple transmission power ratios for wideband and subband scheduling for FTP traffic with RU around 80%. For the candidates of transmission power ratios, 1/2/3 optimized transmission power ratios of non-uniform constellation and legacy power ratios of uniform constellation for each modulation combination are adopted. From the results, it can be observed that MUST with different configurations of transmission power ratios can achieve significant performance gains for mean UPT with 80% RU. The performance gain for 5%ile UPT of MUST is slightly larger when more adaptive transmission power ratios of uniform constellation are adopted. On the other hand, similar performance gains of MUST can be achieved for 1, 2 or 3 transmission power ratios of uniform constellation plus legacy power ratios of uniform constellation. For subband scheduling, the same tendency can be observed.

Observation 1: Similar performance gains of MUST can be achieved for 1, 2 or 3 transmission power ratios of uniform constellation plus legacy power ratios of uniform constellation with wideband or subband scheduling.

(a) 1 adaptive power ratios + legacy ratios for near UE           (b) 2 adaptive power ratios + legacy ratios for near UE

(c) 3 adaptive power ratios +legacy ratios for near UE

Figure 1: Scheduling PDF of power ratios (Number of power ratios = 2/3/4, including 1/2/3 power ratios for non-uniform composite constellation) with different modulation combinations (MOD_Near_UE, MOD_Far_UE)

Figure 1 presents the scheduling PDF of power ratios with 2/3/4 transmission power ratios in total, each of which includes 1/2/3 power ratios for non-uniform composite constellation and legacy power ratios for uniform composite constellation. Note that exact value of legacy power ratio depends on the combination of transmission rank among multiplexed UEs. In QPSK-QPSK case, for example, the legacy power ratio is 0.2 for Rank 1/1 and 2/2 cases but it is 0.33 (1/3) for Rank 2/1 case. Hence, there are totally 2 power ratios as legacy power ratio for each modulation combination. From Fig.1, we can observe that the adaptive transmission power ratios for non-uniform composite constellation are selected with high probability for all combinations of modulation. It may indicate that adaptive transmission power ratios are frequently selected by eNB to better match channel conditions of the superposed UEs so as to maximize the scheduling metric of the system. For all the modulation combinations, power ratio of 0.14 is used with highest probability. On the other hand, the scheduling probability of power ratios is different for different modulation combinations. For (QPSK, QPSK), scheduling probability of power ratio 0.23 is not so frequently used, while there is still high probability of 0.23 for (16QAM, QPSK). It indicates that different power ratios values or numbers could be adopted for different modulation combinations.

Observation 2: Adaptive transmission power ratios for non-uniform composite constellation are selected with high probability for all combinations of modulation.

Observation 3: Different power ratios values or numbers could be adopted for different modulation combinations.

Conclusion

In this contribution, we discussed the optimization of transmission power ratios for MUST scheme. The system-performance of MUST with different number and different candidates of transmission power ratios is evaluated and discussed. According to the discussions, we have following observations and proposal.

Observation 1: Similar performance gains of MUST can be achieved for 1, 2 or 3 transmission power ratios of uniform constellation plus legacy power ratios of uniform constellation with wideband or subband scheduling.

Observation 2: Adaptive transmission power ratios for non-uniform composite constellation are selected with high probability for all combinations of modulation.

Observation 3: Different power ratios values or numbers could be adopted for different modulation combinations.

- References

- 3GPP RP-160680, Mediatek, “New work item proposal: Downlink Multiuser Superposition Transmission for LTE”, Mar. 2016.

- 3GPP TR 36.859 V13.0.0, “Study on Downlink Multiuser Superposition Transmission (MUST) for LTE,” Dec. 2015.

- 3GPP TR 36.814 V9.0.0, “Further advancements for E-URRA physical layer aspects”, Mar. 2010.

- Annex A: Simulation assumption

Table A-I: Simulation parameters

| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (ISD = 500 m) |
| --- | --- |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Shadowing | ITU Uma |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 25 m |
| UE antenna Height | 1.5 m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 2Tx (0.5 lambda), cross-polarized UE: 2Rx (0.5 lambda), cross-polarized |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| minimum distance from macro-cell to UEs | 35 m |
| Traffic model | FTP: 0.1 MByte, RU= 80% |
| UE receiver | - MMSE-IRC is assumed for inter-cell interference suppression - R-ML for inter-spatial-layer - R-ML for inter-user interference |
| Transmission mode | TM4 |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Cell selection criteria | RSRP |
| Handover margin | 3 dB |
| Scheduling algorithm | Proportional fairness maximization |
| Control delay (scheduling, AMC) | 5 msec |
| HARQ | Chase combining |
| Round trip delay (HARQ) | 8 msec |
| Granularity of CSI feedback | 5 msec |
| Granularity of rank adaptation | 100 msec |
| CQI quantization | Yes |
| Codebook | LTE Rel. 8 |
| Power ratio sets | Adaptive power ratios: 1/2/3 power sets, (α1, α2), α1={0.14, 0.17, 0.23}; Legacy power ratios: (α1, α2) for near and far UEs - (QPSK, QPSK), (1/3, 2/3) rank 2-1, (1/5, 4/5) rank 1-1 and rank 2-2 - (16QAM, QPSK), (5/13, 8/13) rank 2-1, (5/21, 16/21) rank 1-1 and rank 2-2 - (64QAM, QPSK), (21/53, 32/53) rank 2-1, (21/85, 64/85) rank 1-1 and rank 2-2 |
| OLLA | Yes |
| Number of superposed signals in superposition transmission | 2 |
| Receiver impairment modeling for demodulation | Non-ideal CRS-based channel estimation |
| EVM | Tx EVM: 8%, UE Rx EVM: 4% |
| Duration of simulation | 50000 msec (5000 pre-run + 45000 simulation) |
| Maximum transfer time (T_drop) | 1.6 sec |